# 3GPP TSG-RAN WG4 Meeting # 115                                R4-2506957

# Malta, MT, 19th – 23rd May, 2025

# Title: 	Discussion on spatial channel model for demodulation performance requirements

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.16.3

# Document for:	Approval

# 1 Introduction

In RAN #104 meeting, the new WID of study on spatial channel model for demodulation performance was approved in [1], wherein the study item includes the following objectives :

- Study practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements: - Identify the limitations of the current (i.e. up to and including Release 18) channel models and corresponding scenarios and how they relate to UE MIMO performance - Consider both Clustered Delay Line (CDL)-based and TDL-based channel modelling approaches - For CDL-based channel modelling, use the tuned repeatable spatial channel model of TR38.827 as the starting point and identify any necessary changes. - Study and verify test methodology feasibility including test complexity and achievable results uncertainty. The test complexity shall not be significantly increased. - The methodology shall include both FR1 (conducted) and FR2 (wireless cable), with first priority for FR1.

In this contribution, we give some initial discussion on the practical spatial channel modelling methodology and other analysis based on the last meeting conclusion [2].

# 2 Discussion

AAV configuration for CDL

Agreement AAV Option 3 to be used for alignment. FFS other AAV options may be used.

Regarding antenna array virtualization, there still have some open issues on the above table. RAN4 may needs to consider downselection based on best performance, ease alignment, and practicality implementation. From previous meetings simulation results, the performance of AAV with sub array is better than without AAV configuration, since AAV can bring the gain of beamforming, antenna radiation power pattern and antenna array gain. Currently, RAN4 agreed to use AAV Option 3 for alignment, it is only has radiation power pattern gain compared to other AAV options. To some extent, it simplifies the effect of other gains on the results, which also makes it possible to align the results. Regarding other AAV options, we prefer not to consider for alignment. Currently, our motivation is to align the results to verify the feasibility of spatial channel model. Even if other options can bring more gains, we can achieve the lower SNR point. However, from the perspective of testing, a result that can be converge is the most meaningful, so as to ensure the feasibility of the test. Even if the SNR is relatively large, it only needs to be ensured to be within the test range. At the same time, from the perspective of BS side implementation, vertical mergers are easier to implement and utilize in practical network. Other AAV options can be used for comparison.

Proposal 1. Propose to only consider AAV Option 3 to be used for alignment.

# 3 Conclusion

In this contribution, we give some discussions on spatial channel model for demodulation performance requirements, The conclusions are:

Proposal 1. Propose to only consider AAV Option 3 to be used for alignment.

# 4 References

- RP-241610, New WID: Study on spatial channel model for demodulation performance requirements, Nokia.

- R4-2504756, Way Forward [114][322] NR_SCM. Nokia.