# 3GPP TSG-RAN WG4 Meeting # 115                                R4-2506958

# Malta, MT, 19th – 23rd May, 2025

# Title: 	Simulation results on spatial channel model for demodulation performance requirements

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.16.3

# Document for:	Discussion

# 1 Introduction

In RAN #104 meeting, the new WID of study on spatial channel model for demodulation performance was approved in [1], wherein the study item includes the following objectives :

- Study practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements: - Identify the limitations of the current (i.e. up to and including Release 18) channel models and corresponding scenarios and how they relate to UE MIMO performance - Consider both Clustered Delay Line (CDL)-based and TDL-based channel modelling approaches - For CDL-based channel modelling, use the tuned repeatable spatial channel model of TR38.827 as the starting point and identify any necessary changes. - Study and verify test methodology feasibility including test complexity and achievable results uncertainty. The test complexity shall not be significantly increased. - The methodology shall include both FR1 (conducted) and FR2 (wireless cable), with first priority for FR1.

In this contribution, we give some initial simulation results on the practical spatial channel modelling methodology.

# 2 Discussion

During the last meeting, RAN4 discussed how to construct the SCM channel model, and agreed some simulation parameters to do link level simulation. Based on this agreement [2][3], we give some initial simulation results for CDL-C channel model.

Regarding simulation assumptions[2][3], and for CDL-C channel model, we reused the channel parameters in TR38.901 and TR38.901 with truncation to generate the channel coefficient. We also evaluated the performance of the TDL based approaches under 4T4R and 8T8R scenarios. The detailed parameters can be found in the following Table 1 to generate the TX-RX beams.

- 4T4R,4Layers: Enhanced TDLC300-100, XPL, high with 2 Tx-Rx beams as defined in Table C.3

Table 1: Cluster model 2 with 4 TX-RX clusters

| Tap | Delay(ns) | Power | Beam steering |
| --- | --- | --- | --- |
| 1 | 0 | -6.9 | Cluster 1 θtx1(1)=0 θrx1(1)=0 |
| 2 | 65 | 0 |  |
| 3 | 70 | -7.7 | Cluster 2 θtx1(2)=−π/2 θrx1(2)=−π/2 |
| 4 | 190 | -2.5 |  |
| 5 | 195 | -2.4 |  |
| 6 | 200 | -9.9 |  |
| 7 | 240 | -8.0 |  |
| 8 | 325 | -6.6 |  |
| 9 | 520 | -7.1 | Cluster 3 θtx1(3)=−π θrx1(3)=−π |
| 10 | 1045 | -13.0 |  |
| 11 | 1510 | -14.2 | Cluster 4 θtx1(4)=−3π/2 θrx1(4)=−3π/2 |
| 12 | 2595 | -16.0 |  |

In this section, we present the simulation results in terms of the normalized PDSCH throughput as function of the signal to noise ratio (SNR). We depict the SNR value corresponding to 30% and 70% of the peak throughput. All figures have been provided in the following Table 2 and Table 3.

Table 2. Summary of simulation results for CDL Option 1 with AAV option3

| Rank | Channel model and antenna virtualization | SNR(dB)@30% of maxTP | SNR(dB)@70% of maxTP |
| --- | --- | --- | --- |
| 4 | (M,N,P,Ms,Ns)=(1,2,2,1,1) based on Table 7.7.1-3 in TR 38.901 without truncation | 8.6 | 17.3 |

Table 3. Summary of simulation results for TDLC300-100 4TX-RX beams

| Rank | Channel model and antenna virtualization | SNR(dB)@30% of maxTP | SNR(dB)@70% of maxTP |
| --- | --- | --- | --- |
| Rank 4 | TDLC300-1000 XP High with 2 TX-RX beams | 6.0 | 15.8 |
| Rank 8 | TDLC300-1000 XP High with 4 TX-RX beams | 6.3 | 16.2 |

Table 5. Throughput curves for TDLC300-100 with 2 TX-RX beams and 4 TX-RX beams

| Figure 5. TDLC XP High with 2 TX-RX beams for Rank 4. | Figure 6. TDLC XP High with 4 TX-RX beams for Rank 8. |
| --- | --- |

# 3 Conclusion

In this contribution, we give some initial simulation results on spatial channel model for demodulation performance requirements.

# 4 References

- RP-241610, New WID: Study on spatial channel model for demodulation performance requirements, Nokia.

- R4-2504756, Way Forward [114][322] NR_SCM. Nokia.

- R4-2504762, Simulation assumptions for SCM. Huawei, HiSilicon, MediaTek