# 3GPP TSG-RAN WG4 Meeting # 115                                R4-2506959

# Malta, MT, 19th – 23rd May, 2025

# Title: 	Draft TP introduction of comparison of spatial channel models for SU-MIMO PMI cases

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.16.3

# Document for:	Approval

# 1 Background

This draft TP captures comparison of PDSCH performance under SU-MIMO scenario for different spatial channel model part.

# 2 Draft TP

# 6  Comparison of Spatial Channel Models

6.1  PDSCH performance under SU-MIMO scenario(PMI Aspects)

This section provides comparison for different spatial channel modes with simulation assumptions captured in Table 6.1-2.

Table 6.1-2: Simulation assumptions for PMI test cases

| Parameter |  | SU-MIMO PMI |
| --- | --- | --- |
| Duplex mode |  | TDD |
| TDD Slot Configuration Pattern |  | 7DS2U |
| FR / Carrier frequency |  | FR1 / 3.5 GHz |
| UE receiver type |  | IRC Baseline |
| Number of Tx Ports |  | 8 |
| Number of Rx Ports/Antennas |  | 4 |
| Number of layers |  | 4,2 |
| PMI |  | Follow, Random |
| Waveform |  | CP-OFDM with normal CP |
| Channel Bandwidth/SCS |  | 40MHz/30kHz |
| MCS |  | 13 (64 QAM table) |
| PDSCH configuration | Mapping type | Type A |
|  | k0 | 0 |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PDSCH aggregation factor | 1 |
|  | Resource allocation type | Type 0 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size | N/A |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 2 (for rank > 4) 1 (for rank <= 4) |
| Codebook configuration | CodebookType | type-I-SP, eType-II |
|  | Codebook configuration | For typeI-SP: (N1,N2,O1,O2) = (4,1,4,1) For eType-II: (N1,N2,O1,O2) = (4,1,4,1) [ paramCombination-r16=5 (L=4, pν=1/4, β=3/4) ] |
| PDSCH DMRS Precoding Configuration |  | For typeI-SP (random and follow): Wideband PMI For eType-II with follow and random: TBD |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type | Aperiodic |
|  | Number of CSI-RS ports (X) | 8 CSI-RS Ports (4,1) |
|  | Density (ρ) | 1 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) | (13) |
| CQI/RI/PMI delay |  | 6.5 ms |
| Number of HARQ Processes |  | 8 |
| Maximum HARQ transmissions |  | 4 |
| Channel Models |  | The purpose of the study item. |
| Testing Metric |  | Full Throughput Curves |
| Channel Geometry (CDL) | LCS UE | α = 180°, β=0°, γ = 0° |
|  | LCS gNodeB | α = 0°, β=10°, γ = 0° |
|  | GCS UE | Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m |
|  | GCS gNodeB | Height = 25 m; Azimuth = 0; X Coordinate = 0 m |
|  | BS Antenna Polarisation | (+45, -45) |
|  | BS Radiation Pattern | defined in TR38.901 Table 7.3-1. |
|  | UE Antenna Polarisation | (0, 90) |
|  | UE Antenna Radiation Pattern | Omnidirectional |
|  | Antenna Panel Placement | YZ Plane |

For the CDL channel model, RAN4 modeled CDL Option 1 (based on Table 7.7.1-3 in TR 38.901 without truncation) and CDL Option 2 (based on Table 7.7.1-3 in TR 38.901 with truncation) with Antenna Array Virtualization (AAV) using a 1x1 subarray configuration (Option 3).

For the enhanced TDL channle model, RAN4 modeled enhanced TDL Option 1 (Cluster model 1) and Option 2 (Cluster model 2). The parameters are shown in the following table.

Table 6.1-3: PMI simulation assumptions for cluster model 1

| Parameters | Values |
| --- | --- |
| Antenna setup: Ntx1, Ntx2, Nrx1, Nrx2 | (4,1,2,1) |
| Number of clusters | 4 |
| Cluster powers p1, p2 | [0dB,0dB,-9dB,-9dB] |
| Per cluster configuration | XPL-High, TDLC300-10 |
| Cluster TX steering in 1st array dim θtx1(1),θtx1(2),θtx1(3),θtx1(4) | θtx1(1)=0, θtx1(2)=−π/2, θtx1(3)=−π, θtx1(4)=−3π/2 |
| Cluster RX steering in 1st array dim θrx1(1),θrx1(2),θrx1(3),θrx1(4) | θrx1(1)=0, θrx1(2)=−π/2, θrx1(3)=−π, θrx1(4)=−3π/2 |

Table 6.1-4: PMI simulation assumptions for cluster model 2

| Parameters | Values |
| --- | --- |
| Antenna setup: Ntx1, Ntx2, Nrx1, Nrx2 | (4,1,2,1) |
| Per cluster configuration | XPL-High, TDLC300-10 |
| Cluster-tap mapping | Table 6.1-5 |

Table 6.1-5: Cluster model 2 with 4 TX-RX clusters

| Tap | Delay(ns) | Power | Beam steering |
| --- | --- | --- | --- |
| 1 | 0 | -6.9 | Cluster 1 θtx1(1)=0 θrx1(1)=0 |
| 2 | 65 | 0 |  |
| 3 | 70 | -7.7 | Cluster 2 θtx1(2)=−π/2 θrx1(2)=−π/2 |
| 4 | 190 | -2.5 |  |
| 5 | 195 | -2.4 |  |
| 6 | 200 | -9.9 |  |
| 7 | 240 | -8.0 |  |
| 8 | 325 | -6.6 |  |
| 9 | 520 | -7.1 | Cluster 3 θtx1(3)=−π θrx1(3)=−π |
| 10 | 1045 | -13.0 |  |
| 11 | 1510 | -14.2 | Cluster 4 θtx1(4)=−3π/2 θrx1(4)=−3π/2 |
| 12 | 2595 | -16.0 |  |

# 3 Conclusion

This draft TP captures comparison of PDSCH performance under SU-MIMO scenario for different spatial channel model part.