# 3GPP TSG-RAN WG4 Meeting #115                                R4-2506960

# Malta, MT, 19th – 23rd May, 2025

# Title: 	Discussion on demodulation CSI reporting requirements for CSI prediction

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.21.7

# Document for:	Approval

# 1 Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. The objectives for RAN4 are shown in following table.

- Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift

During the last meeting, some conclusions has been reached for CSI compression and CSI prediction[2]. In the following section, we will provide the detailed discussions for CSI prediction.

# 2 Discussion

In last meeting, RAN4 achieved the following agreement for CSI prediction[2].

Issue 1-1: Codebook to be used in the tests Agreement: - In Rel-19 only PMI requirements based on AI/ML CSI prediction will be defined - use Rel-18 eType II Doppler codebook for AI/ML based CSI predictions Issue 1-2: Requirement baseline Agreement: - Requirement to be introduced for Dopler domain basis N4=1 - N4>1 can be further discussed in the future if needed Issue 1-4: Test metric Requirements to be defined based on the ratio (same metric as for the current PMI reporting requirements in 38.101-4) - tue_AI/ML is 90 % of the maximum throughput obtained at SNRue_AI/ML [a given SNR] using the precoders configured according to the UE reports, - trnd is the throughput measured at SNRue_AI/ML [the same SNR] with random precoding. - trnd is obtained with Rel-15 type I single panel codebook - Note: SNRue_AI/ML is calculated in the same way as SNRue and SNRfollow1,follow2 in section 6.3 of 38.101-4

According to the previous process, RAN4 still needs to provide 2nd round of CSI prediction simulation results in RAN4#115 meeting. The test metric still uses SGCS in order to verify whether a reference model is needed for CSI prediction. From this point of view, we use single training dataset and mixed dataset to train AI model. The test datasets use a single datset. And other configuration, we followed the last meeting’s conclusion[3], focusing mainly on generating the training dataset, and did not restrict model assumptions. We followed the assumptions, and our results of SGCS can be found in the following Table 1. Compared to the legacy approach, RAN4 needs to discuss how to infer convergence based on the results, whether to follow the legacy method, if most companies results are within a certain range. For legacy approach, without no doubt, that the results could be aligned, which only needs to remove the results of the span. Therefore, if RAN4 wants to align the results, a potential criteria may be needed.

Table 1. Simulation results of SGCS for CSI prediction

| Duplex Mode and SCS | CHBW | MIMO | Training dat | Test data | SGCS | Nearest historical CSI |
| --- | --- | --- | --- | --- | --- | --- |
| FDD 15KHz SCS | 10 MHz | 16T4R | 20Hz Doppler | 20Hz Doppler | 0.96 | 0.81 |
|  |  |  | 100Hz Doppler | 100Hz Doppler | 0.94 | 0.81 |
|  |  |  | Mixed dataset (20Hz, 50Hz, 100Hz) | 20Hz Doppler | 0.92 | 0.81 |
|  |  |  |  | 100Hz Doppler | 0.91 | 0.80 |

Observation 1. From our simulation results, it can be observed that a little performance degradation when using mixed dataset training and testing on single dataset.

# 3 Conclusion

In this contribution, we give some detailed discussions on testability and interoperability issues for CSI prediction , The conclusions are:

Observation 1. From our simulation results, it can be observed that a little performance degradation when using mixed dataset training and testing on single dataset.

# 4 References

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface.

- R4-2505105, WF on requirements for AI/ML air interface normative work. Qualcomm.

- R4-2505229, Updated simulation assumptions for AIML CSI prediction. OPPO, CAICT, vivo, Nokia, Ericsson, Qualcomm, Huawei, Hisilicon, Mediatek, CATT, Samsung, ZTE Corporation, Apple.

# 5 Appendix

Table 1: Baseline Simulation assumptions for AI/ML based CSI prediction

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, TDD, OFDM |
| Carrier frequency | 2GHz for FDD 4GHz for TDD |
| Bandwidth | 10MHz for FDD, 40MHz for TDD |
| Symbol | 1 symbol |
| Number of subbands | 13 subbands, 4RB as a bundle for FDD 14 subbands, 8RB as a bundle for TDD |
| Subcarrier spacing | 15kHz for 2GHz, 30kHz for 4GHz |
| gNB TX antennas | Option1: 32 (N1,N2)=(4,4) as defined in Rel-15 type1 Option2: 16 (N1,N2)=(4,2) as defined in Rel-18 CSI prediction requirements |
| UE RX antennas | 4 |
| Channel model | TDLA30 as baseline with XP medium correlation |
| Doppler spread | 20Hz, 50Hz and 100Hz |
| Delay spread | 30ns |
| Channel estimation | Start with Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Further consider practical channel estimation in the next step - Use a common CSI-RS configuration as in demodulation test (may need to define/choose a CSI-RS configuration) |
| Rank per UE | Rank 1 and 2 |
| CSI feedback assumption | Step1: check the CSI prediction (outputs Raw channel) performance, do not need to take CSI feedback into account Step2: take CSI feedback into account (wait more related agreements in RAN4) - Option 1: Use Rel-18 eTypeII-Doppler - Companies are encouraged to check RAN1 agreements |
| CSI feedback assumption | Step1: check the CSI prediction (outputs Raw channel) performance, do not need to take CSI feedback into account Step2: take CSI feedback into account (wait more related agreements in RAN4) - Option 1: Use Rel-18 eTypeII-Doppler - Companies are encouraged to check RAN1 agreements |
| CSI feedback overhead | Option1: Parameter combination = 2 (PC2) Option2: PC4 Option3: PC6 Note: this part relates to step2, may be down selected later |
| Baseline | Random PMI with Rel-15 Type I single panel codebook (for Step 2 or performance requirements) |
| KPI | Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2), - comparing the SVD of model output with SVD of the ground truth Raw channel Note: - Step1: check SGCS before CSI feedback (between the raw channel matrix and ground-truth at UE) - FFS Step2: check SGCS (or other metrics, e.g. TPs) after CSI feedback |
| Model input type | Raw channel matrix |
| Model out type | Raw channel matrix |
| Observation window | Observation window (number/distance): 5/5ms as baseline Optional for 10/5ms |
| Prediction window | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms |