3GPP TSG-RAN WG4 Meeting #115	R4-2506974

# Malta, MT, 19th – 23rd May, 2025

Title:	On additional Test Tolerance for STxMP testing

Source:	Samsung

Agenda item:	7.15.3

Document for:	Discussion

1.	Introduction

In last RAN4 meeting, MU assessment was discussed focusing on how to accommodate the difference from the core requirement derivation and test system restrictions including finite resolution, the agreement was as following [1, R4-2504727]:

Sub-topic 2-1: MU assessment Issue 2-1-1: How to accommodate the difference from the core requirement derivation and test system restrictions including finite resolution. - Agreement: - The value for the uncertainty source of systematic error related to beam peak search is set as 0. The impact due to finite testing directions including limited choices for AoA1&AoA2 test directions can be handled in an additional Test Tolerance, TTFiniteRes. This additional TT should be determined in RAN4 and then communicated to RAN5 as recommended value to be added to the TT determined as a function of MU, i.e., TTTotal = x*MUTotal+ TTFiniteRes where x is determined in RAN5.

So the additional Test Tolerance due to finite testing directions including limited choices for AoA1&AoA2 test directions (TTFiniteRes) should be determined in RAN4 and then recommend to RAN5. In this contribution, we share our initial views on this additional test tolerance.

2. 	Discussion

Finite testing directions includes two aspects, one is the finite measure grip step size, the other is the limited AoA1&AoA2 selection under a non-full-freedom test system.

According to the conclusion of AoA pair (AoA1 and AoA2) selection, AoA1 need to be aligned with measurement grid point for beam peak search, and AoA2 selection is in a 30degree granularity [2].

After RAN4 discussion, the conclusion on AoA pair (AoA1 and AoA2) selection is as following which is a combination of Option 1 and Option 2: - If UE declares the AoA1, then a) The declaration of AoA1 direction can be any direction but need to be aligned with the measurement grid of beam peak search of single CC without STxMP. b) AoA2 direction is to use AoA1 + or - UE declared offset from set of {30deg, 60deg, 90deg, 120deg, 150deg} in xz plane of test system c) AoA1+ UE declared offset or AoA1 - UE declared offset needs to be declared by UE d) If AoA1 and AoA2 directions pass EIRP PUMAX,f,c,k requirements, the DUT pass the test. - If UE doesn’t declare AoA1, then a) the default AoA1, i.e., the beam peak direction of single CC operation without STxMP is applied b) AoA2 direction is to use AoA1 + and - UE declared offset from set of {30deg, 60deg, 90deg, 120deg, 150deg} in xz plane of test system c) AoA2 direction is AoA1 + the declared offset d) AoA2 direction is AoA1 - the declared offset e) If AoA1 and any of the two AoA2 directions pass EIRP PUMAX,f,c,k requirements, the DUT pass the test. Note that the above AoA pair (AoA1 and AoA2) selection is based on the UE declared UE orientation. mance metric for dynamic MIMO OTA to reflect an overall performance of a UE.

TTFiniteRes for AoA1 could be same as legacy for beam peak search which is 0.7dB. For AoA2, the additional test tolerance is originated from the AoA offset in 30deg granularity on top of 0.7dB.

To evaluate the test tolerance originated from the limited AoA offset, the simulation data for Multi-RX can be taken as a reference.

Taking the two panels in adjacent faces as example, the simulation data is shown as below table.

|  | Panel1 AoA1 |  |  | Panel2 AoA2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Theta | Phi | Gain | Theta | Phi | Gain |
| Beam peak | 92 | 88 | 12.13 | 156 | 90 | 12.00 |
| AoA1&AoA2 | 90 | 90 | 12.11 | 150 | 90 | 10.53 |
|  |  |  |  |  |  | ~1.5dB |

Up to 1.5dB tolerance is observed due to the limited AoA offset. So for AoA2 TTFiniteRes should be around 1.5dB + 0.7dB = 2.2dB.

Proposal 1:	TTFiniteRes is 0.7dB for AoA1, and 2.2dB as starting point for AoA2

3. 	Conclusion

Proposal 1:	TTFiniteRes is 0.7dB for AoA1, and 2.2dB as starting point for AoA2

4. 	References

- R4-2504727    “WF for [114bis][329] NR_FR2_OTA”, Qualcomm

- R4-2504061     TR 38.771 v0.3.0