3GPP TSG-RAN4 Meeting #115	R4-2506980

# Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.1.2.1.1

Source:	MediaTek

Title:	MSD simplification rules for HPUE CA/DC UE

Document for:	Approval

# Introduction

In RAN4#114bis meeting, the WF[1] has been approved and recommended to further discuss and encourage more companies’ input for each open issues i.e., MSD simplification and others for HP CA/DC UE.

Way forward [1]: WF on 1 band 1UL CC HPUE MSD simplification Agreement 1 on simplification Look-Up Tables (LUTs): - Scope: In Rel-19, the 1 band UL 1CC HPUE MSD simplification only applies to PC2 1Tx, PC2 2Tx and PC1.5 achieved with 2Tx. Other Rel-19 1UL HPUE power classes are out of scope, e.g. PC1. - For new Rel-19 inter-band CA combinations, and for all valid 1UL HPUE configurations, specify the 1 band 1 UL CC HPUE MSD requirements due to uplink harmonic, harmonic mixing, cross-band isolation caused by the same aggressor UL band as: PCx MSD = PC31Tx MSD + MSD where, - x = 2 1Tx, 22Tx, or 1.52Tx and, MSD is specified in three LUT columns with a maximum MSD (“MSDmax”) of 3, 6 and 9dB for harmonic, cross band isolation, harmonic mixing. One example of such LUT is captured below. Coarser granularity and other tabulated values are not precluded. PC31Tx MSD (dB); MSDmax (dB). 3; 6; 9. 0.1≤ ≤0.3; 0.3; 0.4; 1.0. 0.4≤ ≤0.6; 0.5; 0.8; 1.8. 0.7≤ ≤0.9; 0.7; 1.1; 2.5. 1.0≤ ≤1.9; 1.2; 2.1; 4.1. 2.0≤ ≤2.9; 1.6; 2.7; 5.1. 3.0≤ ≤3.9; 1.8; 3.3; 5.9. 4.0≤ - ≤4.9; 2.0; 3.8; 6.5. 5.0≤ - ≤6.9; 2.3; 4.5; 7.3. 7.0≤ - ≤9.0; 2.5; 5.0; 7.9. ≥10; 3; 6; 9. - and where the mapping of the 1UL MSD type to the “MSDmax” LUT is specified with the following companion table. MSD type; MSDmax (dB). 3; 6; 9. 1UL; UL harmonic; PC21Tx; PC22Tx; [PC1.52Tx]. Harmonic mixing; PC21Tx; PC22Tx; PC1.52Tx. Cross-band isolation; PC21Tx; PC22Tx; PC1.52Tx. - It should be noted that for some band combinations, LUTs may represent a slight tightening of the PC22Tx/PC1.52Tx MSD requirements, especially for UL harmonic, while for other band combinations LUTs may represent a slight relaxation of the PC22Tx/PC1.52Tx MSD requirements, especially for harmonic mixing. - Agreement 2 on handling band combinations without MSD for PC3 but with small PC2/PC1.5 MSD: - For new HPUE band combinations, do not specify the MSD requirements for 1UL HPUE PC21Tx PC22Tx and PC1.5 (2Tx). - For legacy PC3 band combinations (i.e. band combinations specified prior to the agreement of the MSD simplification CR for 1UL HPUE), where no PC3 MSD is specified, companies are encouraged to browse through these band combinations and bring correction CRs if 1UL HPUE MSD requirements are needed. Background on the companion table. This table is needed for two reasons: - to anticipate future 1UL power classes and / or new MSD requirements. For example, at the time of writing, in Rel-19 there are no PC1.52Tx MSD requirements due to UL harmonic interference. Hence, no mapping exists for this case yet. In anticipation of these new 1UL CC HPUE MSD requirements, it is proposed to capture this mapping using square brackets. - to facilitate the possible introduction of a single “unified” mapping table for both 1UL and 2UL HPUE MSD simplification [9]. An extension of the 1UL companion table is anticipated to address 2UL mapping cases of Rel-19 different power classes. WF on 2 bands 2UL CC HPUE MSD simplification Agreement: - 2UL HPUE MSD requirements can be mapped to a separate LUT with maximum of [8] columns, e.g. at least with ΔMSDmax = [3], 6, 9, 12, 15, [18, 24, 30] dB. - Other values are not precluded and the need for additional columns is FFS. - FFS on optimized mapping to avoid penalizing 2UL combos with too high ΔMSDmax.

In this paper, we we share our view on the MSD simplification methodology according to 1UL and 2UL interference sources in Rel-19.

# MSD simplification

## 1UL HPUE MSD simplification

In Rel-19, RAN4 can apply the following MSD simplification rules for new Rel-19 high power inter-band CA/DC band combinations within FR1 based on the 1UL PC3 MSD requirements due to uplink harmonic, harmonic mixing, and cross-band isolation interference, are specified as:

- PC21Tx MSD = PC31Tx MSD + PC21Tx,

- PC22Tx MSD = PC31Tx MSD + PC22Tx,

- PC1.5 MSD = PC31Tx MSD + PC1.5.

Table 1: 1UL Delta MSD Table according to power classes [1]

Proposal 1: For the 1UL MSD simplification rules of HPUE inter-band CA/DC UE, RAN4 can apply the above LUT based MSD simplification approach due to harmonic, harmonic mixing and crossband isolation problems.

## 2UL HPUE MSD simplification

For 2UL MSD analysis, the IMD products’ power is mainly influenced by factors such as changes in transmit power, variations in the power class of each PA, the number of TX paths, and the IMD index of the frequency bands involved in intermodulation distortion. Also, HP PA improvement in linearity can bring about by the increase will be leaded to a reduction in product power.

In Agreed WF on MSD simplification rules in RAN4 #113 meeting, RAN4 derived detail MSD calculation principle for 10 cases for total CA UE power class in Table 2 according to the number of Tx Chain, PA combinations and expected interference levels in own Rx band.

Table 2. HPUE MSD analysed table from Rel-15 to Rel-19

| case |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ULCA power class | Type | PC3 2Tx | PC2 2Tx | PC2 2Tx | PC2 3Tx | PC2 2Tx | PC2 3Tx | PC1.5 2Tx | PC1.5 3Tx | PC1.5 3Tx | PC1.5 3Tx |
|  | PCmax | 23 | 26 | 26 | 26 | 26 | 26 | 29 | 29 | 29 | 29 |
| per band power class | Bd1 | PC3 1Tx | PC3 1Tx | PC3 1Tx | PC3 1Tx | PC2 1TX | PC2 1TX | PC2 1TX | PC2 1TX | PC3 1Tx | PC2 1TX |
|  | Bd2 | PC3 1Tx | PC3 1Tx | PC2 1Tx | PC2 2Tx | PC2 1Tx | PC2 2Tx | PC2 1Tx | PC2 2Tx | PC1.x 2Tx | PC1.5 2Tx |
| Valid Release |  | R15 | R17 | R17 | R18 | R19 | R19 | R19 | R19 | R18 | R19 |
| 1UL power | Bd1 | 23 | 23 | 23 | 23 | 26 | 26 | 26 | 26 | 23 | 26 |
|  | Bd2 | 23 | 23 | 26 | 2x23 | 26 | 2x23 | 26 | 2x23 | 2x26 | 2x26 |
| 2UL power | Bd1 | 20 | 23 | 23 | 23 | 23 | 23 | 26 | 26 | 23 | 26 |
|  | Bd2 | 20 | 23 | 23 | 2x20 | 23 | 2x20 | 26 | 2x23 | 2x24.7 | 2x23 |
| 2UL interference level |  | Ref | +3xorder | <+3xorder | <+3xorder | +3dB | +3dB | +3dB +3xorder | +3dB +3xorder | <4.7xorder -1.7 | ≤+3dB +3xorder |
| Unified 2UL interference level |  | Ref | 3xorder |  |  | 3dB |  | +6xorder -3 |  | <4.7xorder -1.7 | +6xorder-3 |

Based on SKW proposal [5] for 2UL MSD simplification, RAN4 can consider the IMD product level will be increased as proportional by the IMD order and we can ignore the asymmetry IMD of UL band to derive the MSD analysis.

To consistency of 1UL and 2UL MSD simplification rules, we prefer to define LUT per each power class according to the IMD order as follow

The original MSD proposed tables from [3]. It can be updated Table format as below to consistent with 1UL MSD table.

Table 3: 2UL Delta MSD Table for Power class 2 according to IMD order [3]

Table 4: 2UL Delta MSD Table for Power class 1.5 according to IMD order [3]

- PC2IMDx MSD = PC3IMDx MSD + PC2IMDx,

- PC1.5IMDx MSD = PC3IMDx MSD + PC1.5IMDx.

Proposal 2: For the 2UL MSD simplification rules of HPUE inter-band CA/DC UE, RAN4 can apply the above LUT based MSD simplification approach due to dual uplink IMD problems.

Proposal 3: For the configurations where PC3 MSD is not specified, the 1UL or 2UL HPUE MSD requirements can be specified in the legacy MSD tables 7.3A.4-2a, 7.3A.4-4a-1, 7.3A.4-4b, 7.3A.6-1a-1, 7.3A.6-1b.

- Conclusion

Observation 1: For harmonic products with option 1’s equation, the PC2 1TX calculated ∆MSD has a good fit for most cases, the PC2 2TX calculated ∆MSD is a little bit lower than the spec value at high PC3 MSD.

Proposal 1: For the 1UL MSD simplification rules of HPUE inter-band CA/DC UE, RAN4 can apply the above LUT based MSD simplification approach due to harmonic, harmonic mixing and crossband isolation problems.

Proposal 2: For the 2UL MSD simplification rules of HPUE inter-band CA/DC UE, RAN4 can apply the above LUT based MSD simplification approach due to dual uplink IMD problems.

Proposal 3: For the configurations where PC3 MSD is not specified, the 1UL or 2UL HPUE MSD requirements can be specified in the legacy MSD tables 7.3A.4-2a, 7.3A.4-4a-1, 7.3A.4-4b, 7.3A.6-1a-1, 7.3A.6-1b.

## References

- R4-2505143, WF on HPUE MSD simplification, Skyworks, Qualcomm, Murata, Nokia, Samsung, Huawei, MediaTek, vovo, ZTE, RAN4 #114bis

- R4-2501955, 1UL MSD simplifications rules for Rel-19, Skyworks, Nokia RAN4#113

- R4-2503362, Discussion on MSD simplification of HPUE CA/DC UE, MediaTek, RAN4 #114bis

- R4-25xxxxx, RAN4 #114bis Meeting report, RAN4 Chairman, RAN4 #115

- R4-2500218, Simplification of HPUE 2UL DeltaMSD, Skyworks, RAN4#114

- R4-2501579, MSD beyond PC3, Qualcomm, RAN4#114