3GPP TSG RAN WG4 Meeting #115		R4-2506981

Malta, Malta, 19th – 23rd May 2025

Source: 		Rohde & Schwarz

Title: 			Discussion on test setup for AI/ML based beam management

Agenda Item: 	7.21.4

Document for: 	Discussion

# Introduction

From the WF in [16], the following analysis is required with regards to the test setup for AI/ML based beam management:

Issue 2-6: Test system setup

Companies are invited to bring further analysis on the test system setup.

At least the following aspects to be considered in the analysis:

    - Whether the test can/should reflect performance in the field and whether field data should be usable in the test

    - What channel model should be used

    - UE prediction model input (e.g. whether any spatial information like UE Rx beam, UE beam pattern, etc is used)

    - Pros and cons of proposed single AoA and multi AoA setups

For the multi AoA test setups, companies are invited to bring further analysis on the following aspects:

  - Define candidate test setups which are already feasible (e.g. based on eIFF, MPAC etc.)

  - Propose simplified channels suitable for the defined candidate test setup(s)

  - Validate the proposed simplified channels through AI/ML BM system level simulations, especially by comparing the performance with the non-simplified (theoretical) channels.

  - Assess the implementation of the simplified channels on the candidate systems, after the probe layout and the channel model have been clarified.

In this contribution, we elaborate further our views on single-AoA and multi-AoA test setups.

# Discussion

## Real data for training and testing

One important aspect discussed in the last meeting was training the model and testing AI/ML BM with real data. In our view this has lots of implications.

From the requirement point of view, we need to clarify whether the content of such real data is standardized or not. In case it is not, every UE vendor would probably prefer to test with different data, which put the focus of the requirement rather in the training performance of the model, and less on its prediction performance. Especially in this case, we would not refer to a standardized and deterministic testing environment, which is one of the key assumptions for conformance testing.

Observation 1: (Real Data) If the real data for training the model and testing AI/ML BM is not standardized, the focus moves from prediction performance, to training performance. Such requirements would also not refer to a deterministic testing environment, which is key assumption for conformance testing.

For a typical conformance testing with focus on the prediction performance, we need to standardize the data, but then the challenge is to agree on the data i.e. also on the respective scenario.

Observation 2: (Real Data) For a typical conformance testing of prediction performance, standardized real data for training and testing is needed, which shifts the challenge to agree on the data and the respective scenario.

From the testability point of view, training and testing with real data would be replicated only in a single-AoA system using the wireless cable method with the UE complex antenna pattern (Reference: Section 2.1.1 in [12]). A multi-AoA implementation would be rather not feasible, since a multi-AoA system has a static probe layout, which is not suitable for every real data scenario.

Observation 3: (Real Data) Training and testing with real data can be replicated only in a single-AoA system (using the wireless cable method with the UE complex antenna pattern), since a multi-AoA system with fixed probe layout cannot mimic the variety of the real data and scenarios.

Furthermore, we need to define an interface to handle how the real data is fed in the test setup, as well as how it is then deployed. This indirectly requires the format definition for the data.

Observation 4: (Real Data) Training and testing with real data requires a standardized data format and interface to the test system, as well as a description how this data is deployed by the test system.

Based on the above, it is recommended to support the usage of real data for training and testing only in the form of 1AoA testing and for R&D purposes, as a complement to the conformance requirement defined with synthetic channels.

Proposal 1: (Real Data) Support of real data training and testing is recommended only in the form of 1AoA testing and for R&D purposes, as a complement to the conformance requirement defined with synthetic channels.

## Single vs. Multi AoA testing

The discussion whether single- or multi-AoA can be used for AI/ML BM requirement and testing is still in ongoing. In our last contribution in [13] we identified several aspects, mostly related to the UE Rx antenna assumptions and consideration, which in our view are the key to conclude about the need of a single-AoA or a multi-AoA solution. These aspects were a) UE Rx antenna pattern knowledge, b) UE Rx antenna performance, c) UE Rx beam sweeping performance, d) UE Rx beam index (as input to the AI/ML model).

In all the cases the basic assumption is that the required channel is a CDL-based model, which means that for a single-AoA implementation, the UE Rx antenna pattern of the UE should be known (agreed / declared / measured) and emulated as part of the channel by the TE in a “wireless cable mode” (WCM) connection between TE and DUT.

In addition, as discussed in section 2.1.1, real data training and testing is also an aspect of interest which is relevant for the single-/multi-AoA discussion.

Observation 5: (Single/Multi-AoA) Several aspects related to the UE Rx Antenna assumptions, the input to the AI/ML model and the real data usage for training and testing, have a major impact on the discussion about single- or multi-AoA testing.

For a better overview, we are summarizing all the aspects above in a tabular format in Table 1.

Table 1 – Single vs. multi AoA testing for AI/ML BM with CDL based channel

| Parameter | Single AoA | Multi AoA | Notes |
| --- | --- | --- | --- |
| CDL Channel Support | Only if UE Rx antenna performance is emulated as part of the channel (WCM*) | Direct, clusters are emulated on different probes / AoAs | * Wireless Cable Method |
| CDL Channel Complexity | Complex Channel (with WCM) | Simplified Channel |  |
| UE Rx Antenna Pattern Knowledge | Required for WCM (reference / declared / measured) | Not required |  |
| UE Rx Antenna Performance | Static pattern during testing simulating the dynamic pattern as part of the WCM | Dynamic pattern |  |
| UE Rx Beam Sweeping Timing | Not considered | Considered |  |
| UE Rx Beam Index as input for AI/ML Model | Supported with test mode* | Supported | * Depends on the complexity of the UE Rx Beam/Antenna pattern (Codebook) considered for single AoA testing |
| Training / testing with real data | Supported with WCM | Unfeasible (fixed probe layout) |  |
| TE Complexity | Low HW Complexity Longer test time WCM & Test mode required | High HW Complexity Shorter test time |  |

RAN4 needs to discuss and conclude on these aspects as part of the single-/multi-AoA discussion.

Proposal 2: (Single/Multi-AoA) RAN4 to discuss and conclude on the aspects in Table 1 as part of the single-/multi-AoA discussion.

## Multi-AoA Testing

### Candidate system

The first step as per the the WF in [16] is to propose a multi-AoA candidate system. As already mentioned in our last contribution in [13], the current definition of the multi-AoA legacy RRM baseline system based on IFF is called eIFF (enhanced IFF) and is specified in TS 38.508-1 [7], Annex B.2.6. It prescribes the support of 2 simultaneous active probes with relative angular offsets of 30°, 60°, 90°, 120° and 150°. These are required for the test cases using Angular Setups 1-5 in TS 38.133 [6], Annex A.3.15.1-5.

However, as described in our previous contribution in [12], most current commercial implementations of eIFF test systems deploy 4x CATRs in a single plane with the layout (or an equivalent) as shown in Figure 1. This is the resulting layout to support all the required angular offsets (5) with the minimal number of static CATRs (4) as shown in Figure 2.

Figure 1 – Example of a commercial implementation of eIFF system with 4 IFF probes / CATRs

Figure 2 – Example of a commercial implementation of eIFF system supporting all (5) the required angular offsets with the minimal number (4) of static CATRs

The implicit understanding of such implementation has been already used to define the Angular Setups 6-7 in TS 38.133 [6], Annex A.3.15.6-7. These angular setups require up to 3 active probes and are used in the test cases defined for Enhanced MIMO and Multi Rx test cases (Rel-19).

As to multi-AoA CDL emulation, the eIFF system can be further enhanced to enable the simultaneous activation of all the 4 available probes / CATRs. For the purpose of AI/ML BM testability discussion, this system with 4 static CATRs placed in a single plane and separated by 30º, 60º and 60º respectively can be called “feIFF” (further enhanced Indirect Far Field). This enhancement (from 2 to 4 simultaneous probes) is feasible, thus we can consider the feIFF system as a candidate to be evaluated for the AI/ML BM multi-AoA baseline system.

How to define the concrete probe layout and the alignment with the coordinate system to be used for testing, can be further discussed. However, this does not prevent to continue the feasibility assessment, when defining the simplified CDL channel models or the system level simulations, as presented in next sections.

Proposal 3: (Multi-AoA Candidate Setup) Define the implementation of eIFF test setup with 4x active 2x2 MIMO probes with probes (CATRs) placed in a single plane and separated by 30º, 60º and 60º respectively, as feIFF (further enhanced IFF) test setup. The concrete probe layout and the alignment with the coordinate system to be used for testing is FFS.

In summary, the feIFF test setup has the following technical capabilities and advantages:

- 4 Active 2x2 MIMO probes with large distinctive angular separation of max 150°  ensures substantial evaluation of UE spherical coverage

- Big quiet zone of at least 30cm for all AoAs  real black box testing

- Best far field conditions with lowest MU (compared to DFF-based systems)  highest (L1-RSRP) accuracy measurements

- Reference for legacy requirements  side conditions (RF-RRM / cross-feature) ensured

Observation 6: (Multi-AoA Candidate Setup) The feIFF test setup supports 4 Active 2x2 MIMO probes with large distinctive angular separation of up to 150°, overall big quiet zone of at least 30 cm, lowest MUs and ensures preservation of (RF-RRM) side conditions.

In addition, the feIFF test setup has practical advantages as a superset of the eIFF setup:

- eIFF is available and matured as already broadly used for FR2 testing

- eIFF supports all RF/ Demod / RRM FR2 tests up to release 19 (incl. Multi-Rx, eMIMO)

- If feIFF concluded to be suitable for AI/ML  one single system for full conformance testing coverage

- Continuity (eIFF  feIFF) in the cost & complexity-sensitive FR2 testing ecosystem

Observation 7: (Multi-AoA Candidate Setup) The feIFF test setup is a feasible upgrade (superset) of the existing legacy eIFF setup. As such, if feIFF is concluded as capable for AI/ML BM testing, a single system covers the the whole conformance testing, ensuring continuity in the cost sensitive FR2 testing ecosystem.

Based on all the advantages described above, we propose the feIFF setup to be a candidate for the AI/ML BM baseline system.

Proposal 4: (Multi-AoA Candidate Setup) Consider the feIFF test setup as candidate for the AI/ML BM baseline system and analyze its suitability.

### Simplified CDL-C channel model

The next step as per the WF in [16], is to define a simplified CDL based channel model supported by the candidate system.

For this purpose, as already presented in our last contribution in [13], using the UMi CDL-C channel model Table 7.2.2-3 from TR 38.827 [3] as a reference, the following simplifications are applied:

- The clusters that have the same AoAs can be merged, following Observation 11 in [15]. To do so, the delay for the first tap is kept and the power of the three taps is combined, resulting in +3dB for clusters #2 and #6.

- The weak clusters that have a limited impact on the beam management (i.e. <-10dB) can be removed, following Observation 8 to Observation 10 in [15], in order to reduce the number of clusters.

Implementing #1 results in a power scaling of all clusters. This is done by adjusting the power for the strongest cluster (i.e. #6) to 0dB and reducing the remaining clusters by the difference to cluster #6. With this scaling, there are more clusters falling below -10dB from the strongest cluster and then they are also removed. The resulting channel model parameters are presented in Table 2.

Table 2 – Channel model parameters for UMi CDL-C at 28 GHz 
after merging clusters with similar AoAs and removing weak clusters

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -134.4434 | 98.9242 | 83.3318 |
| 2 | 12.594 | -1.2500 | -20.9269 | 129.1633 | 99.1915 | 72.5229 |
| 5 | 13.056 | -5.5318 | -28.0782 | -152.8206 | 99.5732 | 71.1282 |
| 6 | 38.196 | 0.0000 | -11.6982 | 164.1145 | 99.306 | 74.7544 |
| 13 | 73.71 | -8.1318 | -33.911 | 93.1719 | 100.165 | 85.4238 |
| 14 | 78.498 | -9.8318 | -37.5066 | -112.0441 | 100.2604 | 64.1548 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 10.4021 | 0.5726 | 4.8814 | 7 |  |

The maximum angular spread required to emulate the down selected clusters in Table 2 is close to 150°, which can be covered by the probe layout of the proposed candidate setup feIFF shown in previous section.

Observation 8: (Multi-AoA Simplified Channel) The required total AoA spread of 150° of the simplified channel in Table 2 can be emulated with the feIFF probe layout.

Looking at further channel model simplifications to fit the candidate test system feIFF described in the previous section, there is one clear change to be implemented: the elevation spread (i.e. ZoA) and the intra-cluster elevation angle spread (i.e. CZSA) must be flattened, to fit the probe layout placed in a single plane. Further discussion could determine whether a flat 90º (i.e. as done for FR1 MIMO OTA as described in TR 38.827 [3]) should be used, or it could be possible to select the weighted average of the ZoA for the remaining clusters (i.e. 74.51º). In Table 3, the weighted average is used for further discussion. It can be decided later whether this single elevation is tested in the chamber by selecting the appropriate DUT testing conditions/orientations.

Table 3 – Channel model parameters for UMi CDL-C at 28 GHz with flat ZoA

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -134.4434 | 98.9242 | 74.51134 |
| 2 | 12.594 | -1.2500 | -20.9269 | 129.1633 | 99.1915 | 74.51134 |
| 5 | 13.056 | -5.5318 | -28.0782 | -152.8206 | 99.5732 | 74.51134 |
| 6 | 38.196 | 0.0000 | -11.6982 | 164.1145 | 99.306 | 74.51134 |
| 13 | 73.71 | -8.1318 | -33.911 | 93.1719 | 100.165 | 74.51134 |
| 14 | 78.498 | -9.8318 | -37.5066 | -112.0441 | 100.2604 | 74.51134 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 10.4021 | 0.5726 | 0 | 7 |  |

Related to how to this channel model can be implemented in the candidate test system feIFF, we see 3 possible options:

- Option A: Using the simplified channel model in Table 3, the strongest cluster (i.e. cluster #6) is aligned to one of the available probes (i.e. 30º). In this approach, like done for MIMO OTA, the test system calculates the weights to simulate the AoAs in between the probes (i.e. reflectors). With this option, there is no need to further modify the channel model table. On the other hand, the main drawback is that there are several clusters falling outside the probe spread.

- Option B: Using the simplified channel model in Table 3, the mid-point of the total angular spread of the clusters is aligned to the mid-point of the spread of the probes (i.e. 75º). With this option, there is no need to further modify the channel model table, and all clusters (nearly) fall into the probe spread.  The main drawback is that the strongest cluster fall between probes separated 60º apart.

- Option C: Using Option B as starting point, AoAs in the channel model parameter table are modified to align with the closest probe location. Here, every cluster will be aligned with one probe location and allows the intra-cluster angle spread (i.e. CASA) to be set to 0º for a very much simplified channel model. This results in the simplified channel model in Table 4.

Table 4 – Channel model parameters for UMi CDL-C at 28 GHz
 with AoA aligned to probe layout and no intra-cluster angle spread

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -114.436 | 98.9242 | 74.51134 |
| 2 | 12.594 | -1.2500 | -20.9269 | 125.5639 | 99.1915 | 74.51134 |
| 5 | 13.056 | -5.5318 | -28.0782 | -174.436 | 99.5732 | 74.51134 |
| 6 | 38.196 | 0.0000 | -11.6982 | -174.436 | 99.306 | 74.51134 |
| 13 | 73.71 | -8.1318 | -33.911 | 95.5639 | 100.165 | 74.51134 |
| 14 | 78.498 | -9.8318 | -37.5066 | -114.436 | 100.2604 | 74.51134 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 0 | 0.5726 | 0 | 7 |  |

Observation 9: (Multi-AoA Simplified Channel) The simplified channel model parameters for UMi CDL-C can be implemented in the candidate test system feIFF, with 3 possible options (Figure 3): Option A: with flattened elevation (Table 3) and strongest cluster aligned to one probe; Option B:  with flattened elevation (Table 3) and mid-point of the total angular spread of the clusters is aligned to the mid-point of the spread of the probes; Option C: with flattened elevation, AoAs aligned to probe layout and no intra-cluster angle spread (Table 4).

Proposal 5: (Multi-AoA Simplified Channel) Consider the channel models described Table 3 (Option A/B) and Table 4 (Option C) as candidates for CDL-based simplified channel models for multi-AoA testing of AI/ML BM.

The resulting alignment of the clusters with respect to the example probe layout for feIFF shown in Figure 1 is presented in Figure 3. Please note that in the figure the channel model cluster AoAs have been transformed to the chamber probe coordinates, and that the strongest cluster (i.e. cluster #6) has been highlighted with a different marker (▲).

Figure 3 – Simplified UMi CDL-C at 28 GHz alignment to the candidate test system feIFF

Observation 10: (Multi-AoA Simplified Channel) The evaluation of Options A and B (Table 3) becomes more complex, if the effect of the test setup / probe layout has to be included.

In this case, it is required first to define the metric to compare with the channel model parameter table to be able to determine the probe weights accordingly.

It has to be noted that these probe weights are different per BS Tx beam, in order to simulate the resulting Power Angular Spectrum (PAS) based on the mean cluster power. The distribution of the PAS can be calculated by directly filtering the clusters in Table 3 with the full set of BS Tx beams (i.e. Set A). In addition to the mean PAS distribution, the power fluctuations over time for a given BS Tx beam from the different AoAs, is determined by the cluster angular spread, the intra-cluster angular spread and the relative ray angles, as well as the initial ray phases defined in TR 38.827 [3].

To emulate these power fluctuations properly, the relative ray phases should be emulated properly by the test setup. Any phase inaccuracy could reduce the validity of the test approach. To avoid this, a reliable relative phase emulation among the rays is required, which is ensured by using an IFF based approach.

Observation 11: (Multi-AoA Simplified Channel) The emulation of the relative ray phases with the feIFF is highly accurate and should satisfy the requirement for the accuracy for the cluster power fluctuations over time.

As mentioned in previous contributions ([8] - [13]), the metric used so far for MIMO OTA (which is CDL channel emulation centric) is not suitable and new metric(s) have to be defined.

These metric(s), should characterize the following aspects:

- Sufficient power variation in the angular domain, dependent on the BS Tx beam.

- Sufficient power variation in the time domain, for a given BS Tx beam.

- Power accuracy in the test volume.

Proposal 6: (Multi-AoA Simplified Channel) Further discuss the metric(s) to evaluate channel model implementation with the feIFF candidate test setup.

On the other hand, Option C (Table 4) presents an extreme case of the CDL simplification, where the clusters are fully aligned with the probes. As such, there is no need to optimize the probe weights, which was required above for Options A and B. Therefore, the implementation with different feIFF systems is straightforward and the performance of the AI/ML BM prediction with this simplified channel model can be evaluated right away, since the channel model parameters in Table 4 contain all the required information.

Observation 12: (Multi-AoA Simplified Channel) For Option C (Table 4), there is no need to optimize the probe weights, and channel model parameters in Table 4 contain all the required information. This option is thus preferred from a TE point of view.

### Preliminary validation through AI/ML BM Simulations

The third step as per the WF [16], is to evaluate the proposed simplified CDL channel for AI/ML BM suitability, by using system level simulations. Comprehensive simulations can and need to be conducted by UE vendors in order to include also UE related performance aspects like BB errors, RF impairments, eventually UE Rx beam sweeping and above all the implemented AI/ML model. However, similarly to our contribution in RAN4#113 [11] and RAN4#114bis [13], we have conducted our own simulations, as a preliminary evaluation of the proposed simplified channel. The detailed simulation assumptions are described in Table 5.

This time we have updated our simulation assumptions for testability to align as much as possible with simulation assumptions for the requirement as agreed in [17]. Due to the simulation capacity as well as the different scope, there are however certain deviations. Some of these deviations have already been mentioned in the Table 5.

Table 5 – Simulation configuration

| Parameter | Value | Notes |
| --- | --- | --- |
| Radio Link | 30 GHz center carrier 120 kHz SCS |  |
| Reference Signal | CSI-RS 16 PRBs Symbol location 6 Subcarrier location 0 CP length Normal Symbol per slot 14 | 48 PRBs used in [17] |
| SNR | 30 dB | Low SNR used in [17] |
| Antenna Assumptions | BS Tx Array 4x8 UE Rx Array 4x1 Single Polarization | Tx-Rx polarization match assumed |
| Channel Model Type | CDL-C Umi |  |
| Channel Model | 24 Clusters (Reference) 6 Clusters (Option A/B) 6 Clusters (Option C) | Table 7.2.2-3 from TR 38.827 [3] Table 3 Table 4 |
| Simulated Cases | 24/24 6/24 6/4 | x/y means: measured w/ x clusters channel, trained w/ y clusters channel |
| Set A Tx beams (BS Codebook) | 32 | As in Figure 4 |
| Set B Tx beams | 8 | Selection as in Figure 4 |
| Rx beams | 4 | Figure 5 |
| AI/ML Model | CNN (approx.150k parameters) | 1.04M parameters used in [17] |
| Input for the model | RSRP of measured Tx-Rx Beam Pair |  |
| Output of the model | 1 & 3 best Tx Beam IDs resulting from the probability of predicted Tx-Rx Beam Pairs |  |
| Ground Truth | 1 & 3 best Tx Beam IDs resulting from expected measured Tx-Rx Beam Pairs |  |

Figure 4 – Selection of 8 Set B Tx Beams to predict 32 Set A Tx Beams

Figure 5 – Selection of 4 Rx Beams

The prediction accuracy results have been summarized in Table 6 and visualized in Figure 6.

Table 6 – Prediction accuracy probability

|  | CNN 24/24 | CNN 6/24 - A/B | CNN 6/6 - A/B | CNN 6/24 - C | CNN 6/6 - C |
| --- | --- | --- | --- | --- | --- |
| Top 3 Set A Beams | 90.24% | 91.21% | 91.07% | 90.82% | 89.73% |
| Top 1 Set A Beam | 64.49% | 64.67% | 65.80% | 64.43% | 64.69% |

Figure 6 – Prediction accuracy probability

The results of these simulations, show that the prediction accuracy probability varies at a neglectable scale (approx. 1%) between cases with the full CDL-C channel with 24 clusters and simplified CDL-C channel with 6 clusters, in both variants described in previous section: Option A/B as per Table 3 and Option C as per Table 4.

Similar neglectable variation is also observed in cases when the prediction with the simplified CDL-C channel (6 clusters) is done, after being trained with this same simplified channel, or with the full CDL-C channel (24 clusters).

Observation 14: (Multi-AoA Channel Validation) Based on preliminary simulations with 8 Set B & 32 Set A beams, there is only a negligible effect (deviation of max ~1%) on the prediction accuracy for Top 3 & Top 1 beams between the cases with full CDL channel model (Reference - 24 clusters) and simplified CDL channel model suitable for feIFF (Options A/B/C - 6 clusters).

Based on these results, we would preliminarily consider the channel models for feIFF in Table 3 and Table 4 as suitable for the AI/ML BM testing.

Proposal 8: (Multi-AoA Channel Validation) Consider preliminarily the CDL-C simplified channel models for feIFF with 6 Clusters in Table 3 and Table 4 as suitable for AI/ML BM testing.

As mentioned before, further comprehensive system simulations are required in order to finalize the validation of the channel for the AI/ML testing. As part of this process an evaluation is required. In our view a similar approach can be followed as above i.e. compare the performance when using a full channel with the performance with the simplified channel. In addition, depending on the expectation how the model training is done, a comparison for the simplified channel case, can be done also when UE is pretrained with the full channel, or trained in the test with the simplified channel.

Proposal 9: (Multi-AoA Channel Validation) Start the validation of CDL-C simplified channel models for feIFF with 6 Clusters in Table 3 and Table 4 through AI/ML BM system level simulations.

Proposal 10: (Multi-AoA Channel Validation) For the scope of validation, compare the prediction accuracy between the full channel and the simplified channel.

Proposal 11: (Multi-AoA Channel Validation) For the scope of validation, compare the prediction accuracy, when the model is trained with the full channel before the test and with the simplified channel during the test.

# Conclusions

In this contribution, the following observations and proposals are made:

Observation 1: (Real Data) If the real data for training the model and testing AI/ML BM is not standardized, the focus moves from prediction performance, to training performance. Such requirements would also not refer to a deterministic testing environment, which is key assumption for conformance testing.

Observation 2: (Real Data) For a typical conformance testing of prediction performance, standardized real data for training and testing is needed, which shifts the challenge to agree on the data and the respective scenario.

Observation 3: (Real Data) Training and testing with real data can be replicated only in a single-AoA system (using the wireless cable method with the UE complex antenna pattern), since a multi-AoA system with fixed probe layout cannot mimic the variety of the real data and scenarios.

Observation 4: (Real Data) Training and testing with real data requires a standardized data format and interface to the test system, as well as a description how this data is deployed by the test system.

Proposal 1: (Real Data) Support of real data training and testing is recommended only in the form of 1AoA testing and for R&D purposes, as a complement to the conformance requirement defined with synthetic channels.

Observation 5: (Single/Multi-AoA) Several aspects related to the UE Rx Antenna assumptions, the input to the AI/ML model and the real data usage for training and testing, have a major impact on the discussion about single- or multi-AoA testing.

Proposal 2: (Single/Multi-AoA) RAN4 to discuss and conclude on the aspects in Table 1 as part of the single-/multi-AoA discussion.

Table 1 – Single vs. multi AoA testing for AI/ML BM with CDL based channel

| Parameter | Single AoA | Multi AoA | Notes |
| --- | --- | --- | --- |
| CDL Channel Support | Only if UE Rx antenna performance is emulated as part of the channel (WCM*) | Direct, clusters are emulated on different probes / AoAs | * Wireless Cable Method |
| CDL Channel Complexity | Complex Channel (with WCM) | Simplified Channel |  |
| UE Rx Antenna Pattern Knowledge | Required for WCM (reference / declared / measured) | Not required |  |
| UE Rx Antenna Performance | Static pattern during testing simulating the dynamic pattern as part of the WCM | Dynamic pattern |  |
| UE Rx Beam Sweeping Timing | Not considered | Considered |  |
| UE Rx Beam Index as input for AI/ML Model | Supported with test mode* | Supported | * Depends on the complexity of the UE Rx Beam/Antenna pattern (Codebook) considered for single AoA testing |
| Training / testing with real data | Supported with WCM | Unfeasible (fixed probe layout) |  |
| TE Complexity | Low HW Complexity Longer test time WCM & Test mode required | High HW Complexity Shorter test time |  |

Proposal 3: (Multi-AoA Candidate Setup) Define the implementation of eIFF test setup with 4x active 2x2 MIMO probes with probes (CATRs) placed in a single plane and separated by 30º, 60º and 60º respectively, as feIFF (further enhanced IFF) test setup. The concrete probe layout and the alignment with the coordinate system to be used for testing is FFS.

Figure 1 – Example of a commercial implementation of eIFF system with 4 IFF probes / CATRs

Observation 6: (Multi-AoA Candidate Setup) The feIFF test setup supports 4 Active 2x2 MIMO probes with large distinctive angular separation of up to 150°, overall big quiet zone of at least 30 cm, lowest MUs and ensures preservation of (RF-RRM) side conditions.

Observation 7: (Multi-AoA Candidate Setup) The feIFF test setup is a feasible upgrade (superset) of the existing legacy eIFF setup. As such, if feIFF is concluded as capable for AI/ML BM testing, a single system covers the the whole conformance testing, ensuring continuity in the cost sensitive FR2 testing ecosystem.

Proposal 4: (Multi-AoA Candidate Setup) Consider the feIFF test setup as candidate for the AI/ML BM baseline system and analyze its suitability.

Observation 8: (Multi-AoA Simplified Channel) The required total AoA spread of 150° of the simplified channel in Table 2 can be emulated with the feIFF probe layout.

Observation 9: (Multi-AoA Simplified Channel) The simplified channel model parameters for UMi CDL-C can be implemented in the candidate test system feIFF, with 3 possible options (Figure 3): Option A: with flattened elevation (Table 3) and strongest cluster aligned to one probe; Option B:  with flattened elevation (Table 3) and mid-point of the total angular spread of the clusters is aligned to the mid-point of the spread of the probes; Option C: with flattened elevation, AoAs aligned to probe layout and no intra-cluster angle spread (Table 4).

Proposal 5: (Multi-AoA Simplified Channel) Consider the channel models described Table 3 (Option A/B) and Table 4 (Option C) as candidates for CDL-based simplified channel models for multi-AoA testing of AI/ML BM.

Table 3 – Channel model parameters for UMi CDL-C at 28 GHz with flat ZoA

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -134.4434 | 98.9242 | 74.51134 |
| 2 | 12.594 | -1.2500 | -20.9269 | 129.1633 | 99.1915 | 74.51134 |
| 5 | 13.056 | -5.5318 | -28.0782 | -152.8206 | 99.5732 | 74.51134 |
| 6 | 38.196 | 0.0000 | -11.6982 | 164.1145 | 99.306 | 74.51134 |
| 13 | 73.71 | -8.1318 | -33.911 | 93.1719 | 100.165 | 74.51134 |
| 14 | 78.498 | -9.8318 | -37.5066 | -112.0441 | 100.2604 | 74.51134 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 10.4021 | 0.5726 | 0 | 7 |  |

Table 4 – Channel model parameters for UMi CDL-C at 28 GHz
 with AoA aligned to probe layout and no intra-cluster angle spread

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -114.436 | 98.9242 | 74.51134 |
| 2 | 12.594 | -1.2500 | -20.9269 | 125.5639 | 99.1915 | 74.51134 |
| 5 | 13.056 | -5.5318 | -28.0782 | -174.436 | 99.5732 | 74.51134 |
| 6 | 38.196 | 0.0000 | -11.6982 | -174.436 | 99.306 | 74.51134 |
| 13 | 73.71 | -8.1318 | -33.911 | 95.5639 | 100.165 | 74.51134 |
| 14 | 78.498 | -9.8318 | -37.5066 | -114.436 | 100.2604 | 74.51134 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 0 | 0.5726 | 0 | 7 |  |

Figure 3 – Simplified UMi CDL-C at 28 GHz alignment to the candidate test system feIFF

Observation 10: (Multi-AoA Simplified Channel) The evaluation of Options A and B (Table 3) becomes more complex, if the effect of the test setup / probe layout has to be included.

Observation 11: (Multi-AoA Simplified Channel) The emulation of the relative ray phases with the feIFF is highly accurate and should satisfy the requirement for the accuracy for the cluster power fluctuations over time.

Proposal 6: (Multi-AoA Simplified Channel) Further discuss the metric(s) to evaluate channel model implementation with the feIFF candidate test setup.

Observation 12: (Multi-AoA Simplified Channel) For Option C (Table 4), there is no need to optimize the probe weights, and channel model parameters in Table 4 contain all the required information. This option is thus preferred from a TE point of view.

Observation 14: (Multi-AoA Channel Validation) Based on preliminary simulations with 8 Set B & 32 Set A beams, there is only a negligible effect (deviation of max ~1%) on the prediction accuracy for Top 3 & Top 1 beams between the cases with full CDL channel model (Reference - 24 clusters) and simplified CDL channel model suitable for feIFF (Options A/B/C - 6 clusters).

Proposal 8: (Multi-AoA Channel Validation) Consider preliminarily the CDL-C simplified channel models for feIFF with 6 Clusters in Table 3 and Table 4 as suitable for AI/ML BM testing.

Proposal 9: (Multi-AoA Channel Validation) Start the validation of CDL-C simplified channel models for feIFF with 6 Clusters in Table 3 and Table 4 through AI/ML BM system level simulations.

Proposal 10: (Multi-AoA Channel Validation) For the scope of validation, compare the prediction accuracy between the full channel and the simplified channel.

Proposal 11: (Multi-AoA Channel Validation) For the scope of validation, compare the prediction accuracy, when the model is trained with the full channel before the test and with the simplified channel during the test.

# References

- 3GPP TR 38.843 V18.0.0, “Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”

- 3GPP TR 38.901 V18.0.0, “Study on channel model for frequencies from 0.5 to 100 GHz”

- 3GPP TR 38.827 V16.8.0, “Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)”

- 3GPP TS 38.151 V18.3.0, “NR; User Equipment (UE) Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements”

- 3GPP TS 38.101-4 V18.6.0, “NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements”

- 3GPP TS 38.133 V18.8.0, “NR; Requirements for support of radio resource management”

- 3GPP TS 38.508-1 V18.5.0, “5GS; User Equipment (UE) conformance specification; Part 1: Common test environment”

- R4-2409739, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #111, May 2024

- R4-2411587, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #112, August 2024

- R4-2416391, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #112bis, October 2024

- R4-2418941, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #113, November 2024

- R4-2501726, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #114, February 2025

- R4-2504268, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #114bis, April 2025

- R4-2502856, “WF on the requirements for AI/ML air interface”, Qualcomm, 3GPP TSG RAN WG4 Meeting #114, February 2025

- R4-2500403, “Testability for AI/ML beam management”, Qualcomm, 3GPP TSG RAN WG4 Meeting #114, February 2025

- R4-2505105, “WF on requirements for AI/ML air interface normative work”, Qualcomm, 3GPP TSG RAN WG4 Meeting #114bis, April 2025

- R4-2505152, “Updated simulation assumptions for AI/ML BM”, Vivo, etc, 3GPP TSG RAN WG4 Meeting #114bis, April 2025