# 3GPP TSG-RAN WG4 Meeting #115                                                                         R4-2506990

# Malta, MT, 19-23 May, 2025

Agenda Item:	7.10.3.2.1

Source: 	CSCN

Title: 	Views on HPUE for NR-NTN

Document for:	Discussion

- Introduction

According to the  Rel-19 RAN4 NTN WID, one of major objectives is to study and specify HPUE (PC2, PC1.5, PC1) for NR-NTN/IoT-NTN with target device types as handheld UE and/or non handheld UE [1].

High power UE (HPUE) for NTN - Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-hand held UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives

In this contribution, some views on remaining open issues for NR-NTN HPUE are provided.

- Discussion

Power class and applicable device types

Supported device types for each PC depend on feasibility study and corresponding requirements. In the table below, we summarize the current progress.

Table 1 Applicable device types for different PC

| Power class | Applicable device types and number of antennas |
| --- | --- |
| PC2 | Handheld: 1Tx, 2Tx Non-handheld: 1Tx, 2Tx |
| PC1.5 | Non-handheld with 2Tx FFS for others pending on available time in Rel-19 |
| PC1 | Non-handheld only: 1Tx, [4Tx] |

For NTN operation, uplink coverage is a major bottleneck. In addition to non-handheld device, we are interested in supporting PC1.5 and PC1 for handheld device as well. For PC1.5, we understand there are some limitations on RF front-end design (e.g. PA and Duplexer). On existing TN TDD bands, PC1.5 was already supported for handheld devices with 2Tx (2 PC2 PA).  One possible way to support PC 1.5 for handheld device on NR NTN FDD bands is to reuse PC2 PA with HD-FDD mode which is also supported on TDD NR bands.

In the last RAN4 meeting, several companies also proposed to study power boosting schemes to achieve high power, e.g.  requirements relaxation and RB allocation restriction [2] [3]. If supporting PC1.5 for handheld devices with FDD full-duplex is relatively difficult due to the limitations on RF front-end design at this stage, it is recommended to perform power boosting operations on some sub-carriers to achieve equivalent high power, thereby improving the uplink performance.

PC1.5

Previous agreements: Issue 2-3: NR-NTN PC1.5 Agreement: - Define the requirements for PC1.5 for non-handheld NR-NTN UE with 2Tx in Rel-19 - FFS on other cases if time is allowed

Issue 1-1-2: Power boosting option for PC2 Agreement: - FFS on the power boosting option for PC2, including - Whether or not and how to define power boosting for PC2 as proposed above. - The power boosting discussion not to impact the requirements discussion for PC2 without power boosting, e.g. MPR/AMPR, for PC2.

Proposal 1: Support 1Tx/2Tx PC1.5 for handheld devices with HD-FDD mode.

Observation 1: It’s worth to further study power boosting schemes for both PC2 and PC1.5.

ACLR for NR-NTN

In previous RAN4 meetings, following agreements for ACLR values were reached based on collected co-existence results [4] [5].

RAN4#114bis Agreements: Issue 1-7-1: PC2 and PC1.5 ACLR Agreement: - Re-use existing TN ACLR: - For 2Tx PC1.5: Reuse TN PC1.5 ACLR - For 1Tx and 2Tx PC2: Reuse TN PC2 ACLR Issue 1-7-2: PC1 ACLR Tentative agreement: - PC1 ACLR is [34] dBc - Companies to further evaluate this, and to be confirmed in next meeting.

One remaining open issue is to determine the PC1 ACLR value. On existing TN PC1 (non-handheld device), 37dBc was specified. From the perspective of ecosystem, the same RF components on non-handheld devices in TN can be reused in NTN. We didn’t see any difficulty to support 37dBc for PC1 non-handheld device at least for 1Tx case.  From collected results of co-existence studies, it’s observed that by assuming no isolation distance between TN and NTN network, required ACIR is much larger than that of the case with 1.5km isolation distance. With this observation, we believe 37dBc ACLR can be helpful to reduce the isolation distance for realistic deployment of NTN network.

Table 2 Required ACIR from co-existence results for PC1 NR-NTN

| NR-NTN PC1 scenario 4 | Samsung | QC | MTK | Xiaomi | Ericsson | Huawei |
| --- | --- | --- | --- | --- | --- | --- |
| Isolation distance 0 km | 31dBc [R4-2501807] | 20dBc [R4-2501731] | 37.6dBc [R4-2501346] | 37dBc [R4-2500638] | >37dBc [R4-2501400] | 27dBc [ACLR,R4-2501457] |

In the future release(s), RAN4 could further discuss the possibility of supporting PC1 with handheld devices, then different ACLR values can be introduced later.

Proposal 2: Reusing TN ACLR (37dBc) for PC1 non-handheld device with 1Tx.

Proposal 3: It’s not precluded that separate ACLR for PC1 handheld device can be introduced later if feasible.

- Conclusion

In this contribution, views on remaining open issues for NR-NTN HPUE are provided.

Proposal 1: Support 1Tx/2Tx PC1.5 for handheld devices with HD-FDD mode.

Observation 1: It’s worth to further study power boosting schemes for both PC2 and PC1.5.

Proposal 2: Reusing TN ACLR (37dBc) for PC1 non-handheld device with 1Tx.

Proposal 3: It’s not precluded that separate ACLR for PC1 handheld device can be introduced later if feasible.

Reference

[1] RP-242901, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”

[2] R4-2504681, “Topic summary for [114] [128] NR_IoT_NTN_HPUE_part1”, Moderator (Samsung)

[3] R4-2504682, “Topic summary for [114] [129] NR_IoT_NTN_HPUE_part2”, Moderator (vivo)

[4] R4-2505113,	 “WF on HPUE RF requirements for NR-NTN”, Samsung

[5] R4-2505114,	“WF on HPUE RF requirements for IoT-NTN”, vivo