3GPP TSG-RAN WG4 Meeting #115	R4-2507004

Malta, Malta, 19th – 23th May, 2025

Agenda item:	7.5.3

Source:            	Google

Title:                	Discussion on methods for reducing the number of UE Rx chains for fragmented carriers

Document for:	Approval

# Introduction

In RAN#106, the revised SID for “Study on NR FR1 DL Fragmented Carriers” was approved as one of Release 19 work items [1]. The objective in the SID is to identify methods for reducing the number of UE Rx chains for two CCs DL NCCA in the single band with frequency span smaller than 100MHz, which is shown as follows. In this contribution, we will focus on the non-RF requirement related yellow part for the fragment carriers study.

### 4.1 Objective of SI or Core part WI or Testing part WI The objective of this study is as follows: - Identify methods for reducing the number of UE Rx RF chains (e.g. from separate Rx RF chains per CC to one Rx RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering: - Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4]; - Other requirements for study purpose are not precluded, if identified - The ability to semi-statically switch hardware resources (i.e. Rx RF chains) [RAN4, RAN2 – See note 2]; - Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4]; - Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4] ; - Impacts on DL performance [RAN4] ; - Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2]. NOTE 1: No RAN1 impact is foreseen NOTE 2: RAN2 work, if necessary, will be triggered by RAN4 LS NOTE 3: This study starts from single DL band. One Rx RF chain is not considered among inter-band DL carriers. - Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

In RAN4#114bis, the methods for reducing the number of UE Rx chains for fragmented carriers have been discussed and some open issues are captured in the approved WF [2]. In the WF, the UE capability issue is still open which still needs further discussion. For semi-statically switch hardware resources, it is still not concluded yet. In this paper, we would like to share our view for these issues in the following.

# Discussion

For fragmented carrier study item, the objective is to study whether the separated Rx RF chain for each CC can be replaced with fully shared Rx RF chain for two CCs in a single DL band under the frequency span smaller than 100MHz. In the previous meeting discussion, the meaning of the shared Rx RF chain is not very clear. Probably, it can be referred to partially-shared Rx RF chain, i.e., with the same BPF/external-LNA and separated internal-LNA/mixer/LO/analog-BB LPF, or fully-shared Rx RF chain, i.e., with the same BPF/LNA/mixer/LO/analog-BB LPF. After several meeting discussion, the fully shared Rx RF is identified as the reference architecture for fragmented carriers and the terminology is replaced with “one Rx RF chain” in the approved revised WID in RAN#106, where one Rx RF chain is shown as the following figure.

In our understanding, two CC NCCA is usually implemented by the shared antenna/BPF/external-LNA, different internal-LNA/mixer/LO/analog-BB LPF as the partially shared Rx RF chain. Supporting one Rx RF chain for fragmented carriers means that the UE can save more mixer/LO/analog-BB LPF for anther inter-band or intra-band carriers. In other words, the maximum supporting CC number for high order CA can be enhanced if the UE supports fragmented carrier feature. However, depending on the different RFIC implementation, the mixer/LO/analog-BB LPF allocation rule for a configured CA combination would be different. Hence, how many one Rx chain can be enabled in a configured CA combination is usually limited by the RFIC capability.

Signalling

One objective in the SID is to discuss the method for a UE to inform the network about which NCCA configuration it can support the fragmented carrier feature with new RF requirements, i.e., the RF requirement for one Rx RF chain which is different from separated Rx RF chains. In RAN4#114bis, several alternatives and one open issue for signalling related issues are captured in WF for further discussion. Alternative 1 and Alternative 1a are based on leveraging the legacy band combination scheme with Rx sharing capability indication including/not including other related information. Alternative 2 is preferred to use the new CA notations for one Rx RF chain and Alternative 2a would like to introduce the new bandwidth class conditioned on the frequency separation between two NCCA carriers. The other open issue is to discuss how to handle the case when the fragmented carrier feature can support larger than 2 NCCA for the future proof.

# Agreements during/after online discussion #### Issue 2-1: How to indicate UE capability for supporting ”one Rx RF chain” for DL fragmented carriers - Consider the following alternatives - Alternative 1: use the legacy band combination reporting scheme + Rx sharing capability indication - Alternative 1 has no modification of CA configurations in RAN4 - Alternative 1a: use the legacy band combination reporting scheme + Rx sharing capability and other related information including the frequency separation between two carriers. - Alternative 1a has no modification of CA configurations in RAN4 - Alternative 2: use the new notation for CA configurations with single Rx RF Chain - Alternative 2a: use the new bandwidth class conditioned on the frequency separation between two carriers # Open issues UE capability indication for supporting ”one Rx RF chain” for DL fragmented carriers For information: Companies are encouraged to provide pros and cons analysis that can further consider aspects below: - For futureproofness, how to clearly indicate applicable intra-band CCs for the band where there are more than two non-contiguous CCs RAN4 spec impact and work load

In our understanding, Rx sharing capability and/or other related information for fragmented carriers feature on one Rx RF chain can be reported by per-UE, per-band, per-BC, per-band per-BC and per CC per-band per-BC capability signalling. For per-UE or per-BC capability signalling, it would mean that the UE needs to support one Rx RF chain for all supported RF bands or for a configured band combination. Based on the above discussion that whether to support the fragmented carriers feature may depend on the different considerations, per-UE or per-BC capability signalling may cause less flexibility to UE RFIC implementation.

Take the following band combination CA_n1A-n3(2A)-n7(2A)-n8A-n78A in TS38.101-1 as an example. It is observed that there are two bands may support fragmented carriers feature in this NCCA band combination. Depending on the different optimization for the mixer/LO/analog-BB LPF allocation rule, there may be a possibility that the UE may use the one Rx RF chain for NCCA CA_n3(2A) but separated Rx RF chains for NCCA CA_n7(2A) in this configured CA band combination. Hence, in our view, it seems that the per-band or per-band per-BC capability signalling is more suitable since it can offer more UE implementation flexibility.

[38.101-1] [V19.0.0]

Table 5.5A.3.4-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (five bands)

| NR CA configuration | Uplink configuration or single uplink carrier 2 | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3(2A)-n7(2A)-n8A-n78A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n8A CA_n1A-n78A CA_n3A-n7A CA_n3A-n8A CA_n3A-n78A CA_n7A-n8A CA_n7A-n78A CA_n8A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3(2A)_BCS0 |  |
|  |  | n7 | CA_n7(2A)_BCS0 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |

In the other way, if per-band per-BC capability signalling is indicated, the UE can clearly indicate which NCCA can support fragmented carriers feature, i.e., CA_n3(2A) and/or CA_n7(2A) can support one Rx RF chain within the band combination CA_n1A-n3(2A)-n7(2A)-n8A-n78A. The advantage for per-band per-BC capability signalling is that the UE can have higher flexibility to decide whether the NCCA should be supported by fragmented carriers feature for the example band combination.

For per-band capability signalling, although it can save multiple signalling bits for the indicated band NCCA to support fragmented carrier feature, the per-band Rx sharing capability should be also applied to all high order CA combination which contains the indicated band NCCA. For example, if the per-band capability bit is indicated in NR band n3, the NCCA CA_n3(2A) is supported by fragmented carrier feature in both band combination CA_n3(2A)-n7(2A) and CA_n1A-n3(2A)-n7(2A)-n8A-n78A. Hence, it also may cause less flexibility to UE RFIC implementation.

For per CC per-band per-BC capability signalling, although it can be more flexible to indicate which two CCs can support fragmented carrier feature among higher order band combination, e.g., CA_n41(3A) or CA_n41(4A), the signaling overhead would be increased due to the possible reporting all CC permutation in NCCA supporting fragmented carrier feature, e.g., CC1/CC2 or CC1/CC3 or CC2/CC3 in CA_n41(3A).

In our view, in order to accommodate the future extension for fragmented carriers feature with NCCA number larger than 2, the per CC per-band per-BC capability signalling seems to be more flexible for the UE can support one Rx RF chain for all NCCA CCs permutation on that reported band. Although the new CA notation or new CA bandwidth is proposed as another solution, the implementation effort may be increased compared to the legacy band combination reporting with Rx sharing capability related information. In addition, more new band combination and new tables need to be introduced in the RAN4 specification.

Considering different implementation can have different limitation for frequency separation on one Rx RF chain for each NCCA, the frequency separation information may need to be associated with the Rx sharing capability indication. Therefore, for DL fragmented carriers supporting one Rx RF chain capability signalling, it is preferred to use legacy band combination reporting scheme with Rx sharing capability indication and frequency separation between two carriers by per CC per-band per-BC reporting as the baseline.

Proposal 1: For DL fragmented carriers supporting one Rx RF chain capability signalling, it is preferred to use legacy band combination reporting scheme with Rx sharing capability indication and frequency separation between two carriers by per CC per-band per-BC reporting as the baseline.

Semi-statically switch hardware resources

Another objective in the SID is to discuss the semi-statically switching hardware resources, i.e., the switching from one Rx RF chain (e.g., FC-mode) to separate Rx RF chains (e.g., non-FC-mode). The issue has been discussed in the previous meetings, but it is not concluded that how the UE/network determine whether the FC-mode is still workable or not when the higher interference is detected inside the gap between two non-contiguous CCs, i.e., the triggering condition to enable/disable one Rx RF chain. Hence, in the following discussion, the triggering condition is assumed to be known for the network and/or the UE.

In RAN4#114bis, several semi-statically switch hardware resources related issues are converged in two issues in WF for further discussion. One issue is to discuss CA fallback with/without UE architecture switching, i.e., the switching between one Rx RF chain and separated Rx RF chains, and the other issue is to focus on whether UE architecture switching is decided by the UE or by the network.

# Ad hoc meeting agreements #### Issue 2-2: Discussion on semi-statically switch hardware resources procedure and fallback considerations - Further discussions on - CA fallback with/without UE architecture switching - Consider the fall back - from high order CA to lower order CA - from CA to single carrier mode - Decided by network - UE architecture switching: - Switching between 2 Rx RF chains and 1 Rx RF chain on CCs of intra-band NC CA - FFS on whether it is decided by UE itself or decided by network - FFS on whether the additional indication from UE side is needed

In our understanding, the FC-mode will be triggered possibly when the network would like to configure the UE by much high order CC band combination. From UE perspective, if there are additional hardware resources which can be used to support the configured band combination including NCCA, whether to use one Rx RF chain or separated Rx RF chain can be up to the UE implementation. If there is no additional hardware resource, the UE can only use one Rx RF chain to support the configured band combination.

From network perspective, one possibility is that the network can rely on the legacy method to configure the reference signal e.g., NZP-CSI-RS or ZP-CSI-RS, for each CC in the NCCA configuration to get the channel status by RSSI/RSRP/RSRQ/SINR/CQI reporting from the UE side. Based on the reported information, the network can decide whether to perform any reconfiguration or fallback to other configuration for the UE. Hence, there it is possible that the network is not necessary to know the exact UE architecture status for NCCA, i.e., FC-mode or non-FC mode, during transmission.

If the higher interference inside the NCCA gap is detected when the UE is in FC-mode, it is straightforward that the network can release/deactivate the CC with worse channel status in NCCA to provide the chance for the UE going to separated Rx RF chains if there are additional hardware resources, or go to single carrier mode if there is no additional hardware resource. In one case, if there are additional hardware resources for separated Rx RF chains, one possibility is that the UE can monitor interference measurement inside the gap by itself and determine whether it should transfer to separated Rx RF chains or not. Hence, the higher interference may not be detected by the network if UE do architecture switching by itself. In another case, if there is no additional hardware resource for separated Rx RF chains, the UE can still keep one Rx RF chain for NCCA until the network release it or other hardware resources are available.

Take the one NCCA high order band combination CA_n1A-n3(2A)-n8A-n78A as an example. If the higher interference inside the NCCA n3(2A) gap are detected, the network can release one CC in CA_n3(2A) to reconfigure the UE operated in CA_n1A-n3A-n8A-n78A. In another case, the network has released n1A to reconfigure the UE operated in CA_n3(2A)-n8A-n78A. If the higher interference inside the NCCA n3(2A) gap are detected, whether the saved n1A hardware resource can be applied to n3(2A) operated in one Rx RF chain or separated Rx RF chain depending on the UE implementation.

In our view, based on the above observation, it seems that UE architecture switching can be transparent to the network when the higher interference inside the NCCA gap is detected. The network can still rely on the legacy method to handle the failure for the UE in one Rx RF chain on NCCA band combination. The network may not need to know whether the UE is currently in one Rx RF chain or separated Rx RF chain for a configured band combination. By following legacy RRC reconfiguration procedure, whether to perform any fallback reconfiguration for the UE can depends on the network decision. Hence, it is proposed that the UE architecture switching can be decided by UE itself, and it is preferred to follow the network decision on whether to perform any CA reconfiguration or fallback configuration for the UE if one Rx RF chain mode can be detected as failure.

Proposal 2: It is proposed that the UE architecture switching can be decided by UE itself, and it is preferred to follow the network decision on whether to perform any CA reconfiguration or fallback configuration for the UE if one Rx RF chain mode can be detected as failure.

# Conclusion

The proposals in this contribution are summarized in the following.

Proposal 1: For DL fragmented carriers supporting one Rx RF chain capability signalling, it is preferred to use legacy band combination reporting scheme with Rx sharing capability indication and frequency separation between two carriers by per CC per-band per-BC reporting as the baseline.

Proposal 2: It is proposed that the UE architecture switching can be decided by UE itself, and it is preferred to follow the network decision on whether to perform any CA reconfiguration or fallback configuration for the UE if one Rx RF chain mode can be detected as failure.

# Reference

- RP-243002, “Revised SI: Study on NR FR1 DL Fragmented Carriers”, MediaTek Inc., RAN#106

- R4-2505109, “WF on NR FR1 DL fragmented carriers study”, MediaTek Inc., RAN4#114bis

- R4-2504102, “Discussion on methods for reducing the number of UE Rx chains for fragmented carriers”, Google, RAN4#114bis