3GPP TSG-RAN WG4 Meeting #115	R4-2507012

St. Julian’s, Malta, 19 - 23 May 2025

Agenda Item:	7.24.4.2

Source: 	Ericsson

Title:	Discussion on CJTC reporting and asymmetric UL TRP requirements

Document for:	Discussion

# Introduction

In this contribution we analyze the potential RAN4 RRM impacts for the CJT calibration reporting.

# Discussion

## Scheduling restrictions

During the previous meeting, the following WF was agreed upon, and this section provides our perspective on the necessity of scheduling restrictions.

- Define core requirements for delay and frequency offset reporting for - Measurement period (FFS number of samples) - Scheduling restriction (if identified)

Assuming our understanding of the proponent’s intention is accurate, the rationale for scheduling restrictions originates from the following RAN1 agreement.

Furthermore, the defined range for the 1-bit indicator "dn" is [0, CP], applicable both internally and externally.

We first examine the MRTD conditions pertinent to the multi-TRP framework. The MRTD table from TS 38.133 is reproduced here for reference.

Table 7.6.8-1: Maximum receive timing difference requirement for multi-TRPs with two TAs for UE not capable of rxTimingDiff-r18 Frequency Range; Maximum receive timing difference. FR1; CP lengthnote 1. FR2-1; CP lengthnote 1. NOTE 1: CP length of the maximum SCS on the carrier.. Table 7.6.8-2: Maximum receive timing difference requirement for multi-TRPs with two TAs for UE capable of rxTimingDiff-r18 Frequency Range of the pair of carriers; Maximum receive timing difference (µs). FR1; 33. FR2-1; 8.

In our understanding, all the specified requirements for the multi-TRP framework are applicable when the MRTD condition is fulfilled. Values of MRTD greater than CP are supported by a separate UE capability, namely rxTimingDiff-r18; however, its applicability to the CJTC reporting feature remains uncertain. Our intent is to specify the requirements under existing MRTD assumptions, thereby avoiding the introduction of additional scheduling restrictions.

We also believe that the cost associated with scheduling restrictions is substantial. As illustrated in the figure 1 [copied from Nokia email discussion], each TRS resource set configured for CJTC comprises 4 symbols. Implementing a scheduling restriction of one additional symbol per TRS symbol in a TRS set would lead to an substantial overhead, effectively resulting in the loss of 4 symbols across two slots. It is important to highlight that an entire OFDM symbol is lost rather than just the RE adjacent to the TRS. We are uncertain if this aligns with the original intention of RAN1 when this field was introduced as none one the scheduling restrictions are discussed in RAN1. We should note that RAN1 also defines dropping rules or scheduling restrictions.

- CJTC measurement requirements are applicable when the RTD is less than CP.

Figure 1: TRS resource set for CJTC reporting

## CJTC reporting delay

In RAN4#112bis meeting, RAN4 agreed to specify the core requirements based on the outcome of the simulation study. Agreement is shown below.

•	RAN4 to simulate the performance of offsets for phase, time delay and frequency, based on one-shot measurement and multiple-sample measurement, and discuss the impact on RRM core requirement later.

•	Further discuss the simulation assumptions offline during this meeting.

Way forward:

•	Companies are encouraged to bring simulation assumption/results in the next meeting. Simulation assumption in Appendix A can be used as a starting point for information only.

In RAN4#113 and RAN4#114 meeting, following simulation assumptions [2], [3] were agreed. It was further agreed that the number of samples should be determined based on their impact on the reporting interval, taking into account the granularity required for the given report resolution.

| Parameter | Value |
| --- | --- |
| Waveform | OFDM |
| Duplex, SCS | - FDD, 15kHz SCS - TDD, 30kHz SCS |
| Carrier Frequency | 3.5 GHz |
| Channel Model | TDLC300-10 |
| Delay Spread | 300ns |
| antenna configuration | 1X2 |
| TRP number | 2 |
| TRS Bandwidth | 48RB |
| Numerology | 14 OFDM symbol per slot |
| UE speed | 3km/h |
| SNR | -3dB, 0dB, 3dB |
| Frequency offset between TRP (only for freq. offset simulations) | 0.1ppm, 0.2ppm 2nd priority: 0.02 ppm, 0.05ppm |
| Delay difference between TRP (only for delay offset simulations) | 0.5CP, CP |
| AD | CP |
| MD | 128 |
| AFO | 0.2ppm |
| MFO | 256 |
| Number of samples (TRS resource sets) for measurement | 1, 3, 5 2nd priority: 2, 4, 10 |
| CSI-RS config | Cjtc-d and cjtc-f: periodic TRS (‘CSI-RS for tracking’) |
| CSI-RS periodicity (TRS) | 20ms / 40 slots for 30kHz; 20 slots for 15kHz |
| CSI-RS offset | Slot 10 and 11 for 15kHz, slot 20 and 21 for 30KHz |

In the following sections we present the simulation results based on the above simulation assumptions for the CDF accuracy at 5%, 50% and 95% and make few observations and proposals concerning the core requirements definition.

### Simulation results for frequency offset

| Number of TRS resource sets in a measurement | 30kHz SCS, A_FO=200 ppb; FO error at 100 ppb (absolute) estimated over 500 UEs |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR = -3 dB |  |  |  | SNR= +3dB |  |  |  |  |
|  | 50%ile | 5%ile in # levels (256) | 50%ile in # levels (256) | 95%ile in # levels (256) | 50%ile | 5%ile in # levels (256) | 50%ile in # levels (256) | 95%ile in # levels (256) |
| 1 TRS | 5.854 | 1 | 8 | 26 | 2.849 | 1 | 4 | 12 |
| 2 TRS | 4.853 | 1 | 7 | 18 | 2.165 | 1 | 3 | 8 |
| 3 TRS | 3.794 | 1 | 5 | 16 | 1.916 | 1 | 3 | 7 |
| 5 TRS | 3.106 | 1 | 4 | 12 | 1.433 | 1 | 2 | 6 |
| 10 TRS | 2.234 | 1 | 3 | 9 | 1.030 | 1 | 2 | 4 |

| Number of TRS resource sets in a measurement | 15kHz SCS, A_FO=200 ppb, FO error at 100ppb (absolute) estimated over 500 UEs |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR = -3 dB |  |  |  | SNR= +3dB |  |  |  |  |
|  | 50%ile | 5%ile in # levels (256) | 50%ile in # levels (256) | 95%ile in # levels (256) | 50%ile | 5%ile in # levels (256) | 50%ile in # levels (256) | 95%ile in # levels (256) |
| 1 TRS | 3.265 | 1 | 5 | 14 | 1.553 | 1 | 2 | 7 |
| 2 TRS | 2.396 | 1 | 4 | 9 | 1.107 | 1 | 2 | 5 |
| 3 TRS | 1.905 | 1 | 3 | 8 | 0.991 | 1 | 2 | 4 |
| 5 TRS | 1.547 | 1 | 2 | 6 | 0.794 | 1 | 2 | 3 |
| 10 TRS | 2.234 | 1 | 3 | 9 | 1.030 | 1 | 2 | 4 |

We also present our simulation results in the time and frequency (e.g., ns and Hz) in the section 5 of this paper.

As we can see from the above simulation results, for frequency offset, for SNR of -3dB and 3dB, frequency offset averaged over multiple samples (e.g., averaged over 3 samples or 5 samples) gives better accuracy. Hence, we suggest defining the CJT requirements with multi-sample measurement.

- RAN4 to define the delay requirements of the CJTC frequency offset with 5 sample averaging.

### Simulation results for delay offset

| Number of TRS resource sets in a measurement | 30kHz SCS, A_DO=CP, DO error at 0.5CP (absolute) estimated over 500 UEs |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR = -3 dB |  |  |  | SNR= +3dB |  |  |  |  |
|  | 50%ile | 5%ile in # levels (128) | 50%ile in # levels (128) | 95%ile in # levels (128) | 50%ile | 5%ile in # levels (128) | 50%ile in # levels (128) | 95%ile in # levels (128) |
| 1 TRS | 0.012 | 1 | 4 | 10 | 0.012 | 1 | 3 | 9 |
| 2 TRS | 0.010 | 1 | 3 | 8 | 0.010 | 1 | 3 | 8 |
| 3 TRS | 0.009 | 1 | 3 | 7 | 0.008 | 1 | 3 | 7 |
| 5 TRS | 0.007 | 1 | 2 | 6 | 0.007 | 1 | 2 | 6 |
| 10 TRS | 0.005 | 1 | 2 | 4 | 0.005 | 1 | 2 | 4 |

| Number of TRS resource sets in a measurement | 15kHz SCS, A_DO=CP, DO error at 0.5CP (absolute) estimated over 500 UEs |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR = -3 dB |  |  |  | SNR= +3dB |  |  |  |  |
|  | 50%ile | 5%ile in # levels (128) | 50%ile in # levels (128) | 95%ile in # levels (128) | 50%ile | 5%ile in # levels (128) | 50%ile in # levels (128) | 95%ile in # levels (128) |
| 1 TRS | 0.009 | 1 | 3 | 7 | 0.008 | 1 | 2 | 6 |
| 2 TRS | 0.007 | 1 | 2 | 6 | 0.006 | 1 | 2 | 5 |
| 3 TRS | 0.006 | 1 | 2 | 6 | 0.005 | 1 | 2 | 5 |
| 5 TRS | 0.006 | 1 | 2 | 4 | 0.004 | 1 | 2 | 4 |
| 10 TRS | 0.004 | 1 | 2 | 3 | 0.003 | 1 | 1 | 3 |

As we can see from the above simulation results, for delay offset, for SNR of -3dB and 3dB, delay offset averaged over multiple samples (e.g., averaged over 3 samples or 5 samples) gives better accuracy. Hence, we suggest defining the delay offset reporting requirements with multi-sample measurement. Further from the simulations results, 5 sample averaging provide significant improvement over the 3-sample averaging and single sample measurement.

- RAN4 to define the delay requirements of the CJTC delay offset with 5 sample averaging

### Simulation assumptions and results for Phase offset

Table below summarizes the simulation parameters for reciprocity-based CJTC phase offset reporting:

| Parameter | Value |
| --- | --- |
| Waveform | OFDM |
| Carrier Frequency | 3.5GHz TDD |
| SCS | 30kHz SCS |
| Channel Model | TDLC300-10 |
| TRP number | 2 |
| TRP antenna configuration | 1x2 |
| UE antenna configuration | 1x2 |
| UE speed | 3km/h |
| CSI-RS Bandwidth | 48 RB |
| CSI-RS configuration | cjtc-p, periodic CSI-RS using 1 or 2 ports. CSI-RS is precoded by phase of channel estimate from latest SRS with a 1-1 map between SRS and CSI-RS port number |
| CSI-RS periodicity | 20 slots, with slot offset 2 and 3 |
| SNR | -3dB, 3dB |
| Initial UL/DL phase offset between TRPs | 0 degree, 60 degree, 90 degree |
| SRS configuration | Periodic SRS every 10 slots, 48 RB bandwidth |

Impact of number of reported bits on wideband UL/DL phase offset estimation error (50 and 95-percentile) is shown in the tables below:

| Initial phase offset (deg), number of CSI-RS ports | 180 UEs, 20 CSI reports per UE (400 slots); 50%ile of error (absolute) in deg over all reports |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR = -3 dB |  |  |  | SNR= +3dB |  |  |  |  |
|  | 50%ile error | 50%ile in # levels (16) | 50%ile in # levels (32) | 50%ile in # levels (64) | 50%ile error | 50%ile in # levels (16) | 50%ile in # levels (32) | 50%ile in # levels (64) |
| 0 deg, 1 port | 7.595 | 1 | 1 | 2 | 5.255 | 1 | 1 | 1 |  |
| 60 deg, 1 port | 7.674 | 1 | 1 | 2 | 5.422 | 1 | 1 | 1 |  |
| 90 deg, 2 ports | 5.026 | 1 | 1 | 1 | 3.483 | 1 | 1 | 1 |  |

| Initial phase offset (deg), number of CSI-RS ports | 180 UEs, 20 CSI reports per UE (400 slots); 95%ile of error (absolute) in deg over all reports |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR = -3 dB |  |  |  | SNR= +3dB |  |  |  |  |
|  | 95%ile error | 95%ile in # levels (16) | 95%ile in # levels (32) | 95%ile in # levels (64) | 95%ile error | 95%ile in # levels (16) | 95%ile in # levels (32) | 95%ile in # levels (64) |
| 0 deg, 1 port | 23.001 | 2 | 3 | 5 | 14.757 | 1 | 2 | 3 |  |
| 60 deg, 1 port | 22.372 | 1 | 2 | 4 | 14.977 | 1 | 2 | 3 |  |
| 90 deg, 2 ports | 15.261 | 1 | 2 | 3 | 10.229 | 1 | 1 | 2 |  |

As we can see from the above simulation results, the error measured at 50% point was 5 degrees at SNR of 3dB. If we use the 16 levels in reporting, UE will report it as 22.5 degrees. That means the quantization introduce additional 17.5 degrees error. This error makes the report almost useless for the CJT. As we analyzed in our previous contribution to RAN4, R4-2416335, phase offset has to be within 30 degrees and 10 degrees for some of the applications to work in the field. Even when UE can estimate the PO as low as 5 degrees, this kind of quantization error makes the report to be useless for CJTC. Hence, we suggest RAN4 sending LS to RAN1 to increase the number of levels to avoid the quantization error impact the quality of the CJT phase offset report.

- RAN4 to send LS to RAN1 to increase the number of levels for the phase offset reporting.

Asymmetric DL sTRP/UL mTRP

Moderator’s note on the scenarios’ definition discussed in this section:

- Scenario#1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s)

- Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE may expect to receive SSB from UL TRP(s).

When UE is configured with PL offset (Scenario 1)

For scenario 1, following agreements are reached.

- RAN4 to define timing requirements for FR1 as well as FR2

- Discuss whether gradual timing adjustment applies to both TAGs or not.

- Discuss whether gradual timing adjustment applies to DL/UL TRP or also to UL TRP.

- Discuss whether RACH transmission requirements need to be updated.

- No RRM impacts on TCI state switching requirement.

UE UL transmit timing requirements:

Transmit timing requirements can follow the legacy principle. That means, for each TAG, the uplink transmission timing takes place (N_"TA" +N_"TA offset" )×T_c before the reception of the downlink reference point. N_TA offset can be used as per the RAN1 definition.

- For each TAG, the uplink transmission timing takes place (N_"TA" +N_"TA offset" )×T_c before the downlink reference point of the respective TAG.

Gradual timing adjustment requirements:

We think Rel-18 principles can be reused for the requirements definition. We copy the Rel-18 requirement below for reference.

For multi-DCI based multi-TRP operation with two TAs, gradual timing adjustment specified in this clause applies for each TAG and shall be met for each TAG. The reference point for each TAG for the PUCCH/PUSCH/SRS and PRACH transmission triggered by PDCCH order is defined in clause 7.1.1.

Since the single DL reference timing is used, the gradual adjustment UE should apply for each TAG can be same.

- For asymmetric DL sTRP/UL mTRP with two TAs, gradual timing adjustment applies for each TAG and shall be met for each TAG. The amount of gradual timing adjustment is same for both TAG.

Timing requirements for PRACH:

Regarding how to update the RRM requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP, following agreement is reached in previous meeting.

For scenario 1, for both TAGs, the uplink transmission(s) timing for PUCCH/PUSCH/SRS take place (NTA + NTA offset) x Tc before the reception of the first detected path in time of one of the corresponding downlink reference signals(s) of the reference cell associated with DL or Joint TCI state.

  - The above agreement can be revisited if not aligned with RAN1 further agreement.

We note that the agreement above pertains to PUCCH, PUSCH, and SRS. We believe it is crucial to discuss the downlink reference timing for PRACH, especially as PRACH is supported for uplink-only TRP.

- For the scenario that the UE is configured with PL offset in joint/UL TCI state(s), when the UE is configured with rel-19 2 TAs, the UE maintains one single downlink reference timing - For the scenario that the UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), when the UE is configured with rel-19 2 TAs: - The UE maintains two downlink reference timings; - Baseline assumption for this feature is that Rx timing difference between two DL reference timings is no larger than one CP length, while it is subject to optional UE capability that the Rx timing difference between two DL reference timings can be assumed to be larger than CP length. - The reference point for PRACH transmission is indicated as follows: - if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG. - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG Above applies for the case UE is configured with SSB-MTC-additionalPCI

To enable two TAs with respect to two TRPs, UE need to obtain DL reference timing and UL TA command towards UL-only TRP. Next, we further discuss the aspects of DL reference timing and UL TA towards the UL-only TRP. The two-TA feature in Rel-18 assumes non-ideal synchronization (i.e., TRPs slot boundaries are not perfectly time aligned and there can be synchronization error of up to e.g., 3µs) between the TRPs. As per the WID, asymmetric and non-collocated deployment is supported for UL-only TRP. That means the synchronization assumptions in Rel-18 are applied for Rel-19 UL-only TRP also as operators may not deploy separate TRPs for using the UL-only TRP feature (e.g., operators may use same deployment for Rel-18 and Rel-19).

As UL-only TRP do not transmit any SSB, UE may have to rely on Anchor TRP SSB to derive the DL reference timing. When slot boundaries of Anchor TRP and UL-only TRP are perfectly aligned in time (i.e., synchronization error is 0µs), UE may use the DL reference timing of Anchor TRP to perform random access procedure and obtain TA command to handle the asymmetric propagation delay difference between Anchor TRP to UE and UL-only TRP to UE.

However, in the practical deployments (such asymmetric deployments considered in this WI) it may be sometimes impossible to maintain perfect synchronization between different TRPs even for intra-DU deployments, since most of the NW deployments use switched fronthaul technic between non-collocated TRPs. Figure 1 illustrates the synchronization scenario where the timing of UL-only TRP is ahead of Anchor TRP. When UL-only TRP is ahead of Anchor TRP slot timing, when UE acquires DL timing from Anchor TRP and transmits PUSCH using the same DL reference timing, it may result in inter-slot interference at UL-only TRP.

Figure 2: Synchronization scenario UL-only TRP ahead of Anchor TRP slot timing

As per TS 38.133, initial transmit timing for RACH transmission, UE needs to acquire frame boundary within ±Te error (Te is much smaller compared to possible synchronization error between TRP), and transmits RACH after acquiring DL synchronization, RACH preamble is always expected to receive after the start of the RACH slot at the NW.

To analyze the issue with practical synchronization error for UL-only TRP, we consider three possible synchronization scenarios as shown in Figure 2. When the UL slot boundaries of anchor TRP and UL- only TRP are not perfectly synchronized (scenario 2 and 3) as shown in Figure 2, using DL timing of anchor TRP to transmit RACH preamble to UL TRP may result in early arrival of RACH preamble or later arrival of the RACH preamble at the UL- only TRP.

Figure 3: Different synchronization scenarios at UL- only TRP and anchor TRP

When UL- only TRP is behind anchor TRP slot timing, when UE acquires DL timing from anchor TRP and transmit RACH using the same DL reference timing, it may result in early arrival of RACH preamble at UL- only TRP (i.e., before the start of RACH occasion or RACH slot at the reception window at the UL TRP as shown in Figure 3). This results in inter-slot interference at the UL- only TRP and UL TRP may not be able to determine the timing advance correctly for the UE.

Figure 4: RACH arrival at UL- only TRP and Anchor TRP with anchor TRP DL reference timing

When UL- only TRP is ahead of anchor TRP slot timing, when UE acquires DL timing from anchor TRP and transmit RACH using the same DL reference timing, it may result in late arrival of RACH preamble at UL- only TRP. Based on the below table showing guard period (GP) for different PRACH preamble formats, we think late arrival may not be an issue for the UL- only TRP deployment scenario where the UE is expected to be close to UL- only TRP and GP is large compared to synchronization error.

Table 1: Guard period for different PRACH preamble formats

| Preamble format | 0 | 1 | 2 | 3 | B1 | B2 | B3 | B4 | C0 | C2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GP in µs | 97 | 716 | 953 | 97 | 2.3 | 7 | 11.7 | 25.8 | 37 | 94 |
| Propagation delay (in km) that GP can cover | 29km | 215km | 285km | 29km | 0.7700m | 2.1km | 3.7km | 7.5km | 11km | 28km |

Based on the above discussion, we have the following proposal to consider practical synchronization error between Anchor TRP and UL-only TRP for UL and DL timing control in an asymmetric deployment, which for example could be handled by introducing a DL timing offset between the Anchor TRP for the RACH transmission towards the UL-only node. The DL reference timing offset can be a fixed value, which may be of the order of TAE or 0.5 TAE. The value can be further discussed.

- UE to apply additional DL reference timing offset for the PRACH transmission. The DL reference timing offset can be [0.5*TAE].

When UE is not configured with PL offset and SSB is available (Scenario 2)

For Scenario 2, RAN1 replied with following LS response [R1-2503091].

Answer: The RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs” are NOT applied for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s).

And RAN1 made the following new agreement for further clarification:

Agreement: - For the scenario that the UE is configured with PL offset in joint/UL TCI state(s), when the UE is configured with rel-19 2 TAs, the UE maintains one single downlink reference timing - For the scenario that the UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), when the UE is configured with rel-19 2 TAs: - The UE maintains two downlink reference timings; - Baseline assumption for this feature is that Rx timing difference between two DL reference timings is no larger than one CP length, while it is subject to optional UE capability that the Rx timing difference between two DL reference timings can be assumed to be larger than CP length. - The reference point for PRACH transmission is indicated as follows: - if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG. - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG - Above applies for the case UE is configured with SSB-MTC-additionalPCI

We consider the RAN1 agreement to be consistent with the Release 18 multi-DCI-based multi-TRP operation involving two Timing Advances (TAs). Based on the RAN1 reply LS, we would like to highlight that Scenario 2 is not supported for intra-cell multi-TRP operations. For inter-cell multi-TRP configurations, we propose that the Release 18 requirements can be effectively reused. That means following requirement can be applied.

For multi-DCI based multi-TRP operation with two TAs, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState TS 38.331 [2] having the same TAG as the uplink signal. For inter-cell multi-DCI based multi-TRP operation with two TAs, the reference point for PRACH transmission triggered by PDCCH order, - if “PRACH association indicator” is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG. - if “PRACH association indicator” is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG.

- For scenario 2 and inter-cell multi-TRP, the reference point for PUCCH/PUSCH/SRS/PRACH defined for multi-DCI based multi-TRP operation with two TAs can be reused.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- CJTC measurement requirements are applicable when the RTD is less than CP.

CJTC reporting

- RAN4 to define the delay requirements of the CJTC frequency offset with 5 sample averaging.

- RAN4 to define the delay requirements of the CJTC delay offset with 5 sample averaging.

- RAN4 to send LS to RAN1 to increase the number of levels for the CJTC phase offset reporting.

Asymmetric DL sTRP/UL mTRP:

- For each TAG, the uplink transmission timing takes place (N_"TA" +N_"TA offset" )×T_c before the downlink reference point of the respective TAG.

- For asymmetric DL sTRP/UL mTRP with two TAs, gradual timing adjustment applies for each TAG and shall be met for each TAG. The amount of gradual timing adjustment is same for both TAG.

- UE to apply additional DL reference timing offset for the RACH transmission. The DL reference timing offset can be [0.5*TAE].

- For scenario 2 and inter-cell multi-TRP, the reference point for PUCCH/PUSCH/SRS/PRACH defined for multi-DCI based multi-TRP operation with two TAs can be reused.

# References

[1] RP-234007, New WID: NR MIMO Phase 5
[2] R4-2502596 WF on RRM requirements for NR_MIMO_Ph5

[3] R4-2420209 Simulation assumptions for CJT calibration

# Simulation results in nano seconds and HZ

| Estimated FO 100.0, AFO=200 |  |  |  |
| --- | --- | --- | --- |
|  | case | mean-ae FO [Hz] | 50p-ae FO [Hz] |
| 19 | SNR+3dB 30kHz FO 100ppb 1TRS | 12.4 | 10.0 |
| 18 | SNR+3dB 30kHz FO 100ppb 2TRS | 9.0 | 7.6 |
| 17 | SNR+3dB 30kHz FO 100ppb 3TRS | 7.5 | 6.7 |
| 16 | SNR+3dB 30kHz FO 100ppb 5TRS | 5.9 | 5.0 |
| 15 | SNR+3dB 30kHz FO 100ppb 10TRS | 4.3 | 3.6 |
| 14 | SNR+3dB 15kHz FO 100ppb 1TRS | 6.8 | 5.4 |
| 13 | SNR+3dB 15kHz FO 100ppb 2TRS | 4.8 | 3.9 |
| 12 | SNR+3dB 15kHz FO 100ppb 3TRS | 4.0 | 3.5 |
| 11 | SNR+3dB 15kHz FO 100ppb 5TRS | 3.3 | 2.8 |
| 10 | SNR+3dB 15kHz FO 100ppb 10TRS | 2.5 | 2.0 |
| 9 | SNR-3dB 30kHz FO 100ppb 1TRS | 26.8 | 20.5 |
| 8 | SNR-3dB 30kHz FO 100ppb 2TRS | 19.6 | 17.0 |
| 7 | SNR-3dB 30kHz FO 100ppb 3TRS | 16.3 | 13.3 |
| 6 | SNR-3dB 30kHz FO 100ppb 5TRS | 12.8 | 10.9 |
| 5 | SNR-3dB 30kHz FO 100ppb 10TRS | 9.3 | 7.8 |
| 4 | SNR-3dB 15kHz FO 100ppb 1TRS | 14.4 | 11.4 |
| 3 | SNR-3dB 15kHz FO 100ppb 2TRS | 9.9 | 8.4 |
| 2 | SNR-3dB 15kHz FO 100ppb 3TRS | 8.0 | 6.7 |
| 1 | SNR-3dB 15kHz FO 100ppb 5TRS | 6.4 | 5.4 |
| 0 | SNR-3dB 15kHz FO 100ppb 10TRS | 4.6 | 3.9 |

| Estimated DO 0.5, ADO=1.0 |  |  |  |
| --- | --- | --- | --- |
|  | case | mean-ae DO [ns] | 50p-ae DO [ns] |
| 19 | SNR+3dB 30kHz DO 0.5CP 1TRS | 32.7 | 27.0 |
| 18 | SNR+3dB 30kHz DO 0.5CP 2TRS | 27.5 | 23.7 |
| 17 | SNR+3dB 30kHz DO 0.5CP 3TRS | 23.7 | 19.7 |
| 16 | SNR+3dB 30kHz DO 0.5CP 5TRS | 19.6 | 16.2 |
| 15 | SNR+3dB 30kHz DO 0.5CP 10TRS | 14.5 | 12.3 |
| 14 | SNR-3dB 30kHz DO 0.5CP 1TRS | 34.1 | 27.8 |
| 13 | SNR-3dB 30kHz DO 0.5CP 2TRS | 28.2 | 23.9 |
| 12 | SNR-3dB 30kHz DO 0.5CP 3TRS | 24.2 | 20.8 |
| 11 | SNR-3dB 30kHz DO 0.5CP 5TRS | 20.0 | 16.7 |
| 10 | SNR-3dB 30kHz DO 0.5CP 10TRS | 14.4 | 12.1 |
| 9 | SNR+3dB 15kHz DO 0.5CP 1TRS | 41.8 | 36.3 |
| 8 | SNR+3dB 15kHz DO 0.5CP 2TRS | 34.7 | 29.6 |
| 7 | SNR+3dB 15kHz DO 0.5CP 3TRS | 29.8 | 24.0 |
| 6 | SNR+3dB 15kHz DO 0.5CP 5TRS | 24.3 | 20.7 |
| 5 | SNR+3dB 15kHz DO 0.5CP 10TRS | 18.3 | 16.4 |
| 4 | SNR-3dB 15kHz DO 0.5CP 1TRS | 54.8 | 44.3 |
| 3 | SNR-3dB 15kHz DO 0.5CP 2TRS | 41.7 | 34.3 |
| 2 | SNR-3dB 15kHz DO 0.5CP 3TRS | 36.3 | 29.5 |
| 1 | SNR-3dB 15kHz DO 0.5CP 5TRS | 29.7 | 26.3 |
| 0 | SNR-3dB 15kHz DO 0.5CP 10TRS | 22.0 | 19.1 |