3GPP TSG-RAN WG4 Meeting #115																	R4-2507014

St. Julian’s, Malta, 19 - 23 May 2025

Agenda Item:	7.24.5

Source: 	Ericsson

Title:	On RRM performance aspects and test cases for MIMO Ph 5

Document for:	Discussion

# Introduction

From RAN4#115, RAN4 will start discussion on the performance part of the MIMO evolution Phase 5 WI, covering:

- Performance requirements,

- Test cases.

# Discussion

As part of the MIMO evolution WI phase 5, in the core part we are discussing following requirements

- CJTC core requirement

- Enhancement for UE-initiated/event-driven beam management

- Asymmetric DL sTRP/UL mTRP

## CJTC measurement performance requirements

The total error is a function of several factors, like quantization error and UE measurement error.

RAN1 defines the number of levels per report and the ranges. This defines the quantization error. We have, for the different Inter-TRP time misalignment and frequency/phase offset measurements, the following report resolutions and ranges:

- Delay: MD  ∈{128, 256}, AD ∈ {0.5⸱CP, 1⸱CP}

- Frequency offset: MFO = 256. AFO ∈ {0.1 ppm, 0.2 ppm}

- Phase offset: MΦ∈{16,32},  AΦ = 2π

In order to have control of total error we need to state the allowed UE measurement error. Agreeing on the UE measurement error budget requires a clear understanding of the intended application.

As discussed in previous meeting, different applications—such as coherent combining or nulling—require varying levels of phase estimation accuracy from the UE, depending on the operating region with respect to SNR. Consequently, each use case demands a different total error budget. For example:

- Coherent combining: total error budget of 50 degrees

- Nulling: total error budget of 30 degrees (low SNR) and 10 degrees (high SNR)

For CJT, we propose adopting a total error budget of 50 degrees for coherent combining. Within this budget, we suggest allocating 60% (i.e., 30 degrees) to UE measurement error. The remaining 20 degrees would account for other contributing error sources, as outlined in our previous paper, R4-2413017. These include:

- UE reporting resolution/quantization error

- BS algorithmic compensation error

- Inter-TRP phase drift

- Potential inter-TRP mast sway

- Potential UE mobility

Additionally, we note that this approach aligns with the analysis and recommendations presented by QC in their paper R4-2503128, which we also include here for convenience.

For DL, no requirements for coherency are defined. However, requirements for coherent UL MIMO are defined in as the maximum allowable difference of relative phase error over a time window. The allowable difference of relative phase error is defined as 40 over a time window of 20 ms. We think that this requirement can be also used as a reference for phase coherency in time and frequency domain for delay and frequency offset reporting. Observation 1: Requirements for coherent UL MIMO are defined in TS 38.101-1 as the maximum allowable difference of relative phase error over a time window. The allowable difference of relative phase error is defined as 40 over a time window of 20 ms. Proposal 1: RAN4 should discuss whether 40 can be used as a reference for phase coherency in time and frequency domain for delay and frequency offset reporting.

- For defining measurement accuracy for delay and frequency offset, we suggest a 50° total phase error budget for CJT coherent combining, with 30° allocated to UE measurement error and the remainder to other contributing factors.

Performance requirements for UE measurements are stated in section 10 of TS 38.133.

Based on the above analysis, we need performance requirements which states the measurement accuracy for different SNR and number of samples, since measurement accuracy improves with higher number of samples and higher SNR.

Observation 2: Accuracy requirements depend on signal level (SNR) and number of samples.

- Develop accuracy requirements for delay, frequency and phase offset UE measurement reports for:

- Different number of samples

- High and low SNR

- Normal and extreme conditions

Finally, we need to define test requirements for delay, frequency and phase offset UE measurement reports.

- Develop test cases for delay, frequency and phase offset UE measurement reports to test core and accuracy requirement.

## UE-initiated/event-driven beam management

RAN4 discussed the following aspects as part of the core requirements design:

- Measurement delay

- Measurement accuracy

- TCI state switching

We discuss the performance requirements for the above aspects.

Measurement delay:

RAN1 defined three events: Event-1, Event-2, and Event-7.

- Event-1: The quality of the current beam is worse than a certain threshold.

- Event-2: The quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

- Event-7: The quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality.

  - Q is RRC configured and subject to UE capability signalling. The UE may only indicate a single candidate value or not support Event-7.

RAN4 discussed the requirements for Event-2 with the assumption that the same framework would be extended to other events. From our perspective, Event-2 and Event-7 are the important events as they resemble the existing TCI state switching framework from the legacy. We believe that at least these events should be tested.

- RAN4 to define the test cases for Event-2 and Event-7

Furthermore, RAN1 defined two different reporting frameworks called Mode-A and Mode-B. At least Mode-A should be tested as this mode is a basic UE capability (i.e., all UEs supporting UE-initiated/event-driven beam reporting should support this feature). The descriptions of Mode-A and Mode-B from the RAN1 agreements are as follows:

- Mode A (dynamically scheduling UCI by gNB):

  - Step 1: UE transmits a first PUCCH (one-bit) to request a resource for a second UL channel to carry the beam report.

  - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry the beam report.

  - Step 3: Beam report is transmitted in the second UL channel.

  - This mode is a basic UE capability (i.e., all UEs supporting UE-initiated/event-driven beam reporting should support this feature).

  - No new DCI format is introduced.

- Mode B (UCI in pre-configured resource(s) for the second UL channel):

  - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry the beam report.

  - Step 2: UE transmits the beam report in the second UL channel.

  - The notification in Step 1 is in a separate reporting instance from the beam report in Step 2.

- RAN4 to consider at least Mode-A in the test case design.

Time window based:

Based on the RAN1 agreement, we believe its functionality is similar to that of the TTT timer, with one key difference: events do not need to persist throughout the entire window duration but must occur M number of times. In the L3 reporting, the UE can assess the event at the end of the TTT to determine if the event criteria have been still met and trigger the report.

In the UEI BM context, the UE must evaluate the event N number of times to confirm that it occurs M number of times. This functionality appears more stringent than the TTT for L3, where the UE can simply check at the end of the TTT to see if the event condition remains satisfied. Therefore, we suggest defining RAN4 testing to ensure that UE measurement behaviour is tested accurately.

- Define test cases with time window-based and non-time window-based configurations.

Test configuration:

For Event-2, we have the following definitions for Scheme-1 and Scheme-2:

- Scheme-1: RS for the current beam is the QCL RS in the indicated TCI state.

- Scheme-2: RS for the current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state.

Generally, the QCL RS in the indicated TCI state could be TRS, and the quality of the TRS may not be fair to compare with the quality of the new beam (which could be SSB). Hence, we believe that at least Scheme-2 should be considered for SSB based event triggered reporting testing. Once the scheme for Event-2 is agreed upon, the scheme for Event-7 can follow the same principle. This is further supported by RAN1 agreement shown below.

- Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam. - Note: In such case, the RS type comprises SSB and CSI-RS configured in a CSI-RS resource set configured with repetition.

- For the SSB based event triggered reporting, RAN4 to define the test cases by considering the Scheme-2 (RS for the current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state).

- For CSI-RS based test case design, RAN4 to define the test cases by considering the Scheme-1 (RS for the current beam is the QCL RS in the indicated TCI state).

Another important test configuration is the PUCCH configuration for the new UCI. One relevant agreement regarding the first PUCCH from RAN1 is listed below. Among the first PUCCH configurations, we believe that at least periodicityAndOffset should be specified in the test configuration. This ensures that the UE can send the indication to the NW node when the event-triggered condition is met during the first PUCCH occasion.

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding first PUCCH channel configuration, Alt-2 is supported for both mode-A and mode-B: the first PUCCH channel is a new UCI type - Alt-2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig- UEIBR, for the periodic PUCCH resource configuration. - It is RAN1’s understanding that the RRC parameter is NOT associated with SchedulingRequestId. - 1-bit to PUCCH resource is encoded by reusing the encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above applies at least for the single CC case. - Reuse multiplexing/dropping rule(s) of SR as baseline FFS: overlapping with SR/LRR or PUSCH

- RAN4 to specify the periodic first PUCCH resource in the test configuration (at least the periodicityAndOffset).

Measurement accuracy:

No filtering is agreed upon, and hence we expect the measurement accuracy to be the same as the legacy. We do not see a need for testing measurement accuracy or introducing new accuracy requirements.

Unified TCI state switching delay:

As discussed in the core part, we do not see MAC CE-based requirements changing, and we do not see a need for testing. For DCI-based TCI state switching, we understand that RAN1 is discussing enhancements for delay reduction in the case of DCI-based TCI state switching. We can revisit this after RAN1 reaches an agreement.

- No MAC CE-based TCI state switching test case is needed with event-driven reporting.

- For DCI-based TCI state switch test cases, wait for further RAN1 progress.

## Asymmetric DL sTRP/UL mTRP

In NR, RAN4 has introduced the following tests to verify the timing requirements for the UE:

- UE Transmit Timing Test

- Timing Advance Adjustment Accuracy

In this section, we examine the purpose and methodology of these tests in the legacy context to determine their relevance for Asymmetric DL sTRP/UL mTRP.

UE Transmit Timing Test:

Purpose and Method: The purpose of this test is to verify that the UE can follow the frame timing changes of the connected gNB. It ensures that the UE's initial transmit timing accuracy, the maximum amount of timing change in one adjustment, and the minimum and maximum adjustment rates are within the specified limits. This test verifies the requirements outlined in clause 7.1.2.

Applicability to Asymmetric DL sTRP/UL mTRP: Although the UE supports two Timing Advances (TAs) for PUCCH/PUSCH/SRS, only a single DL reference timing is supported. For the UL-only TRP case, this test does not add significant value, as the UE only needs to track the frame timing changes of the reference TRP.

Timing Advance Adjustment Accuracy:

Purpose and Method: Timing advance commands are sent to the UE and Sounding Reference Signals (SRS) are transmitted from the UE and received by the test equipment. By measuring the reception of the SRS, the transmit timing, and hence the timing advance adjustment accuracy, can be assessed.

Applicability to Asymmetric DL sTRP/UL mTRP: We believe that timing advance adjustment accuracy can be tested for this feature. However, there is nothing particularly unique about UE supporting UL-only TRP in this context of the test we will be verifying. Therefore, we do not see significant benefits in conducting this test specifically for UL-only TRP.

As part of the core requirement design, RAN4 is discussing the PDCCH order-based DL reference timing aspects. Test case for that can be discussed once the core requirements are agreed.

# Summary

The following have been proposed in this contribution.

- For defining measurement accuracy for delay and frequency offset, we suggest a 50° total phase error budget for CJT coherent combining, with 30° allocated to UE measurement error and the remainder to other contributing factors.

- Develop accuracy requirements for delay, frequency and phase offset UE measurement reports for:

- Different number of samples

- High and low SNR

- Normal and extreme conditions

- Develop test cases for delay, frequency and phase offset UE measurement reports to test core and accuracy requirement.

- RAN4 to define the test cases for Event-2 and Event-7

- RAN4 to consider at least Mode-A in the test case design.

- Define test cases with time window-based and non-time window-based configurations.

- For the SSB based event triggered reporting, RAN4 to define the test cases by considering the Scheme-2 (RS for the current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state).

- For CSI-RS based test case design, RAN4 to define the test cases by considering the Scheme-1 (RS for the current beam is the QCL RS in the indicated TCI state).

- RAN4 to specify the periodic first PUCCH resource in the test configuration (at least the periodicityAndOffset).

- No MAC CE-based TCI state switching test case is needed with event-driven reporting.

- For DCI-based TCI state switch test cases, wait for further RAN1 progress.

# References

[1] R4-2317371