3GPP TSG-RAN WG4 Meeting # 115	R4-2507022

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.21.4

Source: 	National Taiwan University

Title: 	Beam Prediction Performance Metrics for Monitoring and Accuracy Requirement

Document for:	Approval

Introduction

We present our view on partially specified decoder and reference decoder derivation procedure in this contribution.

Discussion

Discussion on Beam ID Prediction KPI

For beam ID prediction, the following was agreed in the previous meeting[1]:

The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values

The proponent of top-N, N>1 options provide the following analysis in [2]:

Soption3=1Nt n=1Ntlmaxbk ∈ set ARSRPbk≥maxbk ∈ set ARSRPbk-X   (1)

And the following proposal:

- The threshold for ith ranked predicted beam, bi can be considered as the RSRP of the corresponding ith ranked genie-aided beam, bi – X

- The success rate of correct prediction can be computed as a weighted average that considers all top-K predicted beams

Weighted success rate, Sw=1Nt n=1Nti=1Kwi,n lRSRPbi,n≥RSRPbi,n-X  (2)

Based on the following observation:

For a fair evaluation, the threshold for ith ranked predicted beam, bi could be set as RSRP of the corresponding ith ranked genie-aided beam bi – X. Hence, the condition to be checked for the ith ranked beam is Ci=lRSRPbi≥RSRPbi-X, where X is a fixed value for all i

However, we found that when we have N>1, we may have conflicted success/failure results when K>1 with the additional criterion introduced by allowing N>1. Here is the example, considering x=3dB and K=2 (as the K defined in WF, not the proposal):

| Beam ID | Genie RSRP RSRPbi,n | Reported ranking |  |
| --- | --- | --- | --- |
| 1 | -90dBm |  |  |
| 2 | -90.5dBm | 2 |  |
| 3 | -92dBm |  |  |
| 4 | -93.1dBm | 1 |  |

Based on the WF

“The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB”

The UE made a correct prediction. However, based on the proposal in [2] for N = 2 (which is K in (2)) case, then we have

i=1:  lRSRPb1,n≥RSRPb1,n-X=l-93.1dBm≥-90dBm-3=0

i=2:  lRSRPb2,n≥RSRPb2,n-X=l-90.5dBm≥-90.5dBm-3=1

The success rate is 50%, we don’t think the success rate of 50% can considered as pass, and therefore we see conflict results when we have N>1. Therefore, we suggest to remove the FFS part regarding N.

Proposal 1: Remove the FFS part related to N in the previous meeting agreement:

The successful rate for the correct prediction,

  - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB,

  - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation

    - FFS on N values

  - FFS on x values

Conclusion

Proposal 1: Remove the FFS part related to N in the previous meeting agreement:

The successful rate for the correct prediction,

  - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB,

  - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation

    - FFS on N values

  - FFS on x values