3GPP TSG-RAN WG4 Meeting #115	R4-2507027

# Malta, MT, 19 - 23 May, 2025

Title: 	Discussion on SBFD demodulation performance requirements

Source: 	Huawei,HiSilicon

Agenda item:	7.25.6

Document for:	Discussion

Background

During RAN#104 meeting, Revised WID [1] on evolution of NR duplex operation: Sub-band full duplex (SBFD) was approved. In this contribution, we share our view on SBFD demodulation performance requirements.

Discussion

UE demodulation performance requirements

General

Number of DL subband

RAN1 is still discussing whether two DL subband and one UL subband case is an optional feature based on UE capability. So we propose to consider one DL subband and one UL subband case as high priority. FFS whether to consider two DL subband and one UL subband case based on RAN1 agreement.

Consider one DL subband and one UL subband case as high priority. FFS whether to consider two DL subband and one UL subband case based on RAN1 agreement.

Number of cases

From our understanding, a UE that supporting SBFD carrier also support TDD carrier, it is most likely a new TDD pattern, so it is enough to verify the performance for a UE supporting SBFD by testing the existing TDD cases and limited number of additional SBFD cases. There is no need to define lots of cases independent of TDD.

Define limited number of additional SBFD cases.

Channel bandwidth & SCS

For the channel bandwidth and SCS, we propose to use the typical value for the existing TDD cases in TS 38.101-4, i.e. 40MHz DL subband + 20MHz UL subband for 30kHz SCS and 100MHz DL subband + 50MHz UL subband for 120kHz SCS.

Use the typical value for the existing TDD cases in TS 38.101-4, i.e. 40MHz DL subband + 20MHz UL subband for 30kHz SCS and 100MHz DL subband + 50MHz UL subband for 120kHz SCS.

TDD pattern

In RAN4 demodulation performance requirements definition and in the real network deployment, DDDDDDDSUU for 30kHz SCS and DDDSU for 120kHz SCS is typical used. To be compatible with the above TDD patterns, we propose to use XXXXXXXXUU for 30kHz SCS and XXXXU for 120kHz for RAN4 SBFD performance demodulation requirements definition.

Use XXXXXXXXUU for 30kHz SCS and XXXXU for 120kHz for RAN4 SBFD performance demodulation requirements definition.

Repetition

For the repetition, RAN1 consider all repetitions restricted to SBFD symbols only or non-SBFD symbols only as baseline and allow considering no such restriction based on UE capability. However, based on our understanding, there is no baseband algorithm changed comparing to the existing repetition cases defined in TS 38.101-4. So we propose to not define repetition cases for SBFD.

Do not consider repetition cases for SBFD.

PDSCH

MCS & Rank

For the MCS and rank, one typical value can be selected, e.g. MCS 13 and rank 2.

For PDSCH demodulation performance requirements, use MCS 13 and rank 2.

Propagation condition

For the propagation condition, one typical value can be selected, e.g. TDLC300-100.

For PDSCH demodulation performance requirements, use TDLC300-100.

HARQ feedback

Considering half-duplex operation and faster HARQ feedback, the HARQ timeline for XXXXXXXXUU TDD pattern for 30kHz SCS and XXXXU TDD pattern for 60kHz SCS is proposed as following Figure 2.1.2-1 and Figure 2.1.2-2 respectively. 4 HARQ processes is enough for both TDD pattern.

Figure 2.1.2-1 HARQ timeline for XXXXXXXXUU TDD pattern for 30kHz SCS

Figure 2.1.2-2 HARQ timeline for XXXXU TDD pattern for 120kHz SCS

For PDSCH demodulation performance requirements, use the HARQ timeline for XXXXXXXXUU TDD pattern for 30kHz SCS and XXXXU TDD pattern for 60kHz SCS as above Figure 2.1.2-1 and Figure 2.1.2-2 respectively.

Use 4 HARQ processes for both TDD pattern.

Other parameters

For other parameters, we propose to use the typical PRB bundling value for the existing TDD cases in TS 38.101-4, as following Table 2.1.2-1.

Table 2.1.2-1 Other parameters for PDSCH demodulation performance requirements

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |

For PDSCH demodulation performance requirements, use the parameters as above Table 2.1.2-1.

CSI reporting

Based on RAN1 agreements, separate CSI-RS resource and reporting for SBFD and non-SBFD are supported. CSI reporting for non-SBFD has been verified by the existing cases in TS 38.101-4. From our understanding, for one DL subband and one UL subband case, the baseband processing of CSI reporting for SBFD is similar as that of CSI reporting for non-SBFD. So we propose to not consider any CSI reporting cases for SBFD if only one DL subband is considered.

Not consider CSI reporting cases for SBFD with one DL subband.

CLI handling

Based on RAN1 agreements, L1 CLI-RSSI/ SRS-RSRP measurement and reporting is supported based on existing CSI framework. From baseband processing point of view, there is only payload content change with new report quantities introduced and no baseband processing impact. Also, whether UE reporting is accurate enough can be verified by RRM test. So we propose to not define any UE demodulation performance requirements for CLI handing.

Do not define any UE demodulation performance requirements for CLI handing.

BS demodulation performance requirements

Default TDD pattern

In RAN4 demodulation performance requirements definition and in the real network deployment, DDDDDDDSUU for 30kHz SCS and DDDSU for 120kHz SCS is typical used as the default TDD pattern. To be compatible with the above TDD patterns, we propose to use XXXXXXXXUU for 30kHz SCS and XXXXU for 120kHz as the default TDD pattern for RAN4 SBFD performance demodulation requirements definition.

Use XXXXXXXXUU for 30kHz SCS and XXXXU for 120kHz as the default TDD pattern for RAN4 SBFD performance demodulation requirements definition.

PUSCH

Based on RAN1 agreements, UL resource muting for PUSCH is supported, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain. PUSCH resource mapping, i.e., rate-matching around the muted REs is considered. From our understanding, there is baseband processing impact if UL resource muting for PUSCH is enabled. In addition, BS can use the muting resource for gNB-gNB CLI handling, e.g. calculate interference covariance for better interference suppression to get better demodulation performance. So we propose to define PUSCH cases with UL resource muting with gNB-gNB interference modeling for both DFT-S-OFDM and CP-OFDM.

Define PUSCH cases with UL resource muting with gNB-gNB interference modeling for both DFT-S-OFDM and CP-OFDM.

PUCCH

From demodulation point of view, we don’t see any enhancement for PUCCH, so we propose to not define any PUCCH demodulation performance requirements for SBFD.

Do not define any PUCCH demodulation performance requirements for SBFD.

PRACH

Based on RAN1 agreements, RO within SBFD symbols is supported. However, there is no new PRACH format defined, we don’t see any baseband processing algorithm changed. So we propose to not define any PRACH demodulation performance requirements for SBFD.

Do not define any PRACH demodulation performance requirements for SBFD.

Proposals

In this contribution, we share our view on SBFD demodulation performance requirements. Our observations and proposals are:

- Consider one DL subband and one UL subband case as high priority. FFS whether to consider two DL subband and one UL subband case based on RAN1 agreement.

Define limited number of additional SBFD cases.

Use the typical value for the existing TDD cases in TS 38.101-4, i.e. 40MHz DL subband + 20MHz UL subband for 30kHz SCS and 100MHz DL subband + 50MHz UL subband for 120kHz SCS.

Use XXXXXXXXUU for 30kHz SCS and XXXXU for 120kHz for RAN4 SBFD performance demodulation requirements definition.

Do not consider repetition cases for SBFD.

For PDSCH demodulation performance requirements, use MCS 13 and rank 2.

For PDSCH demodulation performance requirements, use TDLC300-100.

For PDSCH demodulation performance requirements, use the HARQ timeline for XXXXXXXXUU TDD pattern for 30kHz SCS and XXXXU TDD pattern for 60kHz SCS as above Figure 2.1.2-1 and Figure 2.1.2-2 respectively.

Use 4 HARQ processes for both TDD pattern.

For PDSCH demodulation performance requirements, use the parameters as above Table 2.1.2-1.

Not consider CSI reporting cases for SBFD with one DL subband.

Do not define any UE demodulation performance requirements for CLI handing.

Use XXXXXXXXUU for 30kHz SCS and XXXXU for 120kHz as the default TDD pattern for RAN4 SBFD performance demodulation requirements definition.

Define PUSCH cases with UL resource muting with gNB-gNB interference modeling for both DFT-S-OFDM and CP-OFDM.

Do not define any PUCCH demodulation performance requirements for SBFD.

Do not define any PRACH demodulation performance requirements for SBFD.

Reference

RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), RAN#104, Huawei