# 3GPP TSG-RAN WG4 Meeting #115	  R4-2507030

# Malta, MT, 19th – 23rd May, 2025

Title: 	Overviews on 6Rx performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	7.1.4

Document for:	Discussion

Background

A WF [1] for 6Rx performance requirements was agreed in last meeting, this contribution provides our views.

- Discussions

- Release independent for 6Rx requirements

- It’s proposed not to introduce release independence for 6Rx requirements, since 6Rx requirements target at handheld UE, which is commonly equipped with 4Rx at early release, there is no evidence showing that handheld UEs are widely equipped with 6Rx at early release. It is aggressive to introduce release independence for 6Rx requirements for handheld UE. Furthermore, regarding 6 MIMO layers, there are inter-operation issues between UE and BS to support such feature, BS needs to support the new UE capability and change the scheduling strategy by taking into max DL 6 MIMO layers on top of the current scheduling.

Proposal 1: Not release independent for 6Rx requirements and applicable from Rel-19.

- Test applicability rule for 6Rx test cases

One issue is test applicability rule for 6Rx test cases, candidate options are:

- Option 1: RAN4 can introduce UE declaration on the 6Rx UE type (e.g., handheld or FWA) without capability signaling and design proper test applicability in the specification. - Option 2: - 6 MIMO layers performance requirements will be only applicable for FWA devices - 4 MIMO layers performance requirements will be applicable to both FWA and handheld UE devices.

- Option 2 is copy of agreement which is introduced in [3] and option 1 provides more information on how to define applicability rules. However, it is still on discussion whether to differentiate UE type for capability signaling of supporting 6 layers. Hence the details of applicability rules could be postponed until there 6 layers capability signaling design is finished.

Proposal 2: RAN4 to agrees option 2 and the details of applicability rules could be postponed until there 6 layers capability signalling design is finished.

PDSCH requirements

Test scenarios

One issue is test scenarios for PDSCH requirements, the candidate options are:

- Way forward - Option 1: Test scenarios without interference: PDSCH Mapping Type A - Option 2: Test scenarios with interference - PDSCH demodulation requirements with inter-cell interference - PDSCH demodulation requirements with intra-cell inter-user interference

We don’t support option 2, Rel-19 is first release to introduce 6Rx, hence RAN4 shall focus on basic feature. There is no motivation to introduce interference scenario. Also, according to the WID [2], test scope is limited to PDSCH requirements with 4 layers and 6 layers, such high rank cannot be scheduled in interference scenario.

Proposal 3: RAN4 shall only focus on test scenarios without interference: PDSCH Mapping Type A and preclude interference scenarios

Test cases for 4 MIMO layers for FWA and Handheld UE

One issue is number of tests for FWA and Handheld UE, the candidate options are:

- Way forward - Option 1: One single test case for 4 MIMO layers that is applicable for both FWA and Handheld UE - Option 2: Two separate test cases for 4 MIMO layers for FWA and Handheld UE - Option 2a: One test case with ULA Low for FWA; Another test case with ULA Medium with β = 0.00718 for Handheld UE - Other options

β = 0.00718 is derived from the assumption that the correlation of 6Rx between antenna pairs is identical to that of 4Rx. I.e. β4Rx1/9=β6Rx1/25and following antenna correlation matrix is applied:

RUE6Rx=1β125β425β925β1625ββ125*1β125β425β925β1625β425*β125*1β125β425β925β925*β425*β125*1β125β425β1625*β925*β425*β125*1β125β*β425*β925*β425*β125*1

To check the performance with such correlation matrix, a simulation is performed with assumption of some MCS values:

Figure 2.1-1: Simulation results for PDSCH with 4-layer with beta=0.00718

It is observed that for MCS17, 70% of max TP is unachievable； for MCS13, the SNR@70% of maxTP is around 24dB, even for QPSK(MCS9), the target SNR is quite high (>20dB). The observation shows that rank4 is less likely to be scheduled under the condition of beta=0.00718 due to the poor performance.

Observation 1:  Under the condition of beta=0.00718, 4 layers is less likely to be scheduled due to the poor performance.

Proposal 4: Consider Low correlation for 4 layers cases. Define single test which is applicable for both FWA and handheld UE.

Propagation channel and MIMO correlation for 6 MIMO layers

One issue is propagation channel and MIMO correlation for 6 MIMO layers, the candidate options are:

- Way forward - Option 1: TDLA 30-10 ULA Low - Option 2: TDLD 30-5 ULA Low

In the deployment, high MIMO layers is more likely be scheduled in the case of a large number of scattering paths, TDLD channel has LOS path with large power, which is not suitable for MIMO transmission since each Tx-Rx path is high correlated. Therefore, option 1 is supported.

Proposal 5: Use TDLA30-10 Low for 6 layers case.

Propagation channel for 4 MIMO layers

One issue is propagation channel for 4 MIMO layer, the options are:

- Way forward - Option 1: TDLA30-10 - Option 2: TDLC300-100 - Option 3: TDLA30-10 and TDLC300-100

We support option 1, TDLA30-10 is most common channel model used for PDSCH test especially for high layer test. We don’t see any technical issues to change it.

Proposal 6: Use TDLA30-10 Low for 4 MIMO layers case.

DMRS configuration for 6 MIMO layers case

One issue is DMRS configuration for 6 MIMO layer case, the options are:

- Way forward - Option 1: Rel-15 Type 1 DMRS 2+2 - Option 2: Rel-15 Type 1 DMRS 2+0 - Option 3: Rel-18 eType1 DMRS 1+1

We support option 1, R15 DMRS type 1 is basic feature, which is widely supported by most UEs. Our investigation shows that additional DMRS symbol is always configured in the deployment to against time-selective fading. For option 3, eType 1 DMRS is advanced feature introduced in Rel-18, there is no evidence that this feature has been widely used in the deployment.

Proposal 7: Use Rel-15 Type 1 DMRS 2+2 for 6 MIMO layers case.

Antenna configuration for 4 MIMO layers case

One issue is antenna configuration for 4 MIMO layer case, the options are:

- Way forward - Option 1: 4Tx6Rx - Option 2: 8Tx6Rx

We support option 1, it is enough to configure 4Tx for 4MIMO layers, increasing number of Tx verifies nothing from UE behaviour point view.

Proposal 8: Use 4Tx6Rx for 4MIMO layer case

MCS for 6 MIMO layers case

One issue is MCS for 6MIMO layers case, options are:

- Way forward - Option 1: MCS13 (Table 2) - Option 2: MCS 13 as starting point (Table 1) - Option 3: 256QAM - Option 4: 64QAM (MCS Table 1) with the same MCS for both CW - Option 5: MCS17 as starting point (Table 1)

A simulation was performed to check the performance for 256QAM, the lowest MCS (MCS 20) corresponding to 256QAM was assumed:

Figure 2.1-1: Simulation results for 6 layers, MCS20

It can be observed that even for MCS20, 70% of max throughput is unachievable.

Observation1: For 6 layers, even for the lowest MCS(MCS20), 70% of max throughput is unachievable.

We don’t see any substantial difference between MCS 13 and MCS17, our preference is MCS13 to be aligned with Rel-15 4Rx 4Layer test.

Proposal 8: RAN4 to use MCS13 (Table 1) for 6-layer test.

MCS for 4 MIMO layer case

The options for MCS for 4 MIMO layer case are shown as follows:

- Way forward - Option 1: MCS13 (Table 2) : 64QAM with CR = 567/1024 - Option 2: MCS 13 as starting point (Table 1) : 16QAM with CR = 490/1024 - Option 3: 256QAM - Option 4: 64QAM (MCS Table 1) - Option 5: MCS17 as starting point (Table 1): 64QAM with CR = 438/1024 - Option 6: MCS13 and MCS 17 as starting point

A simulation was performed to check the performance for 256QAM, the lowest MCS (MCS 20) corresponding to 256QAM was assumed:

Figure 2.1-2: Simulation results for 4 layers, MCS20

It can be observed that the SNR@70% of max TP is around 22dB, considering additional margin to be added, the final may be 25dB, which is too high for requirement definition. Hence we propose not consider 256QAM for rank 4.

We don’t see any substantial difference between MCS 13 and MCS17, our preference is MCS13 to be aligned with Rel-15 4Rx 4Layer test.

Proposal 9: RAN4 to use MCS13 (Table 1) for 4-layer test.

SDR requirements

DMRS configuration

The purpose of the test is to verify that the Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by UE capabilities. To achieve the maximum date rate indicated by UE capabilities, lower DMRS overhead could be considered for 6 layers, such as R15 DMRS type 2. Since the channel model is static channel without external noise, DMRS configurations has negligible impact on performance.

Proposal 10: RAN4 to consider R15 DMRS type 2 1+1 for 6-layer test.

Static channel model

The test setup for SDR test shall ensure that no any kind of interference (Including inter layer interference) and external noise exists so that max data rate declared by UE can be verified correctly.

Proposal 11: The test setup for SDR test shall ensure that no any kind of interference and external noise exists so that max data rate declared by UE can be verified correctly.

Traditional static channel matrix has the characteristic that the power of each element is 1 and all column vectors of the matrix are orthogonal to each other, the second characteristic ensures that given assumption of random PMI, no inter-layer interference exists. In other words, if static channel is not column orthogonal, there will be inter layer interference with some PMI. Noted that random precoder is configured for SDR test.

Observation 2: Traditional static channel matrix in 38.101-4 has two characteristics:

The power of each element is 1

All column vectors of the matrix are orthogonal to each other, which ensures that given assumption of random PMI, no inter-layer exists.

However, it is challengeable to apply both characteristics to 4T6R static channel. Our proposal is to use Gram-Schmidt Orthogonalization scheme to generate four orthogonal vectors as four columns of channel matrix. The proposed static channel matrix is shown as follows:

- H=111111111-1-1-122j-2j 22j3010j305j-33010j-22j2j-22j-3010j305j-3010j

The power of some elements may not equal to 1, this may not be an issue since the average power of all elements is 1, hence the total signal power is equal to the case that the power of all elements of channel matrix equal to 1.

Proposal 12: Confirm following static channel for 4T6R:

- H=111111111-1-1-122j-2j 22j3010j305j-33010j-22j2j-22j-3010j305j-3010j

For 8T6R, the problem is that we cannot find a column-orthogonal static channel matrix since number of column vectors is more than dimension of column vector, which cause an issue that static channel is directional and some PMI will cause inter-layer interference.

Observation 3: For 8T6R, the problem is that a column-orthogonal static channel matrix cannot be found since number of column vectors is more than dimension of column vector, which cause an issue that static channel is directional and some PMI will cause inter-layer interference.

Our proposal is to fix PMI firstly and derive the channel matrix to make the final channel matrix column orthogonal, the procedure is as shown as follows:

          - Fixed precoder w with i1,1=i1,2=i2=0

          - Derive the static channel model for 8T6R as follows:

H=wHw-1wH=34*11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11

          - Normalize the H to make power of each element of H equal to 1:

H=11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11

It can be easily proved that the final channel matrix from layer to Rx is Hw=43wHw-1wHw=43I6x6, which means layers to Rx is one to one mapping and there is no inter layer interference. Noted that 43 is beamforming gain which is derived from NTxNLayer

Proposal 13: Confirm following as static channel for 8T6R with fixed precoder i1,1=i1,2=i2=0

H=11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11

Simulation results

Figure 2.2-1 and Table 2.2-1 show our initial simulation results based on our proposed static channel matrix.

Figure 2.2-1: Simulation assumptions for SDR test

Table 2.2-1: Simulation summary for SDR test

| MCS | SNR(dB)@85% of maxTP |  |  |
| --- | --- | --- | --- |
|  | MCS Table 1 | MCS Table 2 | MCS Table 4 |
| 28 | 19.9 |  |  |
| 27 | 18.8 | 26.0 |  |
| 26 | 17.7 | 24.6 |  |
| 25 | 16.7 | 23.7 | 28.3 |
| 24 | 15.7 | 22.7 | 26.1 |
| 23 | 14.7 | 20.8 | 24.5 |
| 22 |  | 19.7 |  |

SDR test for 1024QAM

One of issues is whether define SDR requirements for 1024QAM with 4 layers:

- Agreement - Define SDR requirements for 1024QAM with 2 layers - FFS Define SDR requirements for 1024QAM with 4 layers

Up to 2 layers is defined for 1024QAM even for 8Rx, we don’t see the reason why 4 layers is not defined for 8Rx but defined for 6Rx considering 8Rx UE has higher capability. Hence, we don’t support defining SDR requirements for 1024QAM with 4 layers.

Proposal 14: Don’t define SDR requirements for 1024QAM with 4 layers.

MCS value

Considering maximum MCS values needs simulations, the results depends on the channel matrix, it’s proposed to defer it until static channel matrix is agreed.

Proposal 15: Defer the MCS value discussion until static channel matrix is agreed.

CSI requirements

CQI test

One of issues is whether to define CQI test, the candidate options are:

- Way forward - Option 1: Not define CQI test - Option 2: Define CQI test - Option 2a: Only define CQI requirements with rank 4 under static channel - Option 2b: Define the following CQI test - Periodic CQI reporting requirements under AWGN - Wideband CQI reporting requirements with inter-cell interference under fading conditions

We prefer to deprioritize CQI test, the motivation of CQI test is not clear. The SINR difference between 4Rx and 6Rx at receiving side is 10log64=1.7dB, which is less than SINR threshold difference between adjacent CQI index. I.e. 2dB. Therefore, we don’t think calculated CQI index will change much between 4Rx and 6Rx.

Observation 4: The SNR increases by 1.7dB when the number of Rx changes from 4 to 6, which is less than SINR threshold difference between adjacent CQI index, i.e. 2dB, which means calculated CQI index will not change much between 4Rx and 6Rx.

Proposal 16: Don’t define CQI test.

RI test

One of issues is whether to define RI test, the candidate options are:

- Way forward - Option 1: Not define RI test for 6 layers - Option 2: Define RI test for 6 layers

The motivation of RI test is not clear, according to the outcome from core part, only FWA device can support 6 layers. However, 8Rx is also only supported by FWA device, but we didn’t define RI test for 8Rx. We don’t see the technical reason why RI test is not defined for high capability FWA device with 8Rx but will be defined for low capability FWA device with 6Rx.

Proposal 17: Don’t define RI test.

- Conclusion

This contribution provides our overviews on 6Rx performance requirements, the proposals and observations are:

Proposal 1: Not release independent for 6Rx requirements and applicable from Rel-19.

Proposal 2: RAN4 to agrees option 2 and the details of applicability rules could be postponed until there 6 layers capability signalling design is finished.

Proposal 3: RAN4 shall only focus on test scenarios without interference: PDSCH Mapping Type A and preclude interference scenarios

Observation 1:  Under the condition of beta=0.00718, 4 layers is less likely to be scheduled due to the poor performance.

Proposal 4: Consider Low correlation for 4 layers cases. Define single test which is applicable for both FWA and handheld UE.

Proposal 5: Use TDLA30-10 Low for 6 layers case.

Proposal 6: Use TDLA30-10 Low for 4 MIMO layers case.

Proposal 7: Use Rel-15 Type 1 DMRS 2+2 for 6 MIMO layers case.

Proposal 8: Use 4Tx6Rx for 4MIMO layer case

Observation1: For 6 layers, even for the lowest MCS(MCS20), 70% of max throughput is unachievable.

Proposal 8: RAN4 to use MCS13 (Table 1) for 6-layer test.

Proposal 9: RAN4 to use MCS13 (Table 1) for 4-layer test.

Proposal 10: RAN4 to consider R15 DMRS type 2 1+1 for 6-layer test.

Proposal 11: The test setup for SDR test shall ensure that no any kind of interference and external noise exists so that max data rate declared by UE can be verified correctly.

Observation 2: Traditional static channel matrix in 38.101-4 has two characteristics:

The power of each element is 1

All column vectors of the matrix are orthogonal to each other, which ensures that given assumption of random PMI, no inter-layer exists.

Proposal 12: Confirm following static channel for 4T6R:

- H=111111111-1-1-122j-2j 22j3010j305j-33010j-22j2j-22j-3010j305j-3010j

Observation 3: For 8T6R, the problem is that a column-orthogonal static channel matrix cannot be found since number of column vectors is more than dimension of column vector, which cause an issue that static channel is directional and some PMI will cause inter-layer interference.

Proposal 13: Confirm following as static channel for 8T6R with fixed precoder i1,1=i1,2=i2=0

H=11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11

Proposal 14: Don’t define SDR requirements for 1024QAM with 4 layers.

Proposal 15: Defer the MCS value discussion until static channel matrix is agreed.

Observation 4: The SNR increases by 1.7dB when the number of Rx changes from 4 to 6, which is less than SINR threshold difference between adjacent CQI index, i.e. 2dB, which means calculated CQI index will not change much between 4Rx and 6Rx.

Proposal 16: Don’t define CQI test.

Proposal 17: Don’t define RI test.

- Reference

[1] R4-2504750 WF for 6Rx UE performance requirements_RAN4#114bis_final

[2] RP-250780 New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4. Huawei, HiSilicon

[3] R4-2503014 WF on 6Rx requirements. AT&T