3GPP TSG-RAN WG4 Meeting #115	R4-2507033

# Malta, MT, 19th – 23rd May, 2025

Title: 	Discussions on 8Rx performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	7.18.2

Document for:	Discussion

Background

WF R4-2502275 [1] was agreed in RAN4-114 meeting, this contribution provides our views on related open issues.

- CQI for inter cell interference

For CQI requirements, there is an open issue whether to define two sets of requirements:

CQI calculation algorithm and Receiver coverage Agreement: - Companies are encouraged to provide suggestions on methodology for deciding CQI reporting requirements in next meeting. - Decision to have one or two sets of requirements is postponed to next meeting when more simulation results are available.

Our simulation results are captured in Table 2-1:

Table 2-1: Simulation results for CQI requirements with inter cell interference

| SINR | FDD |  |  | TDD |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Baseline receiver | Simplified receiver | Gap Gama(Baseline receiver)- Gama(Simplified receiver) | Baseline receiver | Simplified receiver | Gap Gama(Baseline receiver)- Gama(Simplified receiver) |
| -6 | 2.93 | 5.17 | -2.24 | 2.64 | 5.46 | -2.82 |
| -5 | 3.97 | 4.56 | -0.59 | 3.71 | 4.53 | -0.82 |
| -4 | 4.41 | 3.31 | 1.1 | 4.43 | 3.05 | 1.38 |
| -3 | 3.55 | 2.67 | 0.88 | 3.26 | 2.51 | 0.75 |
| -2 | 3.00 | 2.91 | 0.09 | 2.85 | 2.68 | 0.17 |
| -1 | 2.85 | 3.33 | -0.48 | 2.87 | 3.03 | -0.16 |
| 0 | 3.04 | 3.12 | -0.08 | 2.96 | 2.77 | 0.19 |
| 1 | 3.42 | 2.67 | 0.75 | 3.48 | 2.37 | 1.11 |
| 2 | 3.89 | 2.40 | 1.49 | 3.97 | 2.30 | 1.67 |
| 3 | 3.99 | 2.51 | 1.48 | 4.09 | 2.56 | 1.53 |
| 4 | 3.42 | 2.66 | 0.76 | 3.71 | 2.70 | 1.01 |
| 5 | 2.59 | 2.64 | -0.05 | 2.53 | 2.84 | -0.31 |

It can be observed that the advantage of baseline receiver over simplified receiver is not stable. In some SNR points baseline receiver outperforms simplified receiver, in some other SNR points simplified receiver outperforms baseline receiver. Based on the observation, it seems unreasonable to differentiate two receivers for CQI test. Our proposal is to select the SINR point where both baseline receiver and simplified receive can show significant gain and define one set of requirements to cover both receivers. Regarding the SINR point and Gama, it’s proposed to set SINR to -3dB and Gama to 2.5 considering some margin.

Observation 1: Regarding the Gama value, the advantage of baseline receiver over simplified receiver is not stable.

Proposal 1: Select the SINR point where both baseline receiver and simplified receive can show significant gain and define one set of requirements to cover both receivers.

Proposal 2: Set SINR=-3dB and Gama=2.5

- Applicability rules

Following options for applicability rules to skip 2/4Rx requirements were made:

Test applicability to skip 2/4Rx requirements Proposals - Option 1: For UEs supporting 8Rx, 2Rx and/or 4Rx, if UE passes 8Rx tests for inter-cell interference (PDSCH and CQI) and intra-cell inter-user interference (PDSCH), the 2 and/or 4Rx tests for inter-cell interference (PDSCH and CQI) and intra-cell inter-user interference (PDSCH) can be skipped, no matter same test configurations as 2RX/4RX are defined or not. - Option 2: To consider if UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed. - Option 3: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed. - Option 4: Introduce test applicability under inter-cell interference and under intra-cell inter user interference to skip a 2/4Rx requirement if at least the following applies for each considered 8Rx test: - UE support both 8Rx and 2/4Rx - The UE passes an 8Rx requirement with same rank and interference configuration as the 2/4Rx requirement. - Option 5: RAN4 shall have the following applicability rules: - UE that passes Rank2 ICI test case with 8Rx shall skip Rank1 ICI requirements with 4Rx. - UE that passes Rank2 MU-MIMO test case with 8Rx shall skip Rank1/2 MU-MIMO requirements with 4Rx.

The key step of MMSE-IRC receiver is interference plus noise covariance matrix calculation, our understanding is that performing interference plus noise covariance matrix calculation with 8Rx is much more complicate than that with 2/Rx. 8Rx UE doesn’t need to pass the test that which is easier to the UE if the UE has passed the test which is complicate to the UE.

Proposal 3: Adopt Option1: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

Following options related independency were made:

Release independency Agreement: - The new Rel-19 requirements shall at least be release independent from Rel-18. Remaining proposals: - Option 1: The new Rel-19 requirements shall also be release independent from Rel-17. - Option 2: The new Rel-19 requirements shall also be release independent from Rel-17. The requirements shall be mandatory for Rel-18 and optional from Rel-17.

Noted that support of 8Rx is introduced in Rel-18 but release independent from Rel-17. Considering that MMSE-IRC is basic feature without any UE capability, it’s reasonable to define the requirements to be release independent from Rel-17. To be conservative, the requirements could be optional for Rel-17.

Proposal 4: The new Rel-19 requirements shall also be release independent from Rel-17. The requirements shall be mandatory for Rel-18 and optional from Rel-17.

- Conclusion

This contribution provides our simulation results for 8Rx requirements with interference. The proposals are:

Observation 1: Regarding the Gama value, the advantage of baseline receiver over simplified receiver is not stable.

Proposal 1: Select the SINR point where both baseline receiver and simplified receive can show significant gain and define one set of requirements to cover both receivers.

Proposal 2: Set SINR=-3dB and Gama=2.5

Proposal 3: Adopt Option1: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

Proposal 4: The new Rel-19 requirements shall also be release independent from Rel-17. The requirements shall be mandatory for Rel-18 and optional from Rel-17.

- Reference

[1]  R4-2419784 Way forward for [114][324] NR_demod_Ph5_Part2_UE. Nokia