3GPP TSG RAN WG4 Meeting #115		 R4-2507060

St Julian’s, Malta  19th – 23th May, 2025

Agenda Item:	7.7.3

Source: 	MediaTek Inc.

Title: 	Discussion on low-band CA switching time mask and fallback applicapability

Document for:	Approval

1. Introduction

The low band carrier aggregation has been discussed. The WI scope, low NR band aggregation scenarios and switching periods has been concluded in RAN4. The time mask and location of the switching period were still open for further discussion. Furthermore, in last RAN1 meeting, some agreements were reached and RAN4 need continue discuss corresponding RAN4 requirements and potential UE capability and UE behaviour.

2. Discussion

Issue 1: The location of switching period

In last RAN4 meeting, it was agreed the location of switching period is still under RAN1 discussion. Referring to last RAN1#120bis meeting(April.2025) outcome, there were some agreements copied below:

Agreement

For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap,

- For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier.

- For switching from FDD carrier to SDL carrier, downselect one of the following solutions

  - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier

  - Alt 1: to protect PDCCH

  - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier

  - Alt 3: Switching gap is always assumed at the SDL carrier

It can be found the location of switching period from SDL carrier to FDD carrier has been concluded, always assumed at the “switch from” carrier, i.e., SDL carrier. The location of switching period from FDD carrier to SDL carrier is still under discussion in RAN1.

Observation 1: The location of switching period from SDL carrier to FDD carrier shall always be at the “switch from” carrier, i.e., SDL carrier. RAN1 will further discuss location of switching period from FDD carrier to SDL carrier

Issue 2: Time mask for switching

In last RAN4 meeting, time mask for switching is still open for discussion. Following the observation in issue 1 discussion, the time mask can be specified for the switching from SDL carrier to FDD carrier. And to follow another RAN1 agreement copied below, we provide the time mask proposal in proposal 1.

RAN1#120bis agreement:

For Rel-19 low NR band carrier aggregation via switching, the semi-static switching pattern and corresponding switching gap are based on the downlink timing (DL reception time at UE side) of Pcell.

Proposal 1: The proposed switching time mask from SDL carrier to FDD carrier is based on the downlink timing (DL reception time at UE side) of Pcell and illustrated in below Figure 1. The network shall guarantee enough switching gap including 10us transient period, switching period Xus and timing advance (TA)

Figure 1. Time mask for the LB CA switching

The switching time mask from FDD carrier to SDL carrier will depend on the discussion outcome of next RAN1 meeting.

Issue 3: Applicable fallback configuration discussion of higher order combo when supporting of the low NR band switching feature is reported

In last RAN1 meeting, the agreements were reached and copied below:

RAN1 Agreement

- For Rel-19 low NR band carrier aggregation via switching, support only semi-static configuration of switching pattern to a UE based on UE-specific RRC configuration between the following two cases

- Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

- Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

- FDD carrier 1 is PCell and SDL carrier 2 is SCell

- SCS 15KHz on both carriers

For the 2nd bullet, if following RAN1 agreement, when basket WI for LB CA switching is introduced, some questions may need to be clarified. Taking an existing higher order CA combo, CA_n5-n29-n66, which includes CA_n5-n29 as example. There are two possible cases for the combo:

- Case A: LB CA switching capability is supported and reported for CA_n5-n29 in CA_n5-n29-n66, then CA_n29-n66 cannot be the fallback due to band n5 shall be the Pcell.

- Case B: LB CA switching is not supported for CA_n5-n29 in CA_n5-n29-n66, CA_n29-n66 can be the fallback when band n66 is configured as Pcell.

Then it would result in a question that when LB CA switching capability is supported and reported, band n5 shall be the Pcell of the combination, what are valid fallback configurations? Refer to TS38.306 clause 3.1 definition of the fallback:

Fallback band combination: A Uu band combination that would result from another Uu band combination (parent band combination) by releasing at least one SCell or uplink configuration of SCell, or SCG, or SUL.

Then the valid fallback configuration for case A can be CA_n5-n66 or CA_n5-n29. The CA_n66-n29 would not be valid configuration. RAN4 need to figure out how to specify the corresponding fallback configuration in the TS38.101-1 and also to make sure the UE capability can indicate the case clearly.

Observation 2: If LB CA switching capability is supported and reported for band pair “CA_n5-n29” in CA_n5-n29-n66, CA_n29-n66 would not be valid fallback configuration due to band n5 shall be the Pcell

Proposal 2: For the example case A: LB CA switching capability is supported and reported for CA_n5-n29 in CA_n5-n29-n66. CA_n29-n66 cannot be valid fallback due to band n5 shall be the Pcell. RAN4 need to figure how to specify corresponding requirements

Proposal 3: UE feature list shall take outcome of proposal 2 into consideration

3. Conclusion

Observation 1: The location of switching period from SDL carrier to FDD carrier shall always be at the “switch from” carrier, i.e., SDL carrier. RAN1 will further discuss location of switching period from FDD carrier to SDL carrier

Proposal 1: The proposed switching time mask from SDL carrier to FDD carrier is based on the downlink timing (DL reception time at UE side) of Pcell and illustrated in below Figure 1. The network shall guarantee enough switching gap including 10us transient period, switching period Xus and timing advance (TA)

Figure 1. Time mask for the LB CA switching

Observation 2: If LB CA switching capability is supported and reported for band pair “CA_n5-n29” in CA_n5-n29-n66, CA_n29-n66 would not be valid fallback configuration due to band n5 shall be the Pcell

Proposal 2: For the example case A: LB CA switching capability is supported and reported for CA_n5-n29 in CA_n5-n29-n66. CA_n29-n66 cannot be valid fallback due to band n5 shall be the Pcell. RAN4 need to figure how to specify corresponding requirements

Proposal 3: UE feature list shall take outcome of proposal 2 into consideration

Reference:

- RP-243315 New WID on low band carrier aggregation via switching, RANP#106

- RP-241537 Rx switched nX-n29 low-low carrier aggregation, RANP#104

- R4-2503016 WF on low-low band switching, RAN4#114

- R4-2501121 Discussion on low-band CA switching, RAN4#114