3GPP TSG-RAN WG4#115	R4-2507063

Malta, MT, May 19th – 23rd, 2025

Agenda item:	6.11.2

Source: 	Qualcomm France

Title: 	CA_n5A-n8A

Document for:	Approval

# Introduction

Aspects for CA_n5A-n8A are discussed in this contribution.

- Background

In previous meeting, the aspect of the need to do dynamic RFFE reconfiguration for 2UL/2DL CA_n5A-n8A was brought up, and was extensively discussed. No official WF was agreed, instead it was written in Chairman notes as follows associated with noted Tdoc [1] R4-2505124

Chair: encourage companies to evaluate the two solutions in this LS.

In addition, associated with noted Tdoc [2] the following is captured in Chairman notes.

Chair: Continue the discussions of CA n5-n8 in the simultaneous Rx-Tx WI and no upscope of discussion is allowed.

Content of [1] is shown below for convenience.

- New issues raised by some companies in this meeting for Option 2

  - Interrupt needed for RF front end configuration change is required

  - Interrupt needed for RFFE configuration change PA ON/OFF: PA On/Off as the switching between the case1 (n5+n8DL/n5UL) and case2 (n5DL/n5+n8UL) cannot be done with keeping the PAs ON while switching

- For the newly raised issues, two solutions are proposed and discussed:

  - Solution #1 – switching by activation and deactivation:

    - If UE reports support of 2UL for CA_n5-n8, network only activate either n5DL or n8UL at a time

    - Change of cases is done in such a way that to no extra interrupts/switching time is needed

      - Staying in Case 1 longer than Case 2:

      - FFS: impact on RRM interrupt requirements

        - If RRM requirement impacts are identified, drop Option 2 in Rel-19

    - RAN2 capability signaling is required

  - Solution #2 - New capability: introduce a UE capability of switching gap (i.e., switching period and location) to cover necessary switching period including RFFE configuration switching, PA On/Off etc.,

    - Can be the same for both directions (n5DL -> n8UL, n8UL->n5DL), or one for each direction

    - Network accommodates the switching gap by scheduling

      - Cross-carrier scheduling might be needed (e.g., from Case 2 with only 1 DL n8DL to Case 1 with two DL)

    - No RAN1 spec change is required

- Further improvement, e.g., via L-L CA via switching is not precluded in Rel-20 or later

- Companies are encouraged to evaluate if solution #1 or solution #2 is feasible to be used to specify n5-n8 2UL/2DL CA in Rel-19

- Discussion

Solution 1:

This solution would require significant amount of work not only on the RAN4 on RRM side but also in RAN2 and potentially RAN1 as the current CA framework does not support such operation mode/procedures. RAN2 work on the framework to activate/deactivate UL or DL carriers as outlined in solution 1 would be required. Hence, as written as part of this solution, this option should be dropped from Rel-19

Proposal 1: Do not pursue solution 1 in Rel-19 due to need of RRM side considerations

Solution 2:

This solution as lots of similarities with LB-LB switching, the difference being than in this case there would be no semi-switching pattern defined by RAN1. An another difference is that in LB-LB switching the switching is between DL CC’s while in this case it’s between DL and UL so also e.g. timing advance comes into play. In previous meeting we considered that this could be workable, but after more detailed look we think there is too much ambiguity and similarities to LB-LB switching associated with this solution. Our understanding is that the definition of semi-static switching patterns for LB-LB switching has turned out to be quite challenging and laborious task in RAN1. The assumption of NW taking care of non-concurrent scheduling and certain gap between n5DL and n8UL and vice versa would go even beyond using the semi-static switching pattern. So in other words, when definition of semi-static switching pattern for LB-LB switching is challenging, why should RAN4 make band combination who is not relying on semi-static switching pattern but instead vague “NW taking care of non-concurrent scheduling of n5 DL and n8 UL with a switching gap”, even when the switching in this case would include more aspects than DL-DL switching.

In previous meeting, there were a few comments that as it was agreed to make 2UL/2DL CA in Rel-19 we should do it, no matter when new aspects are brought up, and the work could still be continued in Rel-20 with another solutions. From deployment and device perspective, this is very problematic. NW scheduling would become very complex when you have UE’s which do assume NW to take care of everything (Rel-19) and UE’s relying on semi-static switching pattern (Rel-20). It would be very bold assumption to build a UE supporting Rel-19 solution, as in reality it could well be that NW eventually goes with Rel-20 solution meaning Rel-19 UE’s would have the extra HW and the large amount work spent on making everything working including a lot of testing but would never operate it. Instead of having step-wise solutions, we should really make one solution which works.

For such a solution, the amount of work needed is not very clear and no full system solution has been proposed so far. Given that Rel-19 is supposed to close in 1-2 meetings in RAN1 and RAN2, there would be no time to specify a clear workable solution.

Proposal 2: Do not pursue solution 2 in Rel-19 due to too high ambiguity.

Wrapping up proposals 1 and 2, it does not look possible to define 2UL/2DL CA for CA_n5A-n8A in Rel-19. Furthermore, a clear use case and system level gains from this feature have not been clearly shown. Consequently, there is no need to send band LS to RAN2 at this point of time, unless RAN4 wants to say the work is not pursued in Rel-19

Proposal 3: Do not send LS back to RAN2 unless RAN4 wants to say the work is not pursued in Rel-19

- Conclusion

Considerations on CA_n5A-n8A with UL CA_n5A-n8A were considered with the following proposals.

Proposal 1: Do not pursue solution 1 in Rel-19 due to need of RRM side considerations

Proposal 2: Do not pursue solution 2 in Rel-19 due to too high ambiguity

Proposal 3: Do not send LS back to RAN2 unless RAN4 wants to say the work is not pursued in Rel-19

- Reference

[1] R4-2505124, “draft reply LS to RAN2 on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx”, CATT

[2] R4/2505093, “WF on simultaneous Rx-Tx issues for UL CA_n5-n8 and others|, Huawei