3GPP TSG-RAN WG4#115	R4-2507064

Malta, MT, May 19th – 23rd, 2025

Agenda item:	6.3.3

Source: 	Qualcomm France

Title: 	About UL CA_n28-n71

Document for:	Approval

# Introduction

Considerations on UL CA_n28-n71 are provided in this contribution.

- WF from RAN4#114bis

WF[1] is as follows.

Proposal 1: RAN4 can investigate the following RF architectures in order to enable the UL CA_n28-n71.

Option 1: Three-antenna RF architecture below as an example. (NOTE: it isn’t precluded to implement Full band duplexer for band n28.)

Option 2: Two-antenna RF architecture with two triplexers.

Option 3: Other RF architectures are not precluded, e.g. studies in TR38.872 on CA_n28-n105.

Proposal 2: RAN4 can investigate the impacts on the following RF requirements in order to enable the UL CA_n28-n71.

- The attenuation between UL band n71 and UL band n28.

- MSD for both DL band n28 and n71 due to IMD3 and IMD5.

- Potential revisit on the existing MSD due to cross band isolation.

- Delta Tib and Rib

- MPR requirements based on the investigation whether intermodulation interference will excceed the SEM or spurious emission requirements.

- The band protection requirements of -50dBm/MHz.

NOTE: Whether to specify 2UL is based on the outcome of the studies.

Proposal 3: it’s encouraged for companies to evaluate the following 2nd test point MSD for DC_28_n71.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | 28 | 680.5 | 35 | 15 | 20 (RBstart=168) | 760.5 | 5 | FFS | >ACLR2 |

- Discussion

The frequency arrangement for this CA combination is illustrated below in Figure 1. There is only 5MHz gap between the UL’s of the bands.

Figure 1 Frequency arrangement of CA_n28-n71

Multiplexing these bands together is not a viable option due to too small gap. Instead, either two antenna solution with triplexing or three antenna solution would need to be considered.

Observation 1: Either two-antenna solution with triplexing or three antenna solution would need to be considered.

Due to only 5MHz gap between the TX passbands, the attenuation between n28 <-> n71 UL’s is single digit dB’s by n28 TX filter, and practically 0dB’s by n71 TX filter. The small difference is due to fact that n28 must have good attenuation below the TX passband to meet emissions levels at 470-694MHz (NS_N17) and up to 698MHz, while there is nothing just above the upper end of n71 passband except n29 RX at 717-729MHz.

Because of the above limited attenuation between TX passbands, this UL CA configuration resembles Intra-band non-contiguous UL CA with 2PA’s quite a bit; two TX chains equipped with filters which do not attenuate each other/attenuate very little. 3GPP has specified requirements for Intra-band non-contiguous UL CA, where the MPR to meet -13dBm/MHz, -25dBm/MHz and -30dBm/MHz emissions limits is high. We illustrate an example case where the 5MHz UL’s are placed closest to each other, which represents the worst case from emissions perspective. We note that while both cases of n28+n71 UL creating IMD’s at n28 and at n71 would need to be evaluated, the case of IMD landing at n71 passband is more crucial because n71 TX filter is not at all attenuating n28 TX at least within the lowermost MHz’s.

Figure 2 Example UL allocation and resulting IMD3 range

We note that this case is not exactly same as already specified Intra-band non-contiguous UL CA, because RF filters do begin to attenuate aggressor at certain offsets, and that the spurious emission limits are different; for below 1GHz, the spurious emission limit is -36dBm/100kHz instead of -30dBm/1MHz. Overall, making power back-off specifications for this, let it be MPR or A-MPR, would require significant amount of work in 3GPP.

The tricky part here is that for the allocations where the back off is needed (= where the RF filter does not attenuate enough), the back off is so large that the combination is likely not meaningful. In other words, rigorous work would be required to find frequencies at which the RF filters attenuate aggressors enough warranting no back-off is required to make the combination meaningful. Quickly looking, the allocations which are located at upper end (n71) and are lower end (n28) are those which require back-off.

Observation 2: Power back-off to meet general emission mask and spurious emissions would be needed, which would require significant amount of work in 3GPP much beyond the timeline of Rel-19.

As evaluated in [2], both IMD3 and IMD5 land on top of both n28 and n71 DL’s, with a whopping 38dB as proposed IMD3 number. We note that if IMD3 is around that level, also IMD5 is substantial. We note that actually the frequencies where IMD3’s exist are such which likely are not subject to (large) back-off. Hence, unfortunately the back-off due to general emissions/spurious emissions or IMD3 at own DL limit the performance is within different locations of the bands.

Observation 3: Power-back off, if done rigorously, impacts UL performance for allocations where CC’s are close to each other while IMD3 impacts DL performance at many other allocations than those impacted by power back-off.

In summary, both the overall work required in 3GPP and the complexity associated with this combination we propose not to specify UL CA_n28-n71. For DC_28_n71 switched UL as agreed in RAN4#114bis can be used.

Proposal 1: Not to specify UL CA_n28-n71

For the DC_28A_n71A MSD with 35MHz n71 UL the TX impairments dominate.

Proposal 2: Use the following number when evaluating MSD for DC_28A_n71A with 35MHz n71 UL

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | 28 | 680.5 | 35 | 15 | 20 (RBstart=168) | 760.5 | 5 | 13.7 | >ACLR2 |

- Discussion

Considerations on UL CA_n28-n71 were provided with the following observations and proposal.

Observation 1: Either two-antenna solution with triplexing or three antenna solution would need to be considered.

Observation 2: Power back-off to meet general emission mask and spurious emissions would be needed, which would require significant amount of work in 3GPP much beyond the timeline of Rel-19.

Observation 3: Power-back off, if done rigorously, impacts UL performance for allocations where CC’s are close to each other while IMD3 impacts DL performance at many other allocations than those impacted by power back-off.

Proposal 1: Not to specify UL CA_n28-n71

Proposal 2: Use the following number when evaluating MSD for DC_28A_n71A with 35MHz n71 UL

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | 28 | 680.5 | 35 | 15 | 20 (RBstart=168) | 760.5 | 5 | 13.7 | >ACLR2 |

- Reference

[1] R4-2505216, “WF for uplink CA n28-n71”, Huawei

[2] R4-2503687, “TP for TR 38.719-02-01 on introduction of CA_n28A-n71A”, Huawei