3GPP TSG RAN WG4 Meeting #115	R4-2507095

Malta, MT, 19th – 23rd May, 2025

Source:	GIST, RRA, CSU

Title:	Discussion on Near-Field to Far-Field Transform Method for OTA TRP Measurement

Agenda Item:		7.14.2

Document for:	Discussion

Introduction

As over-the-air (OTA) total radiated power (TRP) measurement evolves to meet the demands of increasingly complex device designs and 6G-capable user equipment (UE), there is a growing need to consider alternative test methodologies that are compact, flexible, and forward-compatible. Among these, the near-field to far-field (NFTF) transformation method offers promising capabilities.

NFTF reconstructs 3D radiation patterns using amplitude and phase data collected in the near field. A prototype system, RRA-MAP (Multi-Antenna Probing system), has been developed through collaboration between National Radio Research Agency (RRA), Gwangju Institute of Science and Technology (GIST), and Chosun University (CSU). This system uses a spherical probe array with simultaneous probe activation and relies on GPS-disciplined rubidium oscillators for multiprobe synchronization. These design choices enable efficient full-sphere measurements suitable for high-frequency and multi-antenna configurations.

Discussion

In previous RAN4 contributions, we have already detailed the measurement procedures and calibration methods necessary to implement NFTF in practice. These include the spatial sampling strategy, phase stability assurance, and probe calibration techniques that ensures the reliability of the RRA-MAP system.

The RRA-MAP system is in the final stages of development, with expected completion by August 2025. A system diagram is repeated below. It features simultaneous probe activation in the elevation. The spatial sampling density can be arbitrarily increased by the azimuthal rotation and goniometer movement.

When compared to existing OTA methods such as anechoic chamber (AC) and reverberation chambers (RC), the NFTF method provides several distinct benefits:

Figure 1. Diagram of the NFTF System

- Accuracy: Enables detailed 3D radiation pattern reconstruction for precise TRP integration.

- Measurement Efficiency: Minimizes mechanical movements and supports simultaneous sampling to reduce test time.

- Device Coverage: Well-suited for complex or distributed antennas such as those in XR devices, wearables, and UAVs.

- ISAC Readiness: The method’s spatial resolution naturally supports integrated sensing and communication (ISAC) applications.

Considering these strengths, we suggest that NFTF be further discussed as a potential addition to future OTA TRP methodologies. Its inclusion may provide enhanced test flexibility and measurement fidelity for emerging device categories.

Conclusion

This contribution presents NFTF as a candidate approach for OTA TRP measurement in both FR1 and FR2, supported by the capabilities of the RRA-MAP system. We invite RAN4 to consider the method’s applicability in future study or technical evaluation discussions.

The RRA-MAP system is expected to be operational by the August 2025 RAN4 meeting, at which point initial measurement results may be available. We would like to continue contributing to the discussion on OTA TRP methodologies, including consideration of NFTF.

- Reference

- R4-2307504: CR for TR38.810: Integrating simultaneouly active probe concept in the NFTF method

- R4-2400727: Discussion on NFTF System for UE Transmitter Characteristics Measurement

- R4-2407599: Discussion on the Operation of the Probe Module Array for NFTF Measurement

- R4-2411308: UE RF Testing Procedure under NFTF Measurement Setup

- TR 38.870: Enhanced OTA Test Methods for NR FR1 TRP and TRS