3GPP TSG-RAN WG4 Meeting #115	R4-2507113

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.10.5.1

Source:	MediaTek Inc.

Title:	Discussion on NSGO channel model for NTN testing

Document for:	Discussion

# Introduction

At RAN4#114-bis meeting, a WF for NSGO channel model was agreed [1]. In this contribution, we provide our views on outstanding issues.

# Discussion

Issue 1-1-7 Accuracy requirements for maximum delay errors to obtain the propagation delay and Doppler shift trajectory - agreement - The number of iterations to solve Kepler’s equation based on Newton-Raphson method is up to TE implementation - The interpolation method for the Doppler shift and Propagation delay trajectory is up to TE implementation - Note: the earth rotation during signal flight time impact should be considered to calculate the propagation delay - To derive accuracy requirement, - Baseline assumption - number of iterations is fixed to 4 for solving Kepler’s equation based on Newton-Raphson method - Calculate the Propagation delay and Doppler shift every 1ms - For accuracy requirement - Apply linear interpolation to calculate the propagation delay and Doppler shift with 1ms granularity as baseline, no restrict the implementation on the TE side - Consider the propagation delay error is less than TBD (s) and Doppler shift error is less than TBD (Hz) - The time period to calculate the error is 1s - TE should meet the minimum accuracy requirement defined. - Companies are encouraged to provide the acceptable value with associated propagation delay error and Doppler shift error calculation method for accuracy requirement in the next meeting

Considering implementation complexity for generating time varying Doppler shift and delay at TE side, RAN4 agreed to left TE having its own implementation about “number of iterations to solve Kepler’s equation” and “interpolation method”. To guarantee that each TE will generate similar results, RAN4 discussed how to provide accuracy requirements for TE. Based on the agreement at RAN#114-bis meeting, there is a baseline:

- number of iterations is fixed to 4 for solving Kepler’s equation based on Newton-Raphson method

- Calculate the Propagation delay and Doppler shift every 1ms

In Figure 1 and 2, we compare the results of baseline “calculate the Propagation delay and Doppler shift every 1ms” with “calculate the Propagation delay and Doppler shift every 1second and linear interpolate with 1ms granularity”. We consider LEO-600 as the error of LEO-1200 should be smaller than that of LEO-600 because of smaller change rate of LEO-1200.

| Figure 1. Difference in Doppler shift between “1ms Kepler” and “1sec Kepler with 1ms liner interpolation” | Figure 2. Difference in time delay between “1ms Kepler” and “1sec Kepler with 1ms liner interpolation” |
| --- | --- |

Also, in Figure 3 and 4, we present the the mean of absolute error in each for each 1 second. Within each m-th 1 second period, there are 1000 points for deriving the mean of absolute error.

EDopplerm=11000n=0999Dopplerbaselinem+n*0.001-Dopplerinterpm+n*0.001

EDelaym=11000n=0999Delaybaselinem+n*0.001-Delayinterpm+n*0.001

| Figure 3. Mean of absolute error for Doppler | Figure 4. Mean of absolute error for Delay |
| --- | --- |

Based on results, we propose that, for each 1 second period, the mean of absolute error for Doppler should be less than 1 Hz and the mean of absolute error for Delay should be less than 0.05µs.

Proposal 1: The mean of absolute error for Doppler should be less than 1 Hz and the mean of absolute error for Delay should be less than 0.05µs.

# Conclusion

In this contribution, we provide our views on NTN testing for NGSO. Proposals are summarized as below.

Proposal 1: The mean of absolute error for Doppler should be less than 1 Hz and the mean of absolute error for Delay should be less than 0.05µs.

# Reference

- R4-2504700, Way Forward for [114bis][323] NTN_testing_NGSO_channel_model_and_demod, Samsung