3GPP TSG-RAN WG4 Meeting #115	R4-2507116

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.31.7

Source:	MediaTek Inc.

Title:	Discussions on UE demodulation requirements for Rel-19 NR-NTN

Document for:	Discussion

# Introduction

Based on the WID [1], RAN4 is tasked to specify UE demodulation requirements for the Rel-19 NTN phase3 WI. In this contribution, we provide our views.

- Discussion

There are several objectives in the WID of Rel-19 NTN phase 3 and only two objectives related to UE demodulation.

| Objectives | Sub-features |
| --- | --- |
| Downlink coverage enhancements | - PDCCH at least for CSS (except for Type-3) via PDCCH repetition - PDSCH with Msg4 via PDSCH repetition - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access |
| RedCap support | - Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands |

We discuss the necessity and feasibility of introducing UE demodulation requirements for each sub-feature.

PDCCH at least for CSS (except for Type-3) via PDCCH repetition

Proposal 1: RAN4 to discuss whether to introduce requirements for PDCCH repetition.

PDSCH with Msg4 via PDSCH repetition

In RAN5 conformance test, it needs enough samples to determine whether UE pass the requirements or not. As Msg4 is a part of RACH process, it may need a long testing time to have enough sample to verify whether UE pass the requirements or not. Therefore, we prefer not to define UE demodulation requirements for PDSCH with Msg4 via PDSCH repetition.

Proposal 2: Do not define demodulation requirements for “PDSCH with Msg4 via PDSCH repetition” as it may require extensive testing time, given that Msg4 is part of the RACH process.

PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration

SIB1 is broadcasted by the gNB periodically with PDSCH. Since it is a broadcast message intended for all UEs in the cell, there is no mechanism for individual UEs to send ACK/NACK for its reception. This feature cannot be tested because there is no ACK/NACK feedback from the DUT to the TE to verify the correctness of reception.

Proposal 3: Do not define demodulation requirements for “PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration” as there is no ACK/NACK feedback from the DUT to the TE to verify the correctness of reception.

Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access

As there is no PBCH conformance test in RAN5 specification, we suggest not to define requirements for PBCH.

Proposal 4: Do not define demodulation requirements for “extended periodicity of the half frames with SS/PBCH blocks” as there is no PBCH conformance test in RAN5 specification.

Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands

Based on the Note in Table 8.2.1.1.2-1, the requirements defined in TS38.101-4 for Redcap and eRedcap are all applicable to RedCap and Rel-18 eRedCap UEs with NR NTN operating. However, we are open to discuss whether to introduce new RedCap and eRedCap requirements under NR NTN operating.

Proposal 5: RAN4 to discuss whether to introduce requirements for Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands.

# Conclusion

In this contribution, we provide our views about performance requirements for Rel-19 NTN phase3. Observations and proposals are summarized as follows.

Proposal 1: RAN4 to discuss whether to introduce requirements for PDCCH repetition.

Proposal 2: Do not define demodulation requirements for “PDSCH with Msg4 via PDSCH repetition” as it may require extensive testing time, given that Msg4 is part of the RACH process.

Proposal 3: Do not define demodulation requirements for “PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration” as there is no ACK/NACK feedback from the DUT to the TE to verify the correctness of reception.

Proposal 4: Do not define demodulation requirements for “extended periodicity of the half frames with SS/PBCH blocks” as there is no PBCH conformance test in RAN5 specification.

Proposal 5: RAN4 to discuss whether to introduce requirements for Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands.

# Reference

- RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT