3GPP TSG-RAN WG4 Meeting #115	R4-2507120

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.26.4

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impacts for R19 A-IoT device

Document for:	Discussion

# Introduction

The study item of Ambient IoT [1] was completed with WI approve in [2] and further updated in [3]. The RRM impacts were initially discussed in last RAN4 meeting with agreements captured in [4]. In this contribution, we provide our views on remaining issues for Ambient-IoT.

# Discussion

In last RAN4 meeting, the agreement on RRM core requirements scope are summarized as follows:

Agreement: - RAN4 not to define RRM requirements for A-IoT paging Agreement: - Excepting the timing issue discussed in Issue 1-4, no other potential RRM requirement impact is identified for “A-IoT data transmission” Agreement: - RAN4 to include RRM requirements for correct A-IoT device behaviour during CBRA and CFRA. FFS the details Agreement: - Further discuss whether there is RRM impact on transmit timing requirements of device pending on RAN1 conclusion.

In the following part of the contribution, we provide our analysis on RRM requirements for RA and transmit timing requirements.

## Random access

It is agreed in last RAN4 meeting to include RRM requirements for correct A-IoT device behaviour during CBRA and CFRA. Based on the progress in last RAN2 meeting, the skeleton of the new MAC spec is endorsed [5], and there is a post meeting discussion on the details of the running CR after April meeting. Based on the progress in last two RAN2 meetings, we have clearer picture on the RA procedures for A-IoT device, and the relevant agreements are summarized as follows:

General RAN2#129 - We will specify both CBRA and CFRA - Re-use the subsequent paging message to trigger re-access. There is no need to differentiate msg1 resource for initial access vs re-access. - NACK based mechanism is supported for D2R messages to determine re-access for at least msg3. FFS details including whether we need a timer or explicit message and when reader sends feedback. - RAN2 assumes that device randomly selects among FDMA occasions as the baseline. RAN2#129bis - Introduce an explicit 1 bit indication to indicate whether it is CFRA or CBRA per paging message. - A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling. The R2D message indicates the start of a set of MSG1 resources that were configured in paging message. - Assumption: The R2D message does not include slot number/count down number. - IoT Msg2 contains one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices. - Same Msg2 format is used for initial transmission and retransmission of Msg2.

CBRA RAN2#129 - For Rel-19, only 3-step CBRA is supported for A-IoT RAN2#129bis - In case of CBRA, only 16 bits random ID is included in Msg1. FFS can be revisited if message type will be needed for other D2R messages purposes - For CBRA, as a baseline, NACK based mechanism is applied only to the Msg3. May come back for D2R data, if the NACK feedback indication is needed for the purpose to stop/terminate the “on-going procedure” and release the AS ID accordingly (depending on other later discussion).

CFRA RAN2#129bis - RN16 is not included in the first D2R message in the CFRA procedure. AS ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader. FFS if we can assume or need to support multiple device scenario. - For CFRA, NACK feedback and re-access is not supported

Considering that the details are still under discussion in RAN2, RAN4 could first agree on the high-level RA requirements for RAN4 CR preparation, and the details can be added when RAN2 running CR is stable. Based on our understanding, following requirements/sub-clause should be included for RRM requirements for RA.

- Contention-Based Random Access procedure

  - Transmission of Random ID message

E.g. Device perform Contention-Based Random Access based RA type indication, and select an access occasion for transmission of the 16-bitRandom ID message

  - Reception of Random ID Response message

E.g. if the value indicated in Echoed Random ID field in Random ID Response message is identical to the value of the Random ID field in the transmitted Random ID message, consider this Random Access procedure is successfully completed, and initiate D2R message transmission.

- Contention-Free Random Access procedure

  - Transmission of first D2R message

E.g. Device perform Contention-Free Random Access based RA type indication, and initiate the D2R message transmission

Proposal 1: RAN4 to include following aspects for RRM requirements for RA. The details of UE behaviour to be updated based on RAN2 MAC spec.

- Contention-Based Random Access procedure

  - Transmission of Random ID message

  - Reception of Random ID Response message

- Contention-Free Random Access procedure

  - Transmission of first D2R message

## Device transmit timing requirements

In last RAN4 meeting, RAN4 discussed the potential RRM impacts on timing requirements. It is agreed to further evaluate whether to define transmit timing requirements of device pending on RAN1 conclusion. RAN1 achieve following agreements in last meeting.

Agreement For Msg 1 transmission determined by one R2D transmission triggering random access, support X=1 and X=2 time domain resource(s) for D2R transmission(s) for Msg1 only, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s) in Rel-19. - All devices support the above - Note: the impact of specification support (at least including signalling overhead) for X=2 to a reader supporting only X=1 should be minimized Only support X=1 time domain resource for D2R transmission for Msg3 in response to a PRDCH for Msg2 transmission. Agreement A device is not required to monitor a corresponding R2D transmission earlier than TD2R_min after the end of a D2R transmission from the device. Conclusion For any timing requirement that needs to be specified from device perspective by RAN1, the timing(s) do not include the impacts of SFO from the RAN1 perspective. - Whether 3GPP needs to specify some requirement on device’s SFO is up to RAN4 discussion. - Note: RAN1 will not specify TR2D_min or TR2D_max in Rel-19 - Note: SFO may still be considered when discussing time-domain resources for Msg1 when X=2 Agreement Use 1 bit to indicate the value of X (X=1 or X=2) time domain resource(s) for Msg1 transmission(s). Agreement For X=1 or X=2, from device perspective, the starting time for the first Msg1 time domain resource is Toffset1 after the end of the corresponding R2D transmission triggering random access. - FFS Toffset1 is predefined in the spec or indicated implicitly or explicitly by the R2D transmission triggering random access. - FFS detailed value(s) of Toffset1, considering at least the device processing time including FEC impact - FFS: how to define the end of the R2D transmission in 9.4.1 Working assumption For X=2, from device perspective, for determination of the starting time for the second Msg1 time domain resource: - Derived based on the starting time of the first Msg1 time domain resource plus Toffset2 - FFS Toffset2 is predefined in the spec or derived by a rule or indicated implicitly or explicitly by the R2D transmission triggering random access.

Based on above RAN1 agreements, the framework of timing relationship is rather stable and we can have clear picture on the how to determine the D2R transmit timing.

Observation 1: Based on latest RAN1 progress, device determines the start time of MSG1 as:

- For MSG 1 X=1, the starting time of first Msg1 based on Toffset1. The starting time for the first Msg1 time domain resource is Toffset1 after the end of the corresponding R2D transmission triggering random access.

- For MSG1 X=2, the starting time of Second Msg1 based on Toffset1 and Toffset2. The starting time for the second Msg1 time domain resource is Toffset1 + Toffset2 after the end of the corresponding R2D transmission triggering random access.

- The value of Toffset1 and Toffset2, and how to define the end of R2D transmission will be defined in RAN1.

In addition, RAN1 clarify that the discussion in RAN1 about the timing relationship will not include the impact on SFO (sampling frequency offset), and it is up to RAN4 to decide whether to define SFO related requirements. We illustrate the impact on D2R transmit timing due to SFO in following figure. The ideal transmit timing of MSG1 shall be Toffest1 after the end the R2D for device perspective. However, due to the poor accuracy of SFO (105 PPM as assumed for device 1), there is timing error between the ideal transmit timing and actual transmit timing.

Figure 1. Timing error due to SFO

Regarding the value/range of timing error for TR2D, we can take the RFID spec [6] as reference which is quoted as follows. In RFID system, T1 serves the same purpose as TR2D for A-IoT. T1 is defined as a window as FrT is considered jointly. FrT is the frequency tolerance which is the SFO used in A-IoT discussion.

<EPC® Radio-Frequency Identity Generation-2 UHF RFID Standard>

In RFID system, T1 is time from the last rising edge of the last bit to the first the rising edge of the Tag reply. It is assumed that Tag can perform synchronization until the last rising edge of the interrogator transmission, and after that the accumulated error increases.

Similar as RFID tag, A-IoT device can achieve synchronization from the R2D transmission by line coding, as captured in the TR. But compared with RFID, the different part for R2D is the postamble and padding at the end of the R2D system. Whether there is available edge for tracking in the postamble and padding is still under discussion in RAN1.

TR.38.769 #### 6.1.1.2 R2D line coding The line codes studied for R2D are Manchester encoding and PIE. For R2D reception, the device can use line codes to achieve at least chip-level time tracking by using the transition(s) of the line code. A chip corresponds to one modulated symbol.

In details, the accumulated timing error for D2R transmission is shown in following figure. In figure 2, considering that there is no edge available for tracking, the last edge is in the last modulated bit.

Figure 2. Time error for D2R transmission

Based on the similar principle as RFID, the timing error is calculated as T1*SFO. The longer T1 will lead to larger timing error. Regarding the value of SFO, it is agreed in RF session [7] in last meeting that 10% is assumed to define the minimum requirements.

Issue 3-2-1: SFO requirement Agreement: - 10% SFO is assumed as the minimum performance for devices. - FFS on whether to define SFO requirement and how to test the SFO requirement if defined.

Based on analysis above, the timing error could be defined as follows:

For MSG1 X=1, The reference point for the device transmission timing is Toffset1 after the end of corresponding R2D transmission as define in TBD (i.e. PHY spec). The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of MSG1.

Proposal 2: Define timing error requirements for D2R as follows:

For MSG1 X=1, The reference point for the device transmission timing is Toffset1 after the end of corresponding R2D transmission as define in TS 38.219. The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of MSG1.

Note 1: The timing error requirements for other D2R could be updated accordingly based on further RAN1 agreements.

Note 2: The end of the R2D transmission and last edge could be updated based on further RAN1 agreements if any.

# Conclusions

Proposal 1: RAN4 to include following aspects for RRM requirements for RA. The details of UE behaviour to be updated based on RAN2 MAC spec.

- Contention-Based Random Access procedure

  - Transmission of Random ID message

  - Reception of Random ID Response message

- Contention-Free Random Access procedure

  - Transmission of first D2R message

Observation 1: Based on latest RAN1 progress, device determines the start time of MSG1 as:

- For MSG 1 X=1, the starting time of first Msg1 based on Toffset1. The starting time for the first Msg1 time domain resource is Toffset1 after the end of the corresponding R2D transmission triggering random access.

- For MSG1 X=2, the starting time of Second Msg1 based on Toffset1 and Toffset2. The starting time for the second Msg1 time domain resource is Toffset1 + Toffset2 after the end of the corresponding R2D transmission triggering random access.

- The value of Toffset1 and Toffset2, and how to define the end of R2D transmission will be defined in RAN1.

Proposal 2: Define timing error requirements for D2R as follows:

For MSG1 X=1, The reference point for the device transmission timing is Toffset1 after the end of corresponding R2D transmission as define in TS 38.219. The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of MSG1.

Note 1: The timing error requirements for other D2R could be updated accordingly based on further RAN1 agreements.

Note 2: The end of the R2D transmission and last edge could be updated based on further RAN1 agreements if any.

# References

[1] RP‑234058 New SID: Study on solutions for Ambient IoT (Internet of Things) in NR

[2] RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

[3] RP-250796 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

[4] R4-2504905 WF on RRM requirements for Ambient_IoT_Solutions

[5] R2-2502258 Skeleton of A-IoT MAC specification (TS 38.391)

[6] EPC® Radio-Frequency Identity Generation-2 UHF RFID Standard

[7] R4-2505230 WF on A-IoT device requirements