3GPP TSG-RAN WG4 Meeting #115	R4-2507123

Malta, 19th – 23rd May 2025

Agenda item:	7.23.3.1

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on requirements for RRM measurement prediction on AIML mobility

Document for:	Discussion

- Introduction

The SI was revised at RAN#105[1] and WF was approved at RAN4#114bis [2]. In this contribution, we further discuss the remaining requirements for RRM measurement prediction on AIML mobility.

- Discussion

According to the latest WF [2], the open issues of RRM core requirements are described as follows.

Issue 2-1-1: Relative RSRP accuracy definition Agreement: - For intra-frequency L3-RSRP, - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency, or - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell. - FFS if this definition will be needed. - For inter-frequency L3-RSRP, - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference. - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, [if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2]. - Depending on the agreement on ground truth definition, we may revisit relative RSRP accuracy definition. - FFS: whether the reported RSRP of cell2 can be predicted or measured or a combination of both predicted and measured samples. - FFS, if there is any impact on this definition for different scenarios/different frequency ranges. (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2),

As above stated, one of the apparent issues is about inter-frequency L3-RSRP,

- FFS: whether the reported RSRP of cell2 can be predicted or measured or a combination of both predicted and measured samples.

The controversial point is whether the reported RSRP of cell2 is predicted, measured or a combination of both predicted and measured samples.

Accuracy of predicted relative L3-RSRP consists of the reported RSRP and ground truth of RSRP. If reported RSRP of cell 2 is based on measured value, it will be the same value as ground truth of RSRP, or almost same value if taking measurement error into account. In consequence, the relative RSRP accuracy becomes absolute RSRP accuracy of cell 1. The reported RSRP of cell 2 should be all predicted value to keep relative RSRP accuracy meaningful.

Observation 1: If reported RSRP is based on measured value, it will be the same value as ground truth of RSRP, or almost same value if taking measurement error into account. In consequence, the relative RSRP accuracy becomes absolute RSRP accuracy of cell 1.

Proposal 1: Predicted samples are applied for the reported RSRP of cell 2 to keep relative RSRP accuracy meaningful.

From RAN4 accuracy requirement perspective, the accuracy can be the value between predicted based and measured based if combination of both predicted and measured samples is applied, and now only predicted samples are considered based on above proposal. Thus, there is no need to consider the combination of predicted and measured samples from RAN4 accuracy requirement perspective.

Observation 2: If combination of both predicted and measured samples is applied for reported RSRP of cell2, the requirement value on accuracy of predicted relative L3-RSRP is between the predicted samples and the measured samples.

Proposal 2: No need to consider the combination of predicted and measured samples from RAN4 accuracy requirement perspective.

- Conclusion

In this contribution, we propose RRM requirements for this WI as follows.

Observation 1: If reported RSRP is based on measured value, it will be the same value as ground truth of RSRP, or almost same value if taking measurement error into account. In consequence, the relative RSRP accuracy becomes absolute RSRP accuracy of cell 1.

Observation 2: If combination of both predicted and measured samples is applied for reported RSRP of cell2, the requirement value on accuracy of predicted relative L3-RSRP is between the predicted samples and the measured samples.

Proposal 1: Predicted samples are applied for the reported RSRP of cell 2 to keep relative RSRP accuracy meaningful.

Proposal 2: No need to consider the combination of predicted and measured samples from RAN4 accuracy requirement perspective.

References

[1] 3GPP, RP-242393, “Revised SID on AIML for mobility in NR”, OPPO.

[2] 3GPP, R4-2504901, “WF on AI/ML Mobility”, Nokia.