3GPP TSG-RAN WG4 Meeting #115		                 R4-2507128

Malta, MT, 19th – 23rd May, 2025

Source:	Sony

Title:	RF requirements of ambient IoT device 1

Agenda item:	7.26.3.2

Document for:	Approval

# Background

The RF requirements of Ambient IoT (AIoT) device type 1 were discussed in RAN4#114bis [1], and further agreements have been made in [2]. In this contribution, we share our further views on how the RF requirements shall be specified for device type 1.

# System parameters

## D2R bandwidth

For D2R bandwidth, the following agreements have been made in RAN4#114bis:

- Define D2R channel bandwidth for A-IoT device:

- Based on 2SB transmission,

- Consider SFO impact

- Consider small frequency shift defined in RAN1

- D2R transmission bandwidth (2 times of 1SB), consider bit rate defined in RAN1

Based on the agreement above, the D2R channel bandwidth is illustrated in Fig. 1, and it can be calculated as in (1)

BWD2R=2*ffreq shift+BWtransmission/2                                                  (1)

Therefore, the key issues in determining the D2R channel bandwidth are the values of small frequency shift R and the values of D2R transmission BW.

Figure. 1. Illustration of D2R bandwidth.

The small frequency shift R values are still pending on RAN1 discussion; thus, RAN4 shall wait for the RAN1 conclusion on the allowed value of a small frequency shift. Another factor would be key for the transmission bandwidth is the value of chip duration and the needed guardband, which can be further defined as

BWtransmission= 2× 1R* Tc×120%                                                          (2)

where the 120% coefficient takes into account a ± 10% SFO of the AIoT device 1 based on the outcome of the study phase [3].

Some potential values on the small frequency shift value and chip duration have been discussed in RAN1#120bis, where the values are as below:

When R = 1 With maximum chip duration equals to 133us, which corresponds to a bandwidth of 2/133e-6*1.2 = 18kHz,while the minimum chip duration equals to 0.52 us, which corresponds to a bandwidth of 2/0.52e-6*1.2 = 4.6 MHz. Those values are still pending on RAN1 to finalizedthough, so RAN4 can wait for RAN1 decision before defining the final value of transmission BW.

Observation 1: Based on the latest progress in RAN1, potential values for transmission BW are in the range of between 18 kHz to 4.6 MHz.

Proposal 1: RAN4 waits for RAN1's decision on the small frequency shift value and the chip duration value to determine the transmission bandwidth. However, RAN4 can consider defining a ±10% transmission BW as a guard band to accommodate the SFO.

# RF requirement

## Reception requirements.

### REFSENS

For REFSENS requirements, the following agreements have been made in RAN4#114bis

- Define minimum requirements of REFSENS based on link budget (target is [15m]) and measurement results reported by companies

- Consider to use inventory procedure for REFSENS testing, success rate ([90%]) or miss detection rate ([10%]) as the metric for inventory

- At least to define maximum gain direction sensitivity requirements

- FFS on how to different sensitivity requirements on different orientations

Based on the agreement above, RAN4 takes a different approach from the conventional method to derive the REFSENS requirements, and it is critical to determine the proper parameters of AIoT reader to determine the link budget. Since the RAN4 requirement needs to guarantee the device performance under the worst-case scenario, it is proposed that the link budget to calculate the REFSENS shall be based on the lowest output power and antenna gain from the AIoT reader. Based on the parameters used in LLS in the study phase, the following parameters are used to derive the link budget for REFSENS

Table. 1. Link budget analysis for REFSENS

| Parameters | values |
| --- | --- |
| Reader output power (dBm) | 24 dBm |
| Reader antenna gain (dBi) | 2 dBi |
| Reader cable loss (dB) | 3 dB |
| Distance (m) | 15m |
| Pathloss model | inF-DH NLOS |
| Received power (dBm) | -37 dBm |

Considering some additional margin for unfavorable propagation conditions, it is proposed that the REFSENS requirement of the AIoT devices be defined with -40 dBm TIS. Further, assuming the peak gain of AIoT devices is about 2 dBi (assuming dipole antenna) will result in an EIS level in the peak direction in the range of -42 dBm.

With such a high input power level, the SNR level on the baseband of the R2D link is no longer critical; thus, no LLS simulation would be further required to determine the threshold SNR, and there is no need to scaling the REFSENS requirement for different BW in our view.

Proposal 2: It is proposed that the REFSENS requirement of the AIoT devices with -40 dBm TIS or -42 dBm EIS be defined in the peak direction for all R2D BW.

Still, a performance metric is needed, and it is proposed to adopt a 10% miss detection rate (MDR) as the metric.

Proposal 3: RAN4 adopts 10% MDR as the metric to test REFSENS requirement for AIoT devices.

### Maximum input level

For maximum input level, it was agreed in RAN4#114bis that further discussion is needed. For the RF-ED receiver, the diode typically operates in the square-law region below the linear region (see Fig. 2). So, the input signal can be self-mixed to the DC. Therefore, ensuring the device can still operate properly is important. A typical square law region for a Schottky diode is below -20 dBm, which can be a starting point to be considered as the maximum input level.

Fig. 2. The operating region of a diode.

Proposal 4: The maximum input level of R2D needs to be defined to ensure the RF-ED receiver operates in the square-law region of the diode and that the signal can be self-mixed to the DC.

Proposal 5: Consider a maximum input power level in the range between -20 and -10 dBm as starting point.

## Transmission part (D2R)

### The transmitted power

For transmitted power from the tag on the D2R, it depends on both the tag implementation and the property of the incoming CW signal, and it was agreed in RAN4#114bis that

- Performance metric of transmit output power:

  - Backscattering loss

- Backscattering output power is defined as the minimum backscattered power of Device 1 at certain input CW Tx power

  - Minimum Backscattering power at device antenna= Input CW Tx power at device antenna-Backscattering loss

- For the Input CW Tx power to define requirements:

  - FFS on the whether to use one value or a range of CW Tx power

  - FFS on the value(s) of CW Tx power

According to the agreement, the backscattering loss has been defined as the metric to evaluate the transmit power performance of AIoT in the D2R link, which is equal to the difference between input CW Tx power at the device antenna and the backscattering power at the device antenna.

Physically, the relation between the impinging power and backscattering power is proportional to the radar cross section (RCS) σ of the object, which is defined as:

PsPin ∝σ=limr→∞4πr2SsSi                                                                      (3)

where Ss and Si are the power density of the impinging power and scattering power.

The bistatic RCS σ of an RFID tag antenna can be described by (4) according to [3]

σ Ωi,Ωs= λ24π GΩiGΩsΓ-A 2                                                    (4)

where Ωi and Ωs are the incident angle and scatter angle. G and  Γ stand for the antenna gain and reflection coefficient on the AIoT device load. A stands for residual scattering due to the antenna structure, which is independent of load conditions. We also assume the insertion loss and mismatch loss between the AIoT antenna and load impedance is contained in the G. From (3) and (4), it can be observed that the backscattering loss is angle-dependent, and a simulated RCS with a different incident angle is shown in Fig. 4 as an example.

Fig. 3. The RCS simulation of an AIoT device with different CW incident angles.

However, (4) only provides the RCS of the AIoT device when the load impedance is fixed. With a modulated signal, the AIoT device's load impedance varies in the test, and the power on the first sideband (SB) is proportional to the difference between the modulation state rather than the absolute RCS. Therefore, in [4], a backscattering efficiency μm based on the power level on the first SB are derived:

PsPin = GTΩincGRΩscatterλ2(4π)3r12r22μm

Where μm Ωinc,Ωscatter= λ22π3 GΩincGΩscatterΓ1-Γ2 2                                     (5)

where GT and GR are the gain of CW and reader antenna, and r1 and  r2 are the distance from the CW antenna to the AIoT device and the distance from the AIoT device to the reader, respectively. Such a backscattering efficiency is essentially the RCS on the first SB.

However, please note that this backscattering efficiency cannot be used to derive the backscattering loss directly as it quantifies a link level loss rather than the loss between incoming power level and outgoing power level on the device. To quantify the backscattering loss, and reference AIoT device (e.g., a lossless AIoT device with 0 dBi realized antenna gain.) may need to be define and it requires further study if this backscattering efficiency can be used to derive the backscattering loss directly.

Nevertheless, From (5), it can be observed that the backscatter loss on the AIoT device depends on the following parameters if we assume no polarization mismatch happens during the link.

- the antenna gain on the incident angle

- the antenna gain on the scattered angle

- the difference between the reflection coefficient of the two states, e.g., the modulation depth.

Observation 2: Assuming no polarization mismatch, the backscatter loss on the first SB of the AIoT device depends on the antenna gain on the incident angle, the antenna gain on the scattered angle and the modulation depth.

Taking the meander dipole antenna in fig. 3 as an example: the antenna directivity on the boresight is typically about 2 dBi; the efficiency of the antenna can be in the range of -3 dB in free space. Therefore, the realized antenna gain is in the range of -1 dBi. The amplitude of modulation depth Γ1-Γ2 can be assumed in the range of about 0.9 for OOK modulation and around 1.9 for BPSK modulation.

Proposal 6: RAN4 can consider adopting -1 dBi realized antenna gain, 0.9 and 1.9 reflection coefficient difference for OOK and BPSK as a starting point to derive the backscattering loss of AIoT devices.

Since it is clear that the performance is angle-dependent, the core requirement may need to be verified under multiple incident angles and scattered angles if the measurement is EIRP-based. Another option here can be using TRP-based measurement. In this case, there is no need to choose the scattering angle; only focus on the incident angle.

Proposal 7: RAN4 considers verifying the backscattering loss based on the following options:

Option 1: EIRP-based measurement of one or more pairs of incident angles and scattering angles.

Option 2: TRP-based measurement with one or more incident angles.

Please note that the above discussion ignores the polarization mismatch between the CW and AIoT devices. As a additional aspect, the test setup may need to ensure the CW can be transmitted in both polarizations, and it is possible to define the requirement based on the average backscattering loss between in the two polarizations. IN this case, a 3dB polarization loss needs to be taken into account. We also have a accompanying paper under the OTA agenda [5] on this issue.

Proposal 8: RAN4 needs to take into account a 3 dB polariation mismatch loss if the backscattering loss is measured as average value between two orthogonal polarized CW waves.

### Transmitted signal quality

As proposed in the D2R bandwidth, a guardband to accommodate the 10% SFO would need to be defined. Meanwhile, it is also critical to ensure that all the devices are within the range of 10% SFO so the guardband can be sufficient in all cases.

Proposal 9: 10% SFO requirement should be defined for AIoT devices.

However, such a requirement may not need to be tested explicitly if other tests ensure the device SFO is within this range, such as an out-of-band emission test or an occupied bandwidth test.

Proposal 10: the SFO requirement may not need to be tested explicitly if there are other requirements that can guarantee the SFO performance.

In addition to the frequency error, some requirements are also needed to ensure the modulation quality of the backscattered signal. For BPSK, the traditional EVM requirements could be applied directly. For the OOK signal, the main performance is the modulation depth.

However, as shown in (5), we can see that when we measure the power of SB in backscattering communication, the modulation depth is actually measured already. Therefore, it is not necessary to test the signal's modulation quality further, at least for the OOK signal. For BPSK, RAN can further discuss if any EVM-based requirement shall be defined or if its modulation quality will be sufficiently tested with backscattering loss as well.

Proposal 11: There is no need to define additional modulation quality requirements for OOK since the modulation depth can be verified via a backscattering loss test. Further discussion on BPSK is needed to determine if any EVM-based requirement shall be defined or if it is sufficient with the backscattering loss test.

# Conclusion

In this paper, we provide the RF requirement for AIoT device 1, where the following observations and proposals have been made:

Observation 1: Based on the latest progress in RAN1, potential values for transmission BW are in the range of between 18 kHz to 4.6 MHz.

Observation 2: Assuming no polarization mismatch, the backscatter loss on the first SB of the AIoT device depends on the antenna gain on the incident angle, the antenna gain on the scattered angle and the modulation depth.

Proposal 1: RAN4 waits for RAN1's decision on the small frequency shift value and the chip duration value to determine the transmission bandwidth. However, RAN4 can consider defining a ±10% transmission BW as a guard band to accommodate the SFO.

Proposal 2: It is proposed that the REFSENS requirement of the AIoT devices with -40 dBm TIS or -42 dBm EIS be defined in the peak direction for all R2D BW.

Proposal 3: RAN4 adopts 10% MDR as the metric to test REFSENS requirement for AIoT devices.

Proposal 4: The maximum input level of R2D needs to be defined to ensure the RF-ED receiver operates in the square-law region of the diode and that the signal can be self-mixed to the DC.

Proposal 5: Consider a maximum input power level in the range between -20 and -10 dBm as starting point.

Proposal 6: RAN4 can consider adopting -1 dBi realized antenna gain, 0.9 and 1.9 reflection coefficient difference for OOK and BPSK as a starting point to derive the backscattering loss of AIoT devices.

Proposal 7: RAN4 considers verifying the backscattering loss based on the following options:

Option 1: EIRP-based measurement of one or more pairs of incident angles and scattering angles.

Option 2: TRP-based measurement with one or more incident angles.

Proposal 8: RAN4 needs to take into account a 3 dB polarization mismatch loss if the backscattering loss is measured as average value between two orthogonal polarized CW waves.

Proposal 9: 10% SFO requirement should be defined for AIoT devices.

Proposal 10: the SFO requirement may not need to be tested explicitly if there are other requirements that can guarantee the SFO performance.

Proposal 11: There is no need to define additional modulation quality requirements for OOK since the modulation depth can be verified via a backscattering loss test. Further discussion on BPSK is needed to determine if any EVM-based requirement shall be defined or if it is sufficient with the backscattering loss test.

# References

- R4-2504687	Topic summary for [114bis][134] A-IoT_device 	CMCC

- R4-2505230	WF on requirements for A-IoT device	CMCC

- C. -C. Yen, A. E. Gutierrez, D. Veeramani and D. van der Weide, "Radar Cross-Section Analysis of Backscattering RFID Tags," in IEEE Antennas and Wireless Propagation Letters

- C. Degen and P. Bosselmann, "On Complex Radar Cross Section and Backscatter Modulation Efficiency in RFID Systems," 2020 14th European Conference on Antennas and Propagation (EuCAP).

- R4-2507129	Consideration on the OTA test of ambient IoT device 1	Sony