# 3GPP TSG-RAN WG4 Meeting #115			R4-2507130

# Malta, MT, 19th – 23rd May, 2025

Source:	Sony

Title:	views on requirements of the low-power wake-up receiver

Agenda item:	7.28.2

Document for:	Approval

- Background

The RF requirements of low-power wake-up signal (LP-WUS) and low-power wake-up receiver (LP-WUR) for NR were further discussed in RAN4#114bis [1], and a WF was agreed [2]. In this contribution, we further discuss the REFSENS and other RF requirements of LP-WUR based on our LLS results.

- Applicability of RF requirement

In the RAN4#114 meeting, defining two types of RF requirements for LP-WUR was agreed, with the NF+ IM as below

- Set 1: LP-WUR IM+NF = 18.0 dB

- Set 2: LP-WUR IM+NF = 13.5 dB

Moreover, during RAN4#114bis, it has been further aggreed that:

- RAN4 define two LP-WUR types (Type 1 and Type 2), no need 1-to-1 mapping between waveform and LP-WUR type.

- Requirement applicability should be clearly defined.

- RAN4 need to further discuss how to design the RMC for each type, how to ensure the performance of each type properly.

Set 1 and set 2 NFs are based on the assumption of OOK receiver and OFDM receiver, respectively. An OOK receiver can only decode OOK waveform, while an OFDM receiver can decode both OFDM and OOK waveform, and therefore, it is reasonable that the LP-WUR types are not mapped with waveform in a 1-1 manner. However, to clarify the applicability, one approach from the RAN4 perspective could be based on the capability of decoding different waveforms or RMC, e.g., define LP-WUR type 1 if it only supports an OOK based waveform with more relaxed RF requirements, while LP-WUR type 2 support both OOK and OFDM waveform with more stringent RF requirements.

Proposal 1: RAN4 agrees with the following principle as requirement applicability:  LP-WUR type 1 only supports an OOK-based waveform with more relaxed RF requirements, and  LP-WUR type 2 supports both OOK and OFDM waveform with more stringent RF requirements.

|  | OOK waveform | OFDM waveform |
| --- | --- | --- |
| LP-WUR Type 1 (relaxed requirements) |  |  |
| LP-WUR Type 2 (stringent requirements) |  |  |

Based on the principle mentioned above and to avoid further fragmentation, it would be beneficial if RAN4 could associate the LP-WUR type with RAN1 UE feature groups (FG) in RAN4 specifications. Although the UE FG discussion is still ongoing, based on some discussion so far, RAN4 can consider the associate the RAN4 define LP-WUR type with RAN1 UE FG.

Proposal 2: Pending the RAN1 conclusion, the RAN4-defined LP-WUR type shall be associated with the RAN1-defined UE feature groups to avoid further fragmentation of the LP-WUR implementation. As a starting point, the following map can be considered:

| RAN4 LP-WUR type | RAN1 UE FG for LP-WUS |
| --- | --- |
| Type 1 | 62-1 LP-WUS operation in IDLE/INACTIVE mode based on OOK signal |
| Type 2 | 62-1a LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence |
| Type 1 | 62-2 LP-WUS operation in CONNECTED mode based on OOK signal |
| Type 2 | 62-2a LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence |

- REFSENS for LP-WUR

Based on our simulation results presented in earlier contributions, different receiver implementations may present different processing gains and trade-offs on the detection threshold, so an SNR value between -7 and -3 dB can be considered.

Observation 1	 The target SNR can range from about -7 dB to -3 dB for an OOK-based receiver with an OOK waveform.

Meanwhile, the OFDM receiver that can decode the OFDM overlaid sequence is similar to the legacy OFDM receiver. The legacy OFDM receiver adopts -1 dB as the threshold SNR value. Considering the decoding effort is smaller for LP-WUS, it is reasonable to assume a threshold SNR is below -1 dB. Based on the companies' contribution in RAN4#114bis [1], the OFDM receiver's SNR mostly drops about -4 to -5 dB.

Observation 2	 The target SNR can be in the range of -4 to -5 dB for OFDM-based receivers with OFDM waveform.

However, considering the agreement that there is no need for a 1-to-1 mapping between waveform and LP-WUR type, it is possible to agree on the SNR value in a waveform-agnostic manner and use the same SNR value for all types of receivers. Therefore, from this perspective, it is proposed that -4 dB SNR be adopted for all types of receivers to define the REFSENS requirement.

Proposal 3: RAN4 adopts the SNR = -4 dB for all types of LP-WURs.

- Testability of LP-WUS

The following agreements have been made for testability of LP-WUS in RAN4#114bis:

Issue 4-1-1: Whether LR can be tested directly

Agreements:

- Down select below two approaches for LP-WUR UE RF requirements:

- Option 1: stick to 1% MDR LP-WUS as performance metric, but with a note in TS that LP-WUR ON is not mandated.

  - The note depends on RAN1 conclusion

- Option 2: update it to 1% MDR of PDCCH triggered by LP-WUR

Practically speaking, it is impossible to define what "LP-WUR" is from a hardware perspective. For example, if UE with OFDM receiver decodes the LP-WUS, it can be just the same hardware or partially the same hardware as the MR. Therefore, it is meaningless from the test or the requirement perspective to mandate the "LP-WUR" to be "on" in the test.

Observation 3: It is impossible to define what "LP-WUR" is from the hardware perspective, and thus, it is meaningless from the test or the requirement perspective to mandate the "LP-WUR" to be "on" in the test.

Moreover, the RAN4 requirement is just to guarantee that the UE can wake up correctly when the network sends an LP-WUS. How UE does it and which hardware UE uses is irrelevant to system performance and should be up to implementation. As long as UE can meet the MDR requirements, it is sufficient from a system performance perspective and thus sufficient for RAN4 core requirements. Therefore, we don't see the point to clarify it in the core requirement.

Observation 4: The RAN4 requirement is to guarantee that the UE can wake up correctly when the network sends an LP-WUS. How UE actually does it is not relevant to the system's performance.

Proposal 4: Stick to 1% MDR LP-WUS as the performance metric but without any additional note.

# Conclusion

In this contribution, we have discussed LP-WUS and LP-WUR for NR. The following observations and proposals have been made:

Observation 1: The target SNR can range from about -7 dB to -3 dB for an OOK-based receiver with an OOK waveform.

Observation 2: The target SNR can be in the range of -4 to -5 dB for OFDM-based receivers with OFDM waveform.

Observation 3: It is impossible to define what "LP-WUR" is from the hardware perspective, and thus, it is meaningless from the test or the requirement perspective to mandate the "LP-WUR" to be "on" in the test.

Observation 4: The RAN4 requirement is to guarantee that the UE can wake up correctly when the network sends an LP-WUS. How UE actually does it is not relevant to the system's performance.

Proposal 1: RAN4 agrees with the following principle as requirement applicability:  LP-WUR type 1 only supports an OOK-based waveform with more relaxed RF requirements, and  LP-WUR type 2 supports both OOK and OFDM waveform with more stringent RF requirements.

|  | OOK waveform | OFDM waveform |
| --- | --- | --- |
| LP-WUR Type 1 (relaxed requirements) |  |  |
| LP-WUR Type 2 (stringent requirements) |  |  |

Proposal 2: Pending the RAN1 conclusion, the RAN4-defined LP-WUR type shall be associated with the RAN1-defined UE feature groups to avoid further fragmentation of the LP-WUR implementation. As a starting point, the following map can be considered:

| RAN4 LP-WUR type | RAN1 UE FG for LP-WUS |
| --- | --- |
| Type 1 | 62-1 LP-WUS operation in IDLE/INACTIVE mode based on OOK signal |
| Type 2 | 62-1a LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence |
| Type 1 | 62-2 LP-WUS operation in CONNECTED mode based on OOK signal |
| Type 2 | 62-2a LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence |

Proposal 3: RAN4 adopts the SNR = -4 dB for all types of LP-WURs.

Proposal 4: Stick to 1% MDR LP-WUS as the performance metric but without any additional note.

- References

R4-2504689		Topic summary for [114bis][134] NR_LPWUS_UERF	Moderator(Vivo)

R4-2505234		WF on UE RF requirements for LP-WUS	Vivo