3GPP TSG-RAN WG4 Meeting #115						       R4-2507132

Malta, MT, 19th – 23rd May, 2025

Source:	Sony

Title:	OCC scheme on the RF requirements for IoT NTN

Agenda item:	7.32.3

Document for:	Discussion

# Background

In RAN4#113, the impact of OCC scheme on IoT NTN RF requirements has been further discussed [1], and in particular, it has been agreed [2]:

#### Issue 2-1: Is it feasible to introduce support of TDM DMRS  as per the description in R1-2501512?

- Agreement:

  - It is feasible for the UE to maintain the phase continuity across the blank DMRS symbol in TDM DMRS scheme.

  - Further discuss potential need for requirements if TDM DMRS would be specified (e.g., the measurement time for frequency error requirement).

Issue 2-2: Whether NW would need to use CFO grouping or resynchronization schemes to avoid frequency inaccuracy between a pair of OCC UEs and whether this will result in UE RF specification impacts.

•	Agreement:

o	FFS CFO group requirements for UE and gNB.

o	Other methods to ensure the CFO grouping performance are not precluded if time allowed.

In this contribution, we provide our views on the issues discussed above.

# UE RF impact to support TDM DMRS

During RAN4#114bis, it has been concluded that there is no feasibility issue from the RAN4 side [2]. However, one particular concern was raised for the UE CFO estimation over the slots with blanked DMRS. Since the DMRS in 2 consecutive slots will be blanked now with TDM DMRS, it is unclear if the network can still estimate the UE phase accurately even if there is no phase jump from the UE side. In particular, the large gap (6 ms) in between slot 2 and slot 5 as in Fig. 1 can cause a phase wrap around issue as the worst-case phase error will go beyond the 360 degree (e.g., 6ms*0.1ppm*2GHz*360 degree = 432 degree).

However, per our knowledge, this issue has been studied in RAN1, and contributions from companies in RAN1 [5] have explained that the wrap-around issue over the blanked DMRS slots can be resolved by using the information of consecutive slots with DMRS. Actually, the TDM DMRS pattern has been specifically designed to allow the SAN receiver to handle the phase wrapping issue in RAN1, with two consecutive slots with DMRS and then another consecutive slots without. The consecutive DMRS in the TDM DMRS pattern are used for coarse frequency estimation. The DRMS that are far apart are used for fine frequency estimation.

Observation 1: the TDM DMRS pattern agreed in RAN1 has been specifically designed to allow the SAN receiver to handle the phase wrapping issue, and it can be resolved by combining with estimation from consecutive slots with DMRS.

Another proposal from companies in RAN4 is to extend the current measurement duration of frequency error from 2ms to a longer duration, e.g., 6-8ms. However, it is unclear if there is any benefit to doing so. Although the phase variation can accumulate over time, the actual frequency error in terms of ppm is usually fairly stable unless there is some significant temperature change or voltage change over time. In addition, a shorter measurement duration would be better for capturing any instant change of frequency error than a prolonged one, which is actually more critical for the network to estimate the CFO over the slot without DMRS in our understanding. Therefore, we don’t think increasing the measurement duration of frequency error is helpful.

Observation 2: Extending the measurement duration of frequency error may not be beneficial to support CFO estimation over blanked DMRS

Therefore, based on the understanding above, we think it is not necessary to define new RF requirements to support TDM DMRS.

Proposal 1: it is not necessary to define new RF requirements, considering the phase wrapping issue can be handled via SAN receiver.

# UE RF impact to support CFO grouping

Regarding the CFO grouping, an LS on NR NTN OCC was sent from RAN1[3], which indicates that phase continuity would be required to support CFO grouping with OCC-2 and OCC-4. Although the LS was not intended for IoT NTN, at least for slot OCC2 with SCS = 15 kHz, we expect to see a similar impact on CFO grouping as for NR NTN.

Without phase continuity, it will be difficult to ensure the phase change between grouped UEs is within the tolerance range, especially it is possible that the phase of the paired UE may drift towards opposite directions. Therefore, it is proposed that the phase continuity requirements for CFO grouping be defined. In our analysis for NR NTN, we showed that the current phase continuity requirement with 25 degrees between slot p-1 and slot p would be sufficient [4].

The DMRS bundling requirements defined for NR NTN can be taken as a starting point, where 25-degree phase difference between slot p and slot p+1 can be used. Meanwhile, the event that may potentially break the phase continuity shall also be defined.

Proposal 2: Take the DMRS bundling requirement for NR NTN as a starting point to define the phase continuity requirements for CFO grouping of IoT NTN with slot level OCC.

On the other hand, if the UE is allowed to resync with the network from time to time, it can minimize the relative error between the paired UEs to avoid the large and opposite frequency error without requiring any advanced estimation algorithm from the network side. In this case, there is no need to define any RF requirements as the large and opposite phase changes can be avoided via the network mechanism.

Proposal 3: As an alternative, RAN4 can consider allowing the UE to resync with the network from time to time to avoid large and opposite frequency errors without requiring any additional RF requirement for CFO grouping.

# Conclusion

In this contribution, we make the following observations and conclusions:

Observation 1: the TDM DMRS pattern agreed in RAN1 has been specifically designed to allow the SAN receiver to handle the phase wrapping issue, and it can be resolved by combining with estimation from consecutive slots with DMRS.

Observation 2: Extending the measurement duration of frequency error may not be beneficial to support CFO estimation over blanked DMRS

Proposal 1: it is not necessary to define new RF requirements, considering the phase wrapping issue can be handled via SAN receiver.

Proposal 2: Take the DMRS bundling requirement for NR NTN as a starting point to define the phase continuity requirements for CFO grouping of IoT NTN with slot level OCC.

Proposal 3: As an alternative, RAN4 can consider allowing the UE to resync with the network from time to time to avoid large and opposite frequency errors without requiring any additional RF requirement for CFO grouping.

# References

- R4-2504640	Topic summary for [114bis][315] IoT_NTN_Ph3​ 	MediaTek

- R4-2504717	Way Forward for [114bis][315] IoT_NTN_Ph3 	MediaTek

- R4-2505305 	LS on requirements for the phase continuity and power consistency for OCC with PUSCH in NR NTN Ph3

- R4-2507131	Discussion on the phase continuity and GNSS issue for NR NTN