3GPP TSG-RAN WG4 Meeting # 115	R4-2507166

# Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | <CR#> | rev | <Rev#> | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.101-5 for section 6.1 and 6.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, China Telecom, OPPO, Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IoT_NTN_req_test_enh-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce NR-NTN HPUE RF requirements to the TS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To lift the limitation of PC3; - To amend MOP requirements and tolerance for PC2/1.5/1; - To amend MPR requirements for PC2/1.5/1 - To amend power configuration requirements for PC2/1.5/1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Tx requirement for NTN HPUE is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1, 6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

6	Conducted transmitter characteristics

6.1	General

Unless otherwise stated, the transmitter characteristics for satellite access UEs are specified at the antenna connector of the UE with a single or multiple transmit antenna(s). For UE with integral antenna only, a reference antenna with a gain of 0 dBi is assumed.

All requirements in this section are applicable to devices supporting GSO and/or NGSO satellites.

6.2	Transmitter power

6.2.1	UE maximum output power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.1-1: UE Power Class

| NR satellite band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n256 | 31 | +2/-3 | 293 | +2/-3 | 26 | +2/-3 | 23 | ±2 |
| n255 | 31 | +2/-3 | 293 | +2/-3 | 26 | +2/-3 | 23 | ±2 |
| n254 |  |  |  |  |  |  | 23 | ±2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Power class 3 is default power class unless otherwise stated. NOTE 3: Achived via 2Tx in this version of the specification. NOTE 4: Generally, PC1 and PC1.5 UE are not targeted for smartphone form factor in this version of the specification. |  |  |  |  |  |  |  |  |

The UE shall meet the following additional requirements for maximum mean transmission power density specified in Table 6.2.1-2 when NS is signaled and when the configured channel overlaps with any portion of the specified frequency range.

Table 6.2.1-2: Additional requirements for transmit power density

| NR Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum power density |
| --- | --- | --- | --- | --- |
| n254 | NS_04N | 5 | 1610 - 1618.25 | 27dBm/4kHz (mean) 15dBm/4kHz (peak limit) |
|  | NS_05N | 5 | 1618.25 - 1626.5 |  |
|  |  | 10, 15 | 1610 – 1626.5 |  |

6.2.2	UE maximum output power reduction

UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations. For UE power class 2 and 3, the allowed maximum power reduction (MPR) is defined as Table 6.2.2-2, Table 6.2.2-1 in 3GPP TS 38.101-1[5] clause 6.2.2 respectively, except for 256QAM. For UE power class 1.5 with 2Tx and power class 2 with 2Tx, the allowed maximum power reduction (MPR) is defined as Table 6.2D.2-3 and Table 6.2D.2-1 in 3GPP TS 38.101-1[5] clause 6.2D.2 respecitvely, except for 256QAM. For UE power class 1, the allowed maximum power reduction (MPR) is defined in Table 6.2.2-1.

[Editor’s note: Place holder for UE power class 1 MPR.]

Table 6.2.2-1 Maximum power reduction (MPR) for power class 1 with 1Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK w/ Rel-15 DMRS |  |  |  |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS |  |  |  |
|  | QPSK |  |  |  |
|  | 16 QAM |  |  |  |
|  | 64 QAM |  |  |  |
| CP-OFDM | QPSK |  |  |  |
|  | 16 QAM |  |  |  |
|  | 64 QAM |  |  |  |
| NOTE: |  |  |  |  |

6.2.3	UE additional maximum output power reduction

[Editor’s note: To be amended if additional A-MPR values/NS signaling is required.]

6.2.3.1	General

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR satellite band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [8].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1-1 except for 256QAM. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2.3 is defined in 3GPP TS 38.101-1 [5] clause 6.2.2 except for 256QAM. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

Table 6.2.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. The mapping of NR satellite band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.1-1A.

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR satellite Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2.2-1 (NOTE 3) | 5, 10, 15, 20 | Table 5.3.2-1 | N/A |
| NS_24 | 6.5.3.3.13 in 3GPP TS 38.101-1 [5] | n256 | 5, 10, 15, 20 | Table 6.2.3.15-1 in 3GPP TS 38.101-1 [5] | Clause 6.2.3.15 in 3GPP TS 38.101-1 [5]2 |
| NS_02N | 6.5.3.3.2 | n255 | 5, 10, 15, 20 |  | N/A |
| NS_100 | 6.5.2.4.2 in 3GPP TS 38.101-1 [5] | n2561 |  |  | Table 6.2.3.1-2 in 3GPP TS 38.101-1 [5] |
| NS_03N | 6.5.3.3.3 | n254 | 5, 10, 15 |  | Clause 6.2.3.2 |
| NS_04N | 6.5.2.3.1 6.5.3.3.4 | n254 | 5 |  | Clause 6.2.3.3 |
| NS_05N | 6.5.2.3.2 6.5.3.3.4 | n254 | 5, 10, 15 |  | Clause 6.2.3.4 |
| NOTE 1: This NS can be signalled for NR satellite bands that have UTRA services deployed. NOTE 2: A-MPR for the upper 5 MHz of the band is not specified, and therefore shall be used as a guard band. NOTE 3: The NS_01 label with the field additionalPmax [8] absent is default for all NR satellite bands. |  |  |  |  |  |

Table 6.2.3.1-1A: Mapping of network signalling label

| NR satellite band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n256 | NS_01 | NS_24 | NS_100 |  |  |  |  |  |
| n255 | NS_01 | NS_02N |  |  |  |  |  |  |
| n254 | NS_01 | NS_03N | NS_04N | NS_05N |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of 3GPP TS 38.331 [8]. |  |  |  |  |  |  |  |  |

6.2.3.2	A-MPR for NS_03N

Table 6.2.3.2-1: A-MPR regions for NS_03N

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 1612.5 <= fc < 1613.9 | <= 0.36 | <= 0.36 | A1 |
|  |  |  | >= 2.88 | A2 |
|  | 1613.9 <= fc < 1615.7 |  | >= 3.24 | A3 |
| 10MHz | 1615 <= fc < 1620.1 | <= 1.8 | <= 5.04 | A4 |
|  |  | <= 1.8 | > 5.04 | A5 |
|  |  | > 7.2 | > 0 | A6 |
|  |  | > 1.8 | >= 2.88 | A2 |
|  | 1620.1 <= fc < 1621.5 |  | <= 6.48 | A6 |
|  |  | <= 0.36 | <= 0.36 | A1 |
|  | fc = 1621.5 |  | >= 7.2 | A1 |
| 15MHz | all | <= 3.6 | <= 5.04 | A4 |
|  |  | <= 3.6 | > 5.04 | A5 |
|  |  | > 10.44 |  | A6 |
|  |  | > 3.6 | >= 4.32 | A2 |

Table 6.2.3.2-2: A-MPR for NS_03N

|  | Modulation | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 2.5 | 3.0 | 1.0 | 4.0 | 6.5 | 1.5 |
|  | QPSK | 2.5 | 4.0 | 2.5 | 6.0 | 7.0 | 2.0 |
|  | 16QAM | 3.0 | 4.5 | 3.0 | 6.5 | 7.5 | 2.5 |
|  | 64QAM | 3.5 | 5 | 3.5 | 7 | 8 | 3 |
| CP-OFDM | QPSK | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |
|  | 16QAM | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |
|  | 64QAM | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |

6.2.3.3	A-MPR for NS_04N

Table 6.2.3.3-1: A-MPR regions for NS_04N

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 1612.5 <= fc < 1613.9 | <= 0.36 | <= 0.36 | A1 |
|  |  |  | >= 2.88 | A2 |
|  | 1613.9 <= fc < 1615.7 |  | >= 3.24 | A3 |

Table 6.2.3.3-2: A-MPR for NS_04N

|  | Modulation | A1 | A2 | A3 |
| --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 2.5 | 3.0 | 1.0 |
|  | QPSK | 2.5 | 4.0 | 2.5 |
|  | 16QAM | 3.0 | 4.5 | 3.0 |
|  | 64QAM | 3.5 | 5 | 3.5 |
| CP-OFDM | QPSK | 3.5 | 6.0 | 4.0 |
|  | 16QAM | 3.5 | 6.0 | 4.0 |
|  | 64QAM | 3.5 | 6.0 | 4.0 |

6.2.3.4	A-MPR for NS_05N

Table 6.2.3.4-1: A-MPR regions for NS_05N

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 1622.4 < fc <= 1624 | >= 3.6 | > 0 | A3 |
|  |  |  | >= 2.88 | A1 |
| 10MHz | 1615 <= fc < 1620.1 | <= 1.8 | <= 5.04 | A4 |
|  |  | <= 1.8 | > 5.04 | A5 |
|  |  | > 7.2 | > 0 | A6 |
|  |  | > 1.8 | >= 2.88 | A2 |
|  | 1620.1 <= fc <= 1621.5 |  | <= 7.2 | A6 |
|  |  | <= 0.36 | <= 0.36 | A1 |
|  |  | > 7.2 | > 0 | A6 |
| 15MHz | all | <= 3.6 | <= 5.04 | A4 |
|  |  | <= 3.6 | > 5.04 | A5 |
|  |  | > 10.44 |  | A6 |
|  |  | > 3.6 | >= 4.32 | A2 |

Table 6.2.3.4-2: A-MPR for NS_05N

|  | Modulation | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 1.5 | 5.0 | 1.5 | 6.5 | 6.5 | 2.0 |
|  | QPSK | 1.5 | 5.0 | 1.5 | 6.5 | 7.0 | 2.5 |
|  | 16QAM | 2.0 | 5.5 | 2.0 | 7.0 | 7.5 | 3.0 |
|  | 64QAM | 2.5 | 6 | 2.5 | 7.5 | 8 | 3.5 |
| CP-OFDM | QPSK | 3.0 | 6.5 |  | 8.0 | 10.0 | 4.5 |
|  | 16QAM | 3.0 | 6.5 |  | 8.0 | 10.0 | 4.5 |
|  | 64QAM | 3.0 | 6.5 |  | 8.0 | 10.0 | 4.5 |

6.2.4	Configured transmitted power

[Editor’s Note: Following changes are proposed to the current equation from TS 38.101-1

- the ∆TC,c, ΔPPowerClass, ΔPPowerBoost, ∆TIB,c have not been introduced to NTN;

- PPowerClass, MPRc and A-MPRc should refer to 38.101-5 other than 38.101-1.

- ∆TRxSRS, NTN by default are verified with 2Rx ports in chapter 7]

The

UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass – ΔPPowerClass + ΔPPowerBoost}

where

PEMAX,c is the value given by either the p-Max IE or the field additionalPmax of the NR-NS-PmaxList IE, whichever is applicable according to TS 38.331[7];

PPowerClass is the maximum UE power specified in Table 6.2.1-1 without taking into account the tolerance specified in the Table 6.2.1-1;

MPRc and A-MPRc for serving cell c are specified in subclause 6.2.2 and subclause 6.2.3, respectively;

∆TRxSRS is applied when UE transmits SRS to other than first SRS port when the SRS-TxSwitch capability is indicated as  '1T2R'.

For other SRS transmissions ∆TRxSRS is zero;

P-MPRc is the allowed maximum output power reduction for

a)	ensuring compliance with applicable electromagnetic energy absorption requirements and addressing unwanted emissions / self desense requirements in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications;

b)	ensuring compliance with applicable electromagnetic energy absorption requirements in case of proximity detection is used to address such requirements that require a lower maximum output power.

The UE shall apply P-MPRc for serving cell c only for the above cases. For UE conducted conformance testing P-MPRc shall be 0 dB

NOTE 1:	P-MPRc was introduced in the PCMAX,f,c equation such that the UE can report to the gNB the available maximum output transmit power. This information can be used by the gNB for scheduling decisions.

NOTE 2:	P-MPRc may impact the maximum uplink performance for the selected UL transmission path.

The reference and evaluation periods for Pcmax, TREF and Teval, are specified in Table 6.2.4-1 in 3GPP TS 38.101-1[5] clause 6.2.4. The measured configured maximum output power PUMAX,f,c and its tolerance T(PCMAX,f,c) are specified in 3GPP TS 38.101-1[5] clause 6.2.4.

<End of change>