3GPP TSG-RAN WG4 Meeting # 115	R4-2507170

# Malta, MT, 19th – 23rd May, 2025

Title:	Discussion on Ku-band SAN RF requirements

Source:	Samsung

Agenda item:	7.11.6

Document for:	Approval

1	Introduction

In last RAN4 #114 meeting, the WF had agreed on the work split, and in 114bis meeting, the WF[1] captures progress in both SAN and VSAT RF requirements for Ku-band.

In this paper, we will continue discussing the SAN and VSAT RF requirements.

2	SAN RF requirements

In this section, we will focus on discussing the OTA out-of-band blocking and OTA in-channel selectivity requirements for Ku-band NTN SAN. And in last meeting WF[1], the agreements related to these requirements are as shown below:

- Sub-topic 2-1: General issues for Ku band SAN RF requirements Issue 2-1-1: General issues for Ku band SAN RF requirements Agreement: - FFS on whether to introduce a new SAN type for FR1-NTN Ku bands which is based on SAN type 2-O RF requirements.

Given it’s still ‘FFS’ on whether to introduce a new SAN type for Ku-band with FR1-NTN numerology based on type 2-O, we would first go through the specific requirements first and then come back to this ‘general’ issue again in the end.

OTA out-of-band blocking:

For OTA out-of-band blocking, we need to discuss and agree on the Ku-band operating band characteristics and Δfoob. According to the WF[2] in #114 meeting, the NTN Ku-band are defined as follows:

### Sub-topic 1-3: Ku bands definition Table 5.2.3-1: Satellite operating bands in FR2-NTN Satellite operating band; Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high; Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high; Duplex mode. n509; 14000 MHz - 14500 MHz; 10700 MHz – 127504 MHz; FDD. n508; 13750 MHz - 14000 MHz; 10700 MHz – 127504 MHz; FDD. NOTE 4: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT... Table 5.2.3-2: Satellite operating bands in FR1-NTN Satellite operating band; Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high; Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high; Duplex mode. n248; 14000 MHz - 14500 MHz; 10700 MHz – 127501 MHz; FDD. n247; 13750 MHz - 14000 MHz; 10700 MHz – 127501 MHz; FDD. NOTE 1: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT...

And in the current TS 38.108, the Δfoob is defined as follows:

For 1-O: Table 10.6.2.1-2: ΔfOOB offset for satellite operating bands SAN type; Operating band characteristics; ΔfOOB (MHz). SAN type 1-O; FUL,high – FUL,low < 100 MHz; 20. For 2-O: Table 10.6.3.1-2: ΔfOOB offset for satellite operating bands SAN type; Operating band characteristics; ΔfOOB (MHz). SAN type 2-O; FUL,high – FUL,low ≤ 4000 MHz; 1500.

In current spec, the 2-O operating band characteristics were defined to accommodate the NTN Ka-band, while the 1-O were considering FR1-NTN bands. The FUL,high – FUL,low of n509/n508 or n248/n247 are 500 MHz and 250 MHz respectively, which does not fit into any of these current ranges.

Observation 1: The existing Δfoob and its corresponding operating band characteristics does not fit the NTN Ku-band.

From the TR 38.922[3], the base station in similar frequency range 14.8~15.35GHz to Ku-band has been studied and agreed with the following:

6.3.2.3 Blocking response The out-of-band blocking requirement applies from 30 MHz to 2nd harmonic of the upper frequency edge of the operating band, excluding the in-band blocking frequency range, but including the downlink frequency range in case of an FDD operating band as listed in Table 6.3.2.3-1. It is agreed to adopt ΔfOOB = 100 MHz for AAS BS. Table 6.3.2.3-1: OTA out-of-band blocking performance requirement for 14800 – 15350 MHz operation Frequency range of interfering signal (MHz); Wanted signal mean power (dBm); Interferer RMS field-strength (V/m); Type of interfering signal. 30 to 12750; EISREFSENS + 6 dB; 0.36; CW. 12750 to FUL,low – 100; EISREFSENS + 6 dB; 0.1; CW. FUL,high + 100 to 2nd harmonic of the upper frequency edge of the operating band; EISREFSENS + 6 dB; 0.1; CW.

Observation 2: Δfoob had been studied and agreed as 100MHz for AAS BS and 60MHz for non-AAS BS in TR 38.922[3].

Observation 3: In TS 38.104[4], the range 100 MHz ≤ FUL,high – FUL,low ≤ 900 MHz seems be able to cover the Ku-bands.

Considering the Observation 1~3 above, we propose the meeting to consider the operating band characteristics for NTN Ku-band as 100 MHz ≤ FUL,high – FUL,low ≤ 900 MHz and Δfoob as 100MHz.

Proposal 1: To consider the operating band characteristics for NTN Ku-band as 100 MHz ≤ FUL,high – FUL,low ≤ 900 MHz and Δfoob as 100MHz.

And if above proposal 1 is agreed, the proposed changes to the TS 38.108 are shown below and also captured in the draft CR in our separate tdoc.

Table 10.6.2.1-2: ΔfOOB offset for satellite operating bands SAN type; Operating band characteristics; ΔfOOB (MHz). SAN type 1-O; FUL,high – FUL,low < 100 MHz; 20. SAN type 1-O; [100 MHz ≤ FUL,high – FUL,low ≤ 900 MHz]; [100]. Table 10.6.3.1-2: ΔfOOB offset for satellite operating bands SAN type; Operating band characteristics; ΔfOOB (MHz). SAN type 2-O; [900 MHz <] FUL,high – FUL,low ≤ 4000 MHz; 1500. SAN type 2-O; 100 MHz ≤ FUL,high – FUL,low ≤ 900 MHz; [100].

OTA in-channel selectivity:

The ICS requirements for 1-O and 2-O in existing TS 38.108 are different in structure. While the refsens and ICS for 1-O are specified in exact numbers for LEO and GEO, and for different channel bandwidth separately, the refsens and ICS for 2-O are specified in a declaration based manner.

Below is the current 2-O refsens requirements from TS 38.108:

EISREFSENS levels are derived from a single declared basis level EISREFSENS_50M, which is based on a reference measurement channel with 50 MHz SAN channel bandwidth. EISREFSENS_50M itself is not a requirement and although it is based on a reference measurement channel with 50 MHz SAN channel bandwidth it does not imply that SAN has to support 50 MHz SAN channel bandwidth. For GEO class SAN, EISREFSENS_50M is an integer value in the range [-140] to [-149] dBm. The specific value is declared by the vendor. For LEO class SAN, EISREFSENS_50M is an integer value in the range [-120] to [-129] dBm. The specific value is declared by the vendor. Table 10.3.3-1: FR2-NTN OTA reference sensitivity requirement SAN channel Bandwidth (MHz); Sub-carrier spacing (kHz); Reference measurement channel; OTA reference sensitivity level, EISREFSENS (dBm). 50, 100, 200; 60; G-FR2-NTN-A1-1; EISREFSENS_50M + ΔFR2_REFSENS. 50; 120; G-FR2-NTN-A1-2; EISREFSENS_50M + ΔFR2_REFSENS. 100, 200, 400; 120; G-FR2-NTN-A1-3; EISREFSENS_50M + 3 + ΔFR2_REFSENS. NOTE 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. NOTE 2: The declared EISREFSENS_50M shall be within the range specified above..

In the WF[5], the SAN channel bandwidth were agreed as follows:

Given the EISrefsens for Ku-band SAN has not been discussed yet, we propose meeting to first decide whether to follow the Ka-band NTN 2-O logic to have the  EISrefsens in a range and declared by vendor.

Observation 4: For ICS requirements, the meeting need to decide whether to have the EISrefsens as declaration based same as existing NTN 2-O SAN first.

And then, considering the minimum supported SAN channel bandwidth starts from 10MHz, the EISrefsens of 10MHz also need to be defined, so as the ΔKu_REFSENS for Ku-band.

Observation 5: The EISrefsens_10M and ΔKu_REFSENS also need to be defined.

And given the rest configurations, including reference measurement channel for testing 15MHz channel bandwidth, together with its type of interfering signal, especially the RB allocations for wanted and interfering signal should be discussed and agreed.

Observation 6: A new reference measurement channel for Ku-band 15MHz need to be agreed and amended to TS 38.108, so as the RB allocations of wanted and interfering signal of OTA ICS testing.

SAN type for NTN Ku-band:

As discussed in the beginning, in last meeting WF, there is a ‘FFS’ on whether to have a new BS type for Ku-band. From our above observations and also by practicing the CR drafting, we believe it could be helpful to have a dedicated SAN type, at least from OTA out-of-band blocking and OTA in-channel selectivity spec drafting perspective.

Observation 7: From spec drafting perspective, we believe it’s beneficial to have a new/dedicated SAN type other than impacting all the existing 1-O and 2-O requirements.

3	Conclusion

In this contribution, we provide views on SAN RF requirements for Ku bands NTN:

Observation 1: The existing Δfoob and its corresponding operating band characteristics does not fit the NTN Ku-band.

Observation 2: Δfoob had been studied and agreed as 100MHz for AAS BS and 60MHz for non-AAS BS in TR 38.922[3].

Observation 3: In TS 38.104[4], the range 100 MHz ≤ FUL,high – FUL,low ≤ 900 MHz seems be able to cover the Ku-bands.

Proposal 1: To consider the operating band characteristics for NTN Ku-band as 100 MHz ≤ FUL,high – FUL,low ≤ 900 MHz and Δfoob as 100MHz.

Observation 4: For ICS requirements, the meeting need to decide whether to have the EISrefsens as declaration based same as existing NTN 2-O SAN first.

Observation 5: The EISrefsens_10M and ΔKu_REFSENS also need to be defined.

Observation 6: A new reference measurement channel for Ku-band 15MHz need to be agreed and amended to TS 38.108, so as the RB allocations of wanted and interfering signal of OTA ICS testing.

Observation 7: From spec drafting perspective, we believe it’s beneficial to have a new/dedicated SAN type other than impacting all the existing 1-O and 2-O requirements.

4	Reference

[1] R4-2504720, Way Forward for [114bis][314] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL)

[2] R4-2502290, Way Forward for [114][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group)

[3] TR 38.922

[4] TS 38.104

[5] R4-2504761, Way Forward for [114bis][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group)