3GPP TSG-RAN WG4 Meeting # 115						        R4-2507180

Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.26.3.3

Source: 	OPPO

Title: 	On RF requirements for CW

Document for:	Approval

- Introduction

The ambient-IoT topic has been fully discussed during the last RAN4bis meeting. A WF [1] has been agreed and further discussion on the RF requirements for CW is provided in this paper.

- Discussion

During the RAN4#114bis meeting, the agreed WF has been captured as below:

Issue 3-2-1: ON/OFF power

- Agreement:

  - Define CW OFF power

  - Value TBD

Issue 3-4: Phase noise

- Agreement:

  - FFS on whether to define phase noise requirements or leave to implementation in Rel-19 WI.

  - Encourage companies provide phase noise profile

Issue 3-5: Unwanted emission

- Agreement:

  - FFS on unwanted emission boundary and corresponding values:

  - Option 1: The unwanted emission of CW is defined from minimum small frequency shift ± D2R transmission bandwidth/2 to the boundary of D2R channel bandwidth

  - Option 2: FFS whether or not to define channel bandwidth for CW

  - Option 3: The spectrum emission mask of the CW node applies to frequencies (Δf) starting from the assigned transmission frequency. The spectrum emission mask for CW node is defined as in Table 2.2-4.

Table 2.2-4: CW node spectrum emission mask

| Δf (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0-90 | N/A |  |
| 90-270 | -15 | 30 kHz |
| 270-450 | -20 | 30 kHz |
| 450- fmax | -27 | 30 kHz |

  - Other options are not precluded.

Issue 3-6: Spurious emissions

- Agreement:

  - FFS on spurious emissions boundary and corresponding values:

  - Option 1: The spurious emission of CW starts from the boundary of D2R channel bandwidth.

  - Option 2: The boundary between spectrum emission mask domain and spurious emission domain for CW node is specified as ΔF = 10 MHz

  - Other options are not precluded.

Figure 1 example to illustrate phase noise test

For the phase noise measurement of a CW, above figure 1 is an example from the R&S manual. We can see 4 points are measured as 10Hz, 100Hz,1kHz and 5kHz with the dBc/Hz metric to calculate the PSD difference of the power at certain frequency offset and compared to the carrier power.

Observation 1: A typical phase noise test has been already defined as dBc/Hz metric for certain frequency offset.

Then the most important point is how this phase noise will impact the system. During the co-existence study as well as the link level simulation, the CW phase noise interference to reader reception has not been discussed. If we use the above CW measurement as a reference to estimate the interference that goes into the reader RX bandwidth. For simplicity, a 15kHz channel bandwidth of the reader RX which results in 7.5kHz single side band bandwidth.

Figure 2 example of a CW source

Above figure 2 is an example CW source noise figure profile from a user manual found online, some typical points with certain noise figure is captured below using 1GHz data from figure 2.

-77dBc/Hz at 10Hz point

-107dBc/Hz at 100Hz point

-119dBc/Hz at 1000Hz point

-122dBc/Hz at 75000Hz point

The total noise is around -38dBm/15kHz at CW antenna connector. If we consider the 20m distance and the FSPL is around 57.5dB and hence the reader will receive a -102dBm/15kHz interference. Currently the REFSENS of reader is still under discussion however the -95dBm/15kHz is a comparable interference compared to the REFSENS. In such case we believe the phase noise needs to be considered.

Observation 2: A comparable interference of phase noise is observed.

From the above estimation, the largest phase noise is within 1kHz offset for single side. This will for sure be covered by the 15kHz REFSENS requirement. Hence one way to consider the phase noise is to consider the de sense. This will need to be together to be considered with the reader REFSENS requirement. Another way is like the observation 1, to test the phase noise in a typical way as with different frequency offset and define certain dBc impression compared to the carrier output power. In our view, since anyway the REFSENS requirement will be defined consider also the phase noise of CW. In such case, we don’t need to further define the phase noise requirement.

Proposal 1: To define the reader REFSENS requirement consider the CW phase noise and not to define a separate requirement.

For the SEM and spurious emission, since the REFSNES only define 15kHz channel bandwidth hence for single side band, it is propose to start the SEM as 7.5kHz. The largest channel bandwidth of D2R considering the largest frequency shift as 960kHz and the largest transmission bandwidth as 3840kHz with 1920kHz for one side band, the SEM edge is assumed to be 1920kHz.

Proposal 2: The SEM range of CW is from 7.5kHz to 1920kHz.

From figure 2, it can be observed that around -120dBc/Hz is almost flat from 5kHz to 500kHz. From 500kHz to 10MHz there is further roll off. In such case, to define the SEM as -120dBc/Hz from 7.5kHz to 500kHz, and from 500kHz to 1920kHz with -148dBc/Hz. With the CW output power of 33dBm, then the SEM is proposed as -47dBm/10kHz and -75dBm/10kHz. Consider this is only one example of CW source for testing, a 7dB variance to consider different implementation is proposed. Hence in the end it is proposed as -40dBm/10kHz and -68dBm/10kHz.

The spurious emission requirement will be -36dBm/100kHz which is around -46dBm/10kHz. From our understanding, the most impact from the phase noise is for the 15kHz channel bandwidth range and in the SEM range there is at least 10dB difference compared to the frequency range near to CW. In this case, putting too stringent requirement for the SEM range does not help to improve signal quality. Hence we believe for the SEM range, a flat limit as -40dBm/10kHz is reasonable and for the spurious range. -46dBm/10kHz or -36dBm/MHz is agreeable from regulation perspective.

Proposal 3: The SEM requirement is defined as flat -40dBm/10kHz in the SEM range.

Proposal 4: The spurious emission requirement is the same as FR1 NR UE requirement.

3	Conclusions

In this paper, we give discussion on the CW RF requirement, the observations and proposals are as below:

Observation 1: A typical phase noise test has been already defined as dBc/Hz metric for certain frequency offset.

Observation 2: A comparable interference of phase noise is observed.

Proposal 1: To define the reader REFSENS requirement consider the CW phase noise and not to define a separate requirement.

Proposal 2: The SEM range of CW is from 7.5kHz to 1920kHz.

Proposal 3: The SEM requirement is defined as flat -40dBm/10kHz in the SEM range.

Proposal 4: The spurious emission requirement is the same as FR1 NR UE requirement.

4 References

[1] R4-2505097	WF on requirements for A-IoT BS and CW, Moderator (Huawei)