# 3GPP TSG-RAN WG4 Meeting #115                                 R4-2507185

# St.Julian’s, Malta, 19th – 23th May, 2025

Source:  ZTE Corporation, Sanechips

Title:   Discussion on the RRM requirements aspects of AI/ML positioning

Agenda Item:	7.21.5

Document for:	Approval

- Introduction

In RAN plenary #94 meeting, a new SID[1] for Rel-18 was approved to study the AI/ML for NR air interface. The objectives for RAN4 of this SID are listed as follows.

- Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

In this document, we will provide some views on the interoperability and testability for AI positioning from the general test framework perspective.

- Discussion

In latest WID, following three cases of AI/ML based positioning accuracy enhancement are agreed for further study：

- Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

Since RAN plenary has already precluded case2a and case2b, RAN4 shall not define the related requirements on them. In further meetings, RAN4 shall focus on case 1, case 3a and case 3b.

Observation 1: RAN plenary has already precluded case2a and case2b.

Proposal 1：RAN4 shall not define any RRM requirements on case 2a and case 2b.

And in previous meeting, RAN4 has reached the related agreements for AI positioning as below:

RAN4 110 meeting: Issue 3-2: Requirements for case 3a/3b - RAN4 will not define positioning accuracy requirements for case 3a/3b RAN4 110bis meeting: Issue 3-1: Requirements for case 1 Agreement: postpone discussion until reporting scheme (if defined) is clear. if reporting scheme is introduced, RAN4 will further discuss whether to define requirements or not. RAN4 will not define any accuracy requirements if no reporting scheme is introduced RAN4 112bis meeting: Issue 3-1: Requirements for case 1 Agreement: Companies are invited to bring further analysis on feasibility of defining requirements for case 1 and how to define requirements, if feasible Issue 3-3: Report applicability for existing reported metrics Agreement: Extend applicability of existing report mappings for UL-RToA and UL SRS-RSRPP to report UL measurements for AI/ML based positioning use cases 3a/3b. RAN4 114 meeting: Issue 3-4: Report mapping for UL SRS-RSRP Agreement: - Reuse the report mapping for UL SRS-RSRP to Case 3a For non-AI, according to TS 38.133, measurement reporting mapping are specified for UL RTOA, UL SRS RSRP measurement and UL SRS RSRPP measurement, which can be reused for case 3a. RAN4 114bis meeting: Issue 3-2: LOS/NLOS indicator Agreement: the LOS/NLOS indicator is out of scope of WID. Stop discussions on it. Issue 3-3: Rx-Tx time difference for case 3a Agreement: - Introduce gNB Rx-Tx time difference reporting accuracy requirements - Reuse legacy accuracy requirements Issue 3-4: Report mapping for gNB Rx-Tx time difference Agreement: - Reuse the existing report mapping for gNB Rx-Tx time difference measurements

In the previous meeting, RAN4 kicked off the discussion on case 1 and reached the agreement as below:

Issue 3-1: Requirements for case 1 Agreement: postpone discussion until reporting scheme (if defined) is clear. If reporting scheme is introduced, RAN4 will further discuss whether to define requirements or not. RAN4 will not define any accuracy requirements if no reporting scheme is introduced

First of all, the following represent case 1:

Figure 1 Case 1

In case 1 the AI/ML model is at UE side, that is, model inference is at UE side which is up to UE implementation. For direct UE-based positioning, the model output provides the predicted/estimated UE location. Case 1 is similar to the legacy UE-based positioning and no requirements are defined for such scenario. In previous meeting, we discussed whether to define the related accuracy requirements for case 1 and the final agreements shows if reporting scheme is introduced, RAN4 will further discuss whether to define requirements or not. That is, the main discussion is what is the reporting scheme for case 1. We can review the legacy reporting scheme firstly. In legacy, there are two kinds of location request: UE-triggered location request and NW-triggered location request. Some companies deem that there is no need to define accuracy requirements for case 1 since the UE triggers the location request and it is no need to report the location to NW which is up to UE implementation. If the location request is triggered by the network, the UE shall report its location. To my understanding, the above agreement contains the implicit reporting scheme which is the NW triggers the location request, otherwise no reporting scheme is needed.

Observation 2: In legacy, there are two kinds of location request: UE-triggered location request and NW-triggered location request. The latest agreement contains the implicit location request which is the NW triggers the location request, otherwise no reporting scheme is needed.

Proposal 2: If RAN1 has the agreement on the reporting scheme can use the legacy location request, there are two options here:

Option 1: If it is NW-triggered location request, UE shall report its location and the report requirements shall be confirmed.

Option 2: If it is UE-triggered location request, it is no need to report the location to NW which is up to UE implementation.

Case 1 is the direct AI/ML positioning, UE-based positioning with UE-side model. UE measures the downlink PRS and the related measurement results can be the inference input. Then, the inference will be directly performed at UE side, the inference output is the AI/ML predicted UE location. After inference, the UE will report the model output to LMF. Thus, the test metric is the positioning accuracy. For test equipment, one main challenge is the accuracy of UE-based direct positioning. In several past meetings, RAN4 did not discuss the ground truth for AI/ML positioning. If the ground truth is known to test equipment, the accuracy can be quantified compared to defined ground truth, such as AI/ML positioning error. Then the model performance can be evaluated and the subsequent operation of model can be clarified. However, how to study or define the positioning ground truth and how to test case 1 are two key points.

Observation 3: If the ground truth is known to test equipment, the accuracy can be quantified compared to defined ground truth, such as AI/ML positioning error. Then the model performance can be evaluated and the subsequent operation of model can be clarified.

In previous meetings, we proposed several times for ground truth on AI/ML positioning which is PRU (Positioning reference unit). If the UE is the PRU, then the LMF/gNB has already known its location. To my understanding, the AI/ML predicted location can be compared to PRU and then the positioning accuracy error will be known. The UE location is the coordinate and the PRU will be set on anywhere, if we set PRU randomly, we do not think the accuracy can be accurately evaluated. Thus, several companies proposed the test map granularity and test map can be defined as below [3]:

Figure 5 Grid map providing a ground-truth information for inference evaluation

While UE moves from one location point to another location, the inference result with AI/ML functionality at each location is reported to the test equipment and compared to the ground-truth information for verification of positioning accuracy.

Also, we deem that if UEs have the GNSS capability, they also can be the reference UEs. Based on TR 38.171, the GNSS error are as below:

| Scenario Type | Weak satellite signal conditions | Ideal conditions | Satellites with rather different signal levels | Multi-path Scenario | Moving scenario with periodic update |
| --- | --- | --- | --- | --- | --- |
| Position Error | 100 m | 15 m | 100 m | 100 m | 50 m |

However, if we choose the reference GNSS UEs, the GNSS error shall be considered when evaluating the AI/ML positioning accuracy error. This kind of ground truth can be discussed.

Observation 4: The UE location is the coordinate and the PRU will be set on anywhere, if we set PRU randomly, we do not think the accuracy can be accurately evaluated.

Observation 5: If UEs have the GNSS capability, they also can be the reference UEs. However, if we choose the reference GNSS UEs, the GNSS error shall be considered when evaluating the AI/ML positioning accuracy error.

Proposal 3: The ground truth can be the PRU and the test map shall be considered. Reference GNSS UEs shall be considered.

- Conclusion

In this contribution, we have the following observations and proposals for the AI/ML positioning:

Observation 1: RAN plenary has already precluded case2a and case2b.

Proposal 1：RAN4 shall not define any RRM requirements on case 2a and case 2b.

Observation 2: In legacy, there are two kinds of location request: UE-triggered location request and NW-triggered location request. The latest agreement contains the implicit location request which is the NW triggers the location request, otherwise no reporting scheme is needed.

Proposal 2: If RAN1 has the agreement on the reporting scheme can use the legacy location request, there are two options here:

Option 1: If it is NW-triggered location request, UE shall report its location and the report requirements shall be confirmed.

Option 2: If it is UE-triggered location request, it is no need to report the location to NW which is up to UE implementation.

Observation 3: If the ground truth is known to test equipment, the accuracy can be quantified compared to defined ground truth, such as AI/ML positioning error. Then the model performance can be evaluated and the subsequent operation of model can be clarified.

Observation 4: The UE location is the coordinate and the PRU will be set on anywhere, if we set PRU randomly, we do not think the accuracy can be accurately evaluated.

Observation 5: If UEs have the GNSS capability, they also can be the reference UEs. However, if we choose the reference GNSS UEs, the GNSS error shall be considered when evaluating the AI/ML positioning accuracy error.

Proposal 3: The ground truth can be the PRU and the test map shall be considered. Reference GNSS UEs shall be considered.

- Reference

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, 3GPP RAN Plenary

- R4-2310433, “WF on RAN4 requirements for NR AI/ML”, Qualcomm Incorporated

- R4-2505105, “WF on requirements for AI/ML air interface normative work”