# 3GPP TSG-RAN WG4 Meeting #115                                  R4-2507191

# St.Julian’s, Malta, 19th – 23th May, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM performance requirements for less than 5MHz in NTN enhancement

Agenda item:	7.10.4.5

Document for:	Approval

Introduction

RRM core requirements for Rel-19 NTN enhancements are discussed in past meetings. Since the scope of the core requirements is stable enough, it is time for RAN4 to start working on the performance part of the WI. We can see the main task on RRM performance part for less than 5MHz NTN as follows:

Less than 5MHz for NTN - Specify the necessary conformance requirements for SAN for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary RRM performance requirements and test cases for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary demodulation performance requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

In this paper we will provide our views on RRM performance requirements on less than 5MHz NTN.

Discussion

In last meeting, we have already confirmed the measurement accuracy requirements and common testing configuration. For test cases, there are still some remaining issues to be discussed. In this paper, we would like to provide the views from our side.

Issue 2-2-2 Test cases to verify the core requirements reusing NTN

The agreements reached in last meeting could be seen as below:

Agreement: - Not define the new test cases for the following requirements RRM requirements; NTN requirements in TS38.133 v18.5.0. RRC connection mobility control; 6.2C RRC Connection Mobility Control for Satellite Access. UE Tx timing, MTTD/ MRTD, timer accuracy, TA accuracy; 7.1C, 7.2C, 7.3C. - Further discuss whether to define the new test case for the following requirement IDLE/inactive mode mobility; 4.2C Cell Re-selection for NR UE for Satellite Access.

To our understanding, we shall confirm the test case for cell re-selection for less than 5M NTN. First of all, we have one agreement in core part as follows:

Agreement: No core part impact. Discussion: However, regarding there is no any exact Tsi-nr given in TS38.133 only the testing parameter will be given.

Although there is no core part impact on cell re-selection requirements, regarding there is no exact Tsi-nr given in current spec, only the testing parameter will be given. Thus, we deem that the testing parameter shall be defined. In legacy TN less than 5M WI, the reason to define new test case for cell re-selection is that UE needs to read SIB/MIB from the target cell, which is same in HO. RAN4 did not define new core requirements because the exact delay for UE to read SIB/MIB is included in Tsi but the exact value is not specified. In the test, Tsi is defined as 1280ms as below:

Legacy TN test requirements for Tsi: TSI-NR Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

Since the legacy delay as 1280ms is not enough to accommodate longer delay for SIB/MIB reading due to punctured PBCH, thus, the legacy test requirements on less than 5M TN can be seen as below:

Legacy TN with less than 5M for Tsi: TSI-NR Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1320 ms is assumed in this test case.

We observe that the exact value in test requirements for legacy NTN on Tsi is as below:

Legacy TN test requirements for Tsi: TSI-NR Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case provided that SIB1 and SIB19 are scheduled with 20 ms period and 80 ms period, respectively.

The same logic shall be considered when defining cell re-selection test requirements with less than 5M in NTN scenario.

Observation 1: The legacy test requirements without less than 5MHz in TN scenario, the legacy delay as 1280ms is not enough to accommodate longer delay for SIB/MIB reading due to punctured PBCH.

Observation 2: The legacy test requirements with less than 5MHz in TN scenario, TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1320 ms is assumed in this test case.

Proposal 1: RAN4 shall define the test case for cell re-selection requirements in NTN scenario with less than 5MHz.

Proposal 2: The concrete test requirements could be seen as below:

TSI-NR Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1320 ms is assumed in this test case provided that SIB1 and SIB19 are scheduled with 20 ms period and 80 ms period, respectively.

Issue 2-2-3 Test cases to verify the new core requirements

The following agreement could be seen:

Agreement: - Define the new test cases for the core requirements for NTN less than 5MHz introduced in Rel19.e.g. - HO-1: Intra-frequency HO, [unknown cell] - CHO-1: [Inter-frequency CHO], time based with L3 measurement - SatSwitch-1: Hard switch - RLM-1: RLM OOS for 15RB - RLM-2: RLM IS for 15RB - BFR-1: BFR - MR-1: Intra-frequency with SSB index reading - [MR-2: Inter-frequency without SSB index reading] - FFS the detailed configuration for each test case

For test cases on handover in NTN scenario, we observe that intra-/inter- frequency measurements with known cell are tested. We deem that HO-1 is the legacy test case in TH with less than 5MHz which can be reused, however, the inter-frequency handover with unknown cell shall be considered. To our understanding, we would like to supplement the primary cases including intra-frequency handover with unknown cell and inter-frequency handover with unknown cell in NTN scenario.

Consequently, for handover, the following test cases shall be considered:

HO-1: Intra-frequency HO including RACH-based HO, unknown cell HO-2: Inter-frequency HO including RACH-based HO, unknown cell

Yet there are two kinds of handover including RACH-based handover and RACH-less handover, we suggest not to test every case since we know that the Tsearch and Tdelta is the same defined in core requirements. Thus, we propose to choose one of them to define the related test cases. The test configuration can be seen as below:

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, 15 kHz SSB SCS, 3 MHz BW |
| 2 | NGSO, NR FDD, 15 kHz SSB SCS, 3 MHz BW |
| NOTE: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. |  |

Observation 3: For test cases on handover in NTN scenario, intra-/inter- frequency measurements with known cell are tested.

Observation 4: The Tsearch and Tdelta is same between RACH-based and RACH-less handover.

Proposal 3: For handover in NTN scenario, the following test cases shall be considered:

HO-1: Intra-frequency HO including RACH-based HO, unknown cell HO-2: Inter-frequency HO including RACH-based HO, unknown cell

For conditional handover in NTN scenario, we observe that test cases are defined for time based CHO with L3 measurement, distance based CHO with L3 measurement and time based CHO without L3 measurement for intra-/inter-frequency. we propose to choose the time-based CHO with L3 measurement for intra-frequency and distance-based CHO with L3 measurement inter-frequency for less than 5MHz to cover all cases.

Proposal 4: For CHO in NTN scenario, the following test cases shall be considered:

CHO-1: Intra-frequency CHO , time based with L3 measurement CHO-2: Inter-frequency CHO , distance based with L3 measurement

For satellite switch in NTN, test cases are defined for hard satellite switch and soft satellite switch. In legacy one UE capability has been defined as below:

softSatelliteSwitchResyncNTN-r18 Indicates whether UE supports soft satellite switch with re-sync, as specified in TS 38.331 [9]. A UE supporting this feature shall also indicate support of hardSatelliteSwitchResyncNTN-r18.

Based on above UE capability, UE supporting soft switch will also support hard satellite switch. Consequently, we prefer to consider soft satellite switch which can cover hard satellite switch.

Proposal 5: For satellite switch, the following test cases shall be considered:

SatSwitch-1: Soft satellite switch

For RLM, only 15PRB is considered. We prefer to consider RLM OOS and IS with DRX for 15PRB, since this case could cover the case without DRX configuration.

For BFR, we could follow legacy test cases.

Proposal 6: For RLM and BFR, the following test cases shall be considered:

RLM-1: RLM OOS with DRX for 15PRB RLM-2: RLM IS with DRX for 15PRB BFR without DRX

Proposal 7: For measurement, the legacy test cases could be used as follows:

Intra-frequency L3 measurement with SSB index reading without DRX Inter-frequency L3 measurement without SSB index reading with DRX

Proposal 8: The test configuration can be seen as below:

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, 15 kHz SSB SCS, 3 MHz BW |
| 2 | NGSO, NR FDD, 15 kHz SSB SCS, 3 MHz BW |
| NOTE: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. |  |

Conclusion

In this paper we provided our views on RRM performance requirements on R19 NR less than 5M NTN enhancement.

Observation 1: The legacy test requirements without less than 5MHz in TN scenario, the legacy delay as 1280ms is not enough to accommodate longer delay for SIB/MIB reading due to punctured PBCH.

Observation 2: The legacy test requirements with less than 5MHz in TN scenario, TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1320 ms is assumed in this test case.

Proposal 1: RAN4 shall define the test case for cell re-selection requirements in NTN scenario with less than 5MHz.

Proposal 2: The concrete test requirements could be seen as below:

TSI-NR Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1320 ms is assumed in this test case provided that SIB1 and SIB19 are scheduled with 20 ms period and 80 ms period, respectively.

Observation 3: For test cases on handover in NTN scenario, intra-/inter- frequency measurements with known cell are tested.

Observation 4: The Tsearch and Tdelta is same between RACH-based and RACH-less handover.

Proposal 3: For handover in NTN scenario, the following test cases shall be considered:

HO-1: Intra-frequency HO including RACH-based HO, unknown cell HO-2: Inter-frequency HO including RACH-based HO, unknown cell

Proposal 4: For CHO in NTN scenario, the following test cases shall be considered:

CHO-1: Intra-frequency CHO , time based with L3 measurement CHO-2: Inter-frequency CHO , distance based with L3 measurement

Proposal 5: For satellite switch, the following test cases shall be considered:

SatSwitch-1: Soft satellite switch

Proposal 6: For RLM and BFR, the following test cases shall be considered:

RLM-1: RLM OOS with DRX for 15PRB RLM-2: RLM IS with DRX for 15PRB BFR without DRX

Proposal 7: For measurement, the legacy test cases could be used as follows:

Intra-frequency L3 measurement with SSB index reading without DRX Inter-frequency L3 measurement without SSB index reading with DRX

Proposal 8: The test configuration can be seen as below:

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, 15 kHz SSB SCS, 3 MHz BW |
| 2 | NGSO, NR FDD, 15 kHz SSB SCS, 3 MHz BW |
| NOTE: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. |  |

- 4 References

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN.

- TS38.133