3GPP TSG-RAN WG4 Meeting #115	R4-2507223

St Julian’s, Malta, May 19th –May 23rd, 2025

Source:	Nokia

Title:	Discussion on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR

Agenda item:

Document for:	Discussion

Introduction

In the last RAN4 meeting, the following issues were left open.

## Sub-topic 2-2 Performance requirements of Fast SCell activation for UE supporting Rel-18 EMR ### Issue 2-2-1: Scenarios of test cases Further discuss this issue. ### Issue 2-2-2: Details of test case design Further discuss this issue.

Discussion

Test scenarios

### Issue 2-2-1: Scenarios of test cases

From the last meetings contributions, it was quite clear that the majority of the companies at least support defining the test cases for 1. Normal SCell activation, 2. Direct SCell activation at SCell addition and 3. PUCCH SCell activation. Hence we can define test case for those scenarios:

Define test cases for 1. Normal SCell activation, 2. Direct SCell activation at SCell addition and 3. PUCCH SCell activation

Regarding the frequency ranges, both FR1 and FR2 should be supported.

Define test cases for both FR1 and FR2

For the configuration and capabilities, we assume that no new capabilities need to  be defined for the Rel-19 core part so RAN4 can define scenarios based on rel-18 capabilities.

Define test case for scenario: UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured

Define test case for scenario: UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured

Define test case for scenario: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

Conclusion

The following Observations and Proposals were made:

Proposal 1: Define test cases for 1. Normal SCell activation, 2. Direct SCell activation at SCell addition and 3. PUCCH SCell activation

Proposal 2: Define test cases for both FR1 and FR2

Proposal 3: Define test case for scenario: UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured

Proposal 4: Define test case for scenario: UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured

Proposal 5: Define test case for scenario: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode