3GPP TSG-RAN Working Group 4 Meeting #115	 R4-2507232

St Julien, Malta, 19th – 23rd May 2025

Source:	Ericsson

Title:	Discussion on the new capabilities UE RF enhancements in Phase 4

Agenda item:	7.1.2

Document for:	Discussion

# Background

The WID on RF enhancements for NR FR1/FR2 and EN-DC, phase 4, was created for Release 19. The WID covers three main areas: High power UE (HPUE) for CA in terrestrial network (TN), Power domain enhancement for NR single carrier and NR intra-band UL CA for PC2 and PC3, and 6Rx for handheld and FWA UEs.

In this paper, Ericsson has collected its view on UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, and where we see the addition of new capabilities.

# Power domain enhancement for NR single carrier and NR intra-band UL CA

- New capability for being configured with a wider CBW than the UE CBW and considering the RBs as inner RBs. Improve MPR when configured with narrower bandwidth within a wider carrier.

- Higher layer parameter needed to enable the UE to do the power domain enhancement; allows relaxed mask (SEM) and ACLR within the wider bandwidth. Parameter in the cell configuration.

# 6Rx

## MIMO layer evaluation

- Ability to support 6 layers (LS already sent to RAN2)

  - An LS is sent to RAN2 to add support of 6 Layers (R4-2505225)

- No new signalling is needed if UE is handheld or FWA

  - From the network perspective, gNB does not necessarily need to know the UE type but the UE capabilities to manage resource, enable features, etc.

  - The applicability rule could apply to 6 layers for FWA, as for the 8Rx case.

- Having one or two test cases is being discussed for performance part.

## SRS

### SRS antenna switching requirements

- No RAN4 capability. t3r6 needs to be added by RAN1/2

### SRS insertion loss

- Capability for the support of Type 3 power headroom reporting with a new MAC-CE (per SRS resource PHR) if it is introduced, or capability to indicate whether the UE performs SRS IL self-compensation (possibly not a capability; reported depending on power level).