3GPP TSG-RAN WG4 Meeting #115                                        R4-2507249

Malta, MT, 19th – 23rd May, 2025

Source:	Samsung

Title:              Discussion on RRM requirements for LB CA switching

Agenda item:			7.7.4

Document for:	Discussion

1. Introduction

In last meeting, RAN4 had some initial discussions on RRM requirements for LB CA switching. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

2 RAN1 agreement

#120b From Wednesday session Agreement - For Rel-19 low NR band carrier aggregation via switching, support only semi-static configuration of switching pattern to a UE based on UE-specific RRC configuration between the following two cases, - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - FDD carrier 1 is PCell and SDL carrier 2 is SCell. - SCS 15KHz on both carriers. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, it is not expected that the same slot (1ms) contains both symbol(s) configured as FDD carrier and symbol(s) configured as SDL carrier. Agreement For Rel-19 low NR band carrier aggregation via switching, the semi-static switching pattern and corresponding switching gap are based on the downlink timing (DL reception time at UE side) of Pcell. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the time unit (resolution) of the semi-static switching pattern configuration is slot (1ms for 15kHz SCS). From Thursday session Agreement - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support bitmap design - NW configures a periodicity, denoted as P slots, and a bitmap of P bits. Each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2. - The pattern starts from the beginning of SFN 0 of PCell. - Restriction of maximum X switch(es) within Y slot(s). - FFS: detailed value and relation of X and Y. - FFS: other restriction. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap, - For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier. - For switching from FDD carrier to SDL carrier, down-select one of the following solutions: - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier. - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier. - Alt 3: Switching gap is always assumed at the SDL carrier. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, semi-static switching pattern is periodic with periodicity of P slots (ms), support P= 40 ms. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the duration of the switching gap is RRC configured by the NW, - Switching gap duration can be different for FDD to SDL switch, and SDL to FDD switch. - Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed). - For the UL TA, - Switching gap for SDL to FDD switch ends at the end of slot on SDL. - Note: RAN1 assumes that switching gap for SDL to FDD switch includes UL TA. - FFS: whether it has specification impact.

3 Discussion

3.1 SSB assumptions for R19 LB CA via switching

Although it is attractive for the system that if the target SDL SCell would not need to transmit SSB but to transmit temporary RS or A-TRS, supposed to be used for SCell measurement and SCell activation under the deployment constraints: sync. co-located FR1 single TAG. As exemplified in Rel-18 NES discussion, the SCell activation procedure can be shorten by using temporary RS or A-TRS. Under such context, in theory, the PCell can be switched back quickly, the sync maintenance and UEs’ QoS requirements won’t be impacted significantly if temporary RS or A-TRS resources based SDL SCell operation is considered. However, considering 1. SSB-less/Rel-18 NES inter-band SSB-less SCell operation is an optional feature, and temporary RS/A-TRS is not always available for the SDL SCell; 2. Based on RAN1 40 ms periodicity bitmap design, the SSB availability on both carriers for a given switching pattern is to be accounted and guaranteed. In this sense, we have the following proposal:

Proposal 1: RAN4 to use SSB based operation as baseline for Rel-19 low band CA via switching

3.2 Necessity of interruption requirement in RRM between FDD CC and SDL CC

As per RAN1 principle guidance,

[Closed][R2] Proposal 2-1 For Rel-19 low NR band carrier aggregation via switching, during a switching gap, a UE is not expected to either transmit or receive on the FDD carrier or receive on the SDL carrier.

- For switching from SDL carrier to FDD carrier: the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier.

Fig. 1 Illustration of SDL to FDD switch

From our understanding, there are two directions to understand “…, a UE is not expected to ….” and handle the above DL OFDM symbols-switching gap collision issue from RAN4 requirement perspective.

- DL interruption requirement due to switching between two low bands. And consequent questions are: 1.  Whether the interruption requirements at Tx switching including the location and the length can be used as the reference for DL switching related discussion; 2. Whether the DL interruption is always to be allowed. To be specific, the actual UE action (e.g., RF returning) causing interruptions can be done in X symbols, and the whole symbols to be interrupted have to be dropped by UE

- DL scheduling restriction requirement. The NW should not schedule UE with the unavailable resources, or in other words, it should be avoided by the NW/is not allowed for the NW to schedule the unavailable resources on SDL SCell that are collided with the switching gap. But 1) The scheduling restriction requirement is restrictive for NW scheduling. 2) Actually, the “scheduling restriction” we discussed here is different from what we defined in Section 9, since it is related to the switching gap and should contain the resources that does not expect to be scheduled considered for Rel-19 low band CA via switching.

- In RAN1, there are two views: 1) With the acceptable understanding that once the switching pattern is configured, the NW would know well on the switching gap duration and the location, and then the smart NW is able to avoid scheduling resources that collided with switching gap to minimize the impact performance; 2) No NW restriction. And to be specific: -The UE does not receive or transmit a semi-static signal/channel on a serving cell if the semi-static signal/channel overlaps with a non-operation period of the serving cell; -The UE does not expect to be scheduled with a transmission/reception on a serving cell by a DCI format and the transmission/reception overlaps with a non-operation period of the serving cell.

Considering the same issue is discussed in RAN1 in parallel, and no conclusion yet, to avoid duplicated discussions, we suggest to

Proposal 2: Whether to/How to define DL interruption/scheduling restriction requirement for Rel-19 low band CA via switching from SDL carrier to FDD carrier case in RAN4 RRM can wait for more progress in RAN1

- For switching from FDD carrier to SDL carrier.

In RAN1, there are two main Alternatives to define the location, every Alt has its own pros and cons, majority view supported that switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier, the main benefit is to help avoid the impact on PDCCH monitoring in the first few symbols on SCell. While inevitably, it would impact the UL transmissions on PCell as well as UCI multiplexing procedure

Observation 1: The assumption of switching gap is always at the “switch from” carrier, i.e., FDD carrier would impact on UL transmissions on PCell as well as UCI multiplexing procedure.

Whilst, the main reason to support switching gap is always assumed at the SDL carrier is that the impact on UL transmission can be avoided and PDCCH reception on SCell can even be avoided if symbol level switching is decided.

Since the how to define the location of switching gap in RAN1 would bring direct impact to interruption/scheduling restriction requirement definition, suggest to

Proposal 3: Whether to/How to define interruption requirement in RRM between FDD CC and SDL CC for Rel-19 low band CA via switching from FDD carrier to SDL carrier case can wait for more progress in RAN1

3.3 Activated SDL SCell L3 measurement

- Skip activated SDL SCell L3 measurement with feasibility conditions

The SSB is assumed to be used as baseline, based on the deployment assumption that co-located and synchronized inter-band CA in a single TAG, and with the MRTD assumption that the maximum receive timing difference for co-located FDD PCell and SDL SCell CA via switching is 3us. From our understanding, if the maximum power difference between the FDD PCell and SDL SCell is within certain range, e.g., [6]dB, that can means the difference on receive power (RSRP difference) could be quite small of the two Cells, and single AGC setting could be accounted, accordingly. In this sense, the AGC gain for FDD PCell can be reused for the SDL SCell, and AGC setting can be skipped for SCell. Besides, since similar channel qualities could be assumed based on < 1GHz consideration, L3 measurement on SDL SCell is not necessary

No doubt that, if the RTD between the two Cells is within a certain range, e.g., <= 3us, the timing can be borrowed directly from the FDD PCell for sync..

At the same time, since protecting SSB on PCell is essential at least for maintaining UL transmit timing requirements (R4-2502697) from switching pattern design point of view, and the SSB pattern configurations are fixed in the spec. , the impact on AGC gain settling/timing acquisition for FDD PCell should be rather limited ideally.

- UE still needs L3 measurement requirement

If feasibility conditions are not met, the L3 measurement requirement is still needed. For instance, the AGC gain settling based on the FDD PCell RS can’t exactly reflect the received signal power from SDL SCell due to the large RSRP difference, AGC for activated SDL SCell is still needed. However, we have the following concerns if to define the L3 requirements:

- K1 ms. i.e., maximum continuous time duration for SDL restriction

Fig. 1 Illustration of HARQ-ACK feedback

Since UCI can only be transmitted on FDD PCell, to guarantee the QoS requirements and the availability of PDSCH-HARQ feedback opportunity, the maximum continuous dwell time UE stay on the SDL SCell should not be larger than the maximum of configured K1 value, while how to define K1 value is still suspending in RAN1.

Back to SSB resources based AGC setting and L3 measurement, and take Lmax=4 and case A into consideration,

- Worst case: SSBs used to adjust AGC is not contained in the dwell time, UE cannot wait on SSB transmissions to set the AGC for the SDL SCell (e.g., the value of K1 is 1 ms)

- Only one SSB sample can be used to perform AGC/L3 measurement per switching (e.g., the value of K1 is 8 ms), how long the next available SSB used to perform fine AGC adjustment (if needed) or L3 measurement on the SDL SCell can be waited depends on RAN1 discussions on restriction of maximum X switch(es) within Y slot(s)

  - As one of the possible AGC implementations, the old AGC state can be simply restored until the next AGC measurement opportunity, but the old AGC state should be indispensable.

Observation 2: How the available SSB resources can be used to perform AGC adjustment and L3 measurement for the activated SCell depends on following restrictions of switching pattern discussed in RAN1

- Restriction of K1 ms

- Restriction of maximum X switch(es) within Y slot(s)

- Mis-alignment between configurations of switching pattern and UE DRX cycle/MG/SMTC period

At least the following two issues will occur:

- Mis-alignment between cell switching pattern and UE DRX cycle.

  - Case 1: Part of the OFF duration of UE DRX is overlapped with the SDL SCell active period of the switching pattern.

  - Case 2: The ON duration of UE DRX is overlapped with the SDL SCell non-active period of the switching pattern.

- Critical signal/channel transmission periodicity occasions are not end in the SDL SCell active period of the switching pattern when L3 multiple samples measurement is performed

To solve the above mis-alignment issue, at least the following solutions can be found:

- UE to define the measurement requirements in conjunction with switching pattern cycle, DRX cycle, MG and RSs period.

  - New scaling factor should be introduced to reflect the switching pattern cycle

- To specify the priority for SDL SCell switching pattern and UE DRX pattern, and the SCell measurement requirements are defined based on the pattern priority and the RS period

Proposal 4: If to define activated SDL SCell L3 measurement, at least the following aspects should be considered

- Restriction of maximum continuous time duration for SDL, K1 ms

- Restriction of maximum X switch(es) within Y slot(s)

- Mis-alignment between configurations of switching pattern and UE DRX cycle/MG/SMTC period

Proposal 5: If to define activated SDL SCell L3 measurement if SSB based SCell operation is used, two Alts can be considered:

- Alt 1. UE to define the measurement requirements in conjunction with SDL SCell switching pattern cycle, DRX cycle and RSs periods.

  - New scaling factor should be introduced to reflect the switching pattern cycle

- Alt 2. To specify priorities for SDL SCell switching pattern and UE DRX pattern, and the SCell measurement requirements are defined based on the prioritized pattern and the RS period.

Based on comprehensive consideration, since no conclusion on how to define K1 ms, X switch(es) and Y slot(s) in RAN1, and how to define the scaling fator is not such easy, we prefer to

Proposal 6: Prefer to skip activated SDL SCell L3 measurement with following certain side conditions for R19 LB CA via switching:

- Power difference between the FDD PCell and SDL SCell is within certain range, FFS: [6] dB

- RTD between the FDD PCell and SDL SCell is within certain range, e.g., 3 us

3.4 SDL SCell activation/deactivation

From our understanding, one of the main benefits of LB switching is to further improve the system throughput and spectral usage of the SDL carrier. From this, in order to offload UE to the SDL SCell without incurring PCell throughput performance degradation, particularly under high traffic load scenario, it is expected that the identified SDL SCell to be activated within a shorter delay after the UE reports the measurement results. In this sense, known SDL SCell activation is much more promising. Otherwise, it may take by dozens of milliseconds to complete the SDL SCell activation process, and PCell will be effected consequently during such processing time.

Proposal 7: RAN4 to define known SDL Scell activation requirement

- The applicability of the existing known condition and the measurement period of the SCell being activated, i.e., 2400 ms should be discussed

  - Under the side condition: RTD between the FDD PCell and SDL SCell <=3us

3.5 Impact on timing requirement for PCell

Based on RAN1 agreement, P=40 ms for switching pattern is supported. Such switching pattern period does not beyond the maximum SSB periodicity (i.e., 160 ms), so it can be guaranteed that the UE has oppotunity at least every 160 ms to measure available SSB to keep sync and to fulfill UL tramsmit timing requirement. In this sence, no impact on timing requirement for Pcell with the P=40 ms switching pattern periodicity design.

Proposal 8: RAN4 can reuse the existing timing requirement for Pcell with the same applicability condition. i.e., at least one SSB is available at the UE during the last 160 ms with the P=40 ms switching pattern periodicity design

3.6 BFD/CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirements for SDL SCell

From our understanding, L1 measurement on activated SCell is necessary, for example, UE would need to provide L1-RSRP report before the network selects active TCI state. In this sense, we prefer to define L1-RSRP/SINR/BFD/CBD requirements for SDL SCell. Considering the switching pattern is regular, a straightforward way to extend the total measurement period is: times the switching pattern cycle

Proposal 9: Prefer to define SSB based RLM/BFD/CBD/L1-RSRP measurement requirements for SDL SCell

- FFS: The measurement period for SSB based L1-RSRP/L1-SINR/BFD/CBD can be extended by multiplying switching pattern cycle

4 Conclusion

In this contribution, we provided our viewpoints on RRM requirements for LB CA switching. The following observations and proposals are obtained:

Observation 1: The assumption of switching gap is always at the “switch from” carrier, i.e., FDD carrier would impact on UL transmissions on PCell as well as UCI multiplexing procedure.

Observation 2: How the available SSB resources can be used to perform AGC adjustment and L3 measurement for the activated SCell depends on following restrictions of switching pattern discussed in RAN1

- Restriction of K1 ms

- Restriction of maximum X switch(es) within Y slot(s)

Proposal 1: RAN4 to use SSB based operation as baseline for Rel-19 low band CA via switching

Proposal 2: Whether to/How to define DL interruption/scheduling restriction requirement for Rel-19 low band CA via switching from SDL carrier to FDD carrier case in RAN4 RRM can wait for more progress in RAN1

Proposal 3: Whether to/How to define interruption requirement in RRM between FDD CC and SDL CC for Rel-19 low band CA via switching from FDD carrier to SDL carrier case can wait for more progress in RAN1

Proposal 4: If to define activated SDL SCell L3 measurement, at least the following aspects should be considered

- Restriction of maximum continuous time duration for SDL, K1 ms

- Restriction of maximum X switch(es) within Y slot(s)

- Mis-alignment between configurations of switching pattern and UE DRX cycle/MG/SMTC period

Proposal 5: If to define activated SDL SCell L3 measurement if SSB based SCell operation is used, two Alts can be considered:

- Alt 1. UE to define the measurement requirements in conjunction with SDL SCell switching pattern cycle, DRX cycle and RSs periods.

  - New scaling factor should be introduced to reflect the switching pattern cycle

- Alt 2. To specify priorities for SDL SCell switching pattern and UE DRX pattern, and the SCell measurement requirements are defined based on the prioritized pattern and the RS period.

Proposal 6: Prefer to skip activated SDL SCell L3 measurement with following certain side conditions for R19 LB CA via switching:

- Power difference between the FDD PCell and SDL SCell is within certain range, FFS: [6] dB

- RTD between the FDD PCell and SDL SCell is within certain range, e.g., 3 us

Proposal 7: RAN4 to define known SDL Scell activation requirement

- The applicability of the existing known condition and the measurement period of the SCell being activated, i.e., 2400 ms should be discussed

  - Under the side condition: RTD between the FDD PCell and SDL SCell <=3us

Proposal 8: RAN4 can reuse the existing timing requirement for Pcell with the same applicability condition. i.e., at least one SSB is available at the UE during the last 160 ms with the P=40 ms switching pattern periodicity design

Proposal 9: Prefer to define SSB based RLM/BFD/CBD/L1-RSRP measurement requirements for SDL SCell

- FFS: The measurement period for SSB based L1-RSRP/L1-SINR/BFD/CBD can be extended by multiplying switching pattern cycle

5 References

[1] R4-2504895, “WF on RRM requirements for NR_LBCA_Sw”, Apple