3GPP TSG-WG4 Meeting #115	 R4-2507254

# Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on OD-SSB based SCell Activation Delay Requirement for Deactivated SCell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Core |  |  |  |  |  | Date: |  |  | 2025/5/6 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In existing SCell activation requirement for deactivated SCell, all the SSB based UE behavior are related to legacy SSB. In Rel-19 NES WI, OD-SSB is introduced, and for a cell supporting OD-SSB SCell operation, the UE shall perform OD-SSB based SCell activation for deactivated SCell when OD-SSB transmission is triggered, and the corresponding requirements should be specified, accordingly. - OD-SSB based SCell activation known and unknown requirements for both Case 1 and Case 2 are specified - To take NCD-SSB as the RS for OD-SSB based SCell Activation delay requirement for deactivated SCell - The agreements on OD-SSB based SCell activation are captured as below #112-bis R4-2416916 Issue 1-4-1: General Agreement: - RAN4 focuses on OD-SSB based SCell activation known and unknown requirement for both Case 1 and Case 2 and not specifically differentiate Scenario 2/2A scenarios. Issue 1-4-3: known cell condition design – principle Agreement: - The legacy known condition principle can be reused, which is defined based on if the UE has sent a valid measurement report during a ‘period’ before the reception of the SCell activation command. - When OD-SSB transmission is before/together with the SCell activation command(Scenario #2/Scenario #2A), UE follows the principles of legacy known/unknown SCell activation requirement once the known/unknown condition is met. - FFS how known/unknown condition is defined i.e. based on always-on SSB and/or OD-SSB - Other principles are not precluded. Issue 1-4-5: Which type of SSB is used during SCell activation Agreement: - In Case 1, UE is required to always use OD-SSB for Scell activation. - In Case 2, when both OD-SSB and always-on SSB are transmitted, - RAN4 assumes UE to use OD-SSB to perform SCell activation as a baseline - The agreement can be revisited if different with other WG #113 R4-2420098 The known cell condition – measurement reporting Agreement: - In case 1, known/unknown condition should be defined based on only OD-SSB. - In case 2, known/unknown condition should be defined based on either always-on SSB or OD-SSB. - Note 1: whether the always-on SSB and OD-SSB can be combined is discussed in another issue. - Note 2: the agreement is valid at least for the scenario with the same frequency for always-on SSB and OD-SSB. - Note 3: RAN4 to further clarify the relation for the SSB measured for known cell condition and the SSB used in SCell activation procedure. Issue 1-4-5: SCell activation requirement – OD-SSB periodicity Agreement: - In Case 1, the SCell activation delay requirement is defined based on OD-SSB periodicity. - Existing SCell activation delay requirement shall apply based on OD-SSB periodicity. - In Case 2, RAN4 to define the SCell activation delay requirement based on OD-SSB periodicity if the time instance receiving OD-SSB activation indication is no later than the time instance when receiving the SCell activation command. Otherwise, RAN4 to define the requirement based on always-on SSB for SCell activation. - This agreement is valid under the assumption of the same spatial relation for OD-SSB and always-on SSB [with the same SSB index], and this can be revisited based on RAN1 LS reply. - Note: T_firstSSB_max for intra-band co-located scenario is discussed in another issue. #114 R4-2502598 Issue 4-2: L1 measurement after SCell activation Agreement: - RAN4 to support OD-SSB L1 measurement during and after SCell activation. - When non-DRX is configured, the current requirement can be reused with the clarification of TSSB in TL1-RSRP_Measurement_Period_SSB is the SSB periodicity of OD-SSB - RAN4 to support OD-SSB L1 measurement during SCell activation. - The requirement when DRX is configured. - Based on OD-SSB periodicity #114-bis R4-2504906 Issue 1-2-2: Requirement for RAN1 Alt Time-C1 Agreement: - RAN4 to update the agreement of EMP for deactivated SCell measurement and SCell activation to apply to RAN1 Alt Time-C1 scenario. - The EMP is the periodicity of the union of AO-SSB and OD-SSB and equals one of legacy SSB periodicities defined in the specification. - FFS for other procedures. Issue 1-4-2: Additional Processing time for OD-SSB reception - value Agreement UE applies the additional processing timing only in deactivated SCell measurement. - Note: The additional UE processing timing is not applied to SCell activation procedure.. - As per agreed WF R4-2504906, draft CR to introduce OD-SSB based SCell Activation Delay Requirement for Deactivated SCell is provided. |
| Summary of change: |  | Define OD-SSB based SCell Activation Delay Requirement for Deactivated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | OD-SSB based SCell Activation Delay Requirement for Deactivated SCell is not supported by the RRM spec.. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 8.3.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

8.3.X	OD-SSB based SCell Activation Delay Requirement for Deactivated SCell

The requirements in this clause shall apply for UEs configured with at least one downlink SCell in standalone NR carrier aggregation in connected mode. For the SCell, the physical cell identity of the SSB is indicated by od-ssb-physCellId. For the UE indicated by [od-ssb-config], when a first SSB block is within the active DL BWP, a second SSB to be within the configured DL BWP as the first SSB block. If both of the first SSB and the second SSB are provided for SCell activation, the requirements in this clause are applicable provided that following conditions are met:

-	the power of a second SSB indicated by od-ss-PBCH-BlockPower is same as a first SSB indicated by ss-PBCH-BlockPower

-	the physical cell identity of a second SSB indicated by od-ssb-physCellId, is the same as a first SSB indicated by physCellId in ServingCellConfigCommon

-	the SCS configuration of a second SSB indicated by od-ssbSubcarrierSpacing is the same as a first SSB indicated by od-ssbSubcarrierSpacing

-	when a second SSB is configured for SCell activation is explicitly configured and is QCLed with active TCI state for PDCCH or PDSCH, the index of the transmitted SSB indicated by od-ssb-PositionsInBurst is the same of the index of a first SSB

Upon receiving a second SSB activation command in slot n1, the UE expects that the transmission of the SSB parameters provided by od-ssb-config no later than in slot n1+m+3 Nslotsubframe,μ+1 , where:

slot n1+m is a slot indicated for PUCCH transmission with HARQ-ACK information for the PDSCH reception,

Nslotsubframe,μ is a number of slots per subframe for the SCS configuration μ of the PUCCH transmission as defined in [4, TS 38.211]

The delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions.

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where:

THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]

Tactivation_time is the SCell activation delay in milliseconds.

If UE receives a second SSB activation command no later than SCell activation command,

If the SCell is known and belongs to FR1, Tactivation_time is:

-	TFirst OD_SSB + T∆+5 ms, if the measurement period of the SCell being activated is equal to or smaller than [2400 ms].

-	TFirst OD_SSB_MAX + Trs + T∆+5 ms, if the measurement period of the SCell being activated is larger than [2400 ms].

If the SCell is unknown and belongs to FR1, and if one of the following conditions is met

-	 ‘od-ssb-PositionsInBurst’ indicates only a second SSB is being actually transmitted, or

-	 ‘od-ssb-PositionsInBurst’ indicates multiple second SSBs and TCI indication is provided in same MAC PDU with SCell activation,

provided that the side condition Ês/Iot ≥ -2 dB is fulfilled, Tactivation_time is:

-	 TFirst OD_SSB_MAX + 2*Trs + T∆ +5 ms, if the following conditions are met,

-	the SCell is contiguous to an active serving cell in the same band, and

-	its od-ssb-PositionsInBurst is same as the one of contiguous FR1 active serving cell, and

-	its RTD with contiguous FR1 active serving cell is smaller than or equal to 260 ns, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6 dB;

-	  TFirst OD_SSB_MAX + 3*Trs + T∆ + 5 ms, otherwise.

Otherwise, provided that the side condition Ês/Iot ≥ -2 dB is fulfilled, Tactivation_time is:

-	6 ms + TFirst OD_SSB_MAX + 2* Trs  + T∆ + TL1-RSRP, measure, OD-SSB + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2 ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, where for case 2, provided that

[-	the CSI report configuration is associated with both of first SSB and second SSB if the center frequency of a second SSB, provided by od-absoluteFrequencySSB is the same from center frequency of a first SSB and the index of a second SSB, provided by od-ssb-PositionsInBurst is the same from center frequency of a first SSB

-	Otherwise, the CSI report configuration is associated with a second SSB if the center frequency of a second SSB, provided by od-absoluteFrequencySSB to be different from center frequency of a first SSB]

-	3 ms + TFirst OD_SSB_MAX + 2* Trs + T∆ + TL1-RSRP, measure, OD-SSB + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5 ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting, where for case 2, provided that

[-	the CSI report configuration is associated with both of first SSB and second SSB if the center frequency of a second SSB, provided by od-absoluteFrequencySSB is the same from center frequency of a first SSB and the index of a second SSB, provided by od-ssb-PositionsInBurst is the same from center frequency of a first SSB,

-	Otherwise, the CSI report configuration is associated with a second SSB if the center frequency of a second SSB, provided by od-absoluteFrequencySSB to be different from center frequency of a first SSB]

-	Otherwise,

If the SCell is known and belongs to FR1, Tactivation_time is:

-	TFirstSSB+ T∆ + 5 ms, if the measurement period of the SCell being activated is equal to or smaller than 2400 ms.

-	TFirstSSB_MAX + Trs+ T∆ + 5 ms, if the measurement period of the SCell being activated is larger than 2400 ms.

If the SCell is unknown and belongs to FR1, and if one of the following conditions is met

-	 ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or

-	 ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation,

provided that the side condition Ês/Iot ≥ -2 dB is fulfilled, Tactivation_time is:

-	 TFirstSSB_MAX + TSMTC_MAX + Trs + 5 ms, if the following conditions are met,

-	the SCell is contiguous to an active serving cell in the same band, and

-	its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and

-	its SMTC offset is same as the one of contiguous FR1 active serving cell, and

-	its RTD with contiguous FR1 active serving cell is smaller than or equal to 260 ns, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6 dB;

-	TFirstSSB_MAX + TSMTC_MAX + 2*Trs + T∆ + 5 ms, otherwise.

-	However, when the following conditions are fulfilled, no activation requirement will be applied for this unknown SCell:

-	the SCell is contiguous to an active serving cell in the same band, and

-	A single SSB is used in the unknown SCell; or multiple SSBs are used in the SCell and TCI state indication for PDCCH is provided by the same MAC PDU used for SCell activation; and

-	its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and

-	its SMTC offset is same as the one of contiguous FR1 active serving cell

-	its RTD with contiguous FR1 active serving cell is larger than 260 ns, or its reception power difference with contiguous FR1 active serving cell is larger than 6 dB;

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, then Tactivation_time is TFirst OD_SSB + 5ms provided:

-	the SSBs in the serving cell(s) and a second SSB in the SCell fulfil the condition defined in clause 3.6.3, and

[-	the index of transmitted SSB provided by od-ssb-PositionsInBurst for the SCell is the same as that of transmitted SSB for the serving cell

-	the half frame index for the transmission of the a second SSB for the SCell is the same as that of  atransmitted first SSB for the serving cell

-	the center frequency and SFN offset of a second SSB for the SCell are as the same that of a transmitted first SSB for the serving cell

-	a first SSB and a second SS/PBCH block provided by od-ssb-config have the same sptial relation [with the same SS/PBCH block index]

If the SCell being activated belongs to FR2 and if there is no active serving cell on that FR2 band provided that PCell or PSCell is in FR1 or in FR2:

If the target SCell is known to UE and semi-persistent CSI-RS is used for CSI reporting, then Tactivation_time is:

-	3 ms + max(Tuncertainty_MAC + TFineTiming + 2 ms, Tuncertainty_SP), where Tuncertainty_MAC=0 and Tuncertainty_SP=0 if

-	UE receives the SCell activation command, semi-persistent CSI-RS activation command and TCI state activation command at the same time, or

-	Provided this is the initial activation after SCell addition, and

-	TCI-ActivatedConfig is configured for the SCell, and UE receives the SCell activation command and semi-persistent CSI-RS activation command at the same time.

[-	If a second SSB activation is indicated when a first SSB is transmitted, provided that

[-	the CSI report configuration is associated with both of first SSB and second SSB if the center frequency of a second SSB, provided by od-absoluteFrequencySSB is the same from center frequency of a first SSB and the index of a second SSB, provided by od-ssb-PositionsInBurst is the same from center frequency of a first SSB,

-	Otherwise, the CSI report configuration is associated with a second SSB if the center frequency of a second SSB, provided by od-absoluteFrequencySSB to be different from center frequency of a first SSB]

If the target SCell is known to UE and periodic CSI-RS is used for CSI reporting, then Tactivation_time is:

-	max(Tuncertainty_MAC + 5 ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ), where Tuncertainty_MAC=0 if

-	UE receives the SCell activation command and TCI state activation commands at the same time, or

-	Provided this is the initial activation after SCell addition, and

-	UE receives the SCell activation command and TCI-ActivatedConfig is configured for the SCell.

-	For case 2, provided that

-	the CSI report configuration is associated with both of always-on SSB and on-demand SSB if always-on SSB and on-demand SSB have the same center frequency and the same positions in the same SSB burst on-demand SSB,

-	Otherwise, the CSI report configuration is associated with on-demand SSB if always-on SSB and on-demand SSB have the have the different center frequencies

If the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA or if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management, and the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2 dB is fulfilled, then Tactivation_time is:

-	6 ms + TFirst OD_SSB_MAX +15*TSMTC_MAX, OD-SSB + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report  + THARQ + max(Tuncertainty_MAC + TFineTiming + 2 ms, Tuncertainty_SP)

If the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA or if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management, and the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2 dB is fulfilled, then Tactivation_time is:

-	3 ms + TFirst OD_SSB_MAX + 15*TSMTC_MAX, OD-SSB + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5 ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}

-	Otherwise,

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, then Tactivation_time is TFirstSSB+ 5ms provided:

-	the UE is provided with SMTC for the target SCell, and

-	the SSBs in the serving cell(s) and the SSBs in the SCell fulfil the condition defined in clause 3.6.3, and

-	the parameter ssb-PositionsInBurst is same for the serving cell(s) and the SCell, and

-	SSB is in the same half-frame on the SCell and the contiguous FR2 active serving cell

If the SCell being activated belongs to FR2 and if there is no active serving cell on that FR2 band provided that PCell or PSCell is in FR1 or in FR2:

If the target SCell is known to UE and semi-persistent CSI-RS is used for CSI reporting, then Tactivation_time is:

-	3 ms + max(Tuncertainty_MAC + TFineTiming + 2 ms, Tuncertainty_SP), where Tuncertainty_MAC=0 and Tuncertainty_SP=0 if

-	UE receives the SCell activation command, semi-persistent CSI-RS activation command and TCI state activation command at the same time, or

-	Provided this is the initial activation after SCell addition, and

-	TCI-ActivatedConfig is configured for the SCell, and UE receives the SCell activation command and semi-persistent CSI-RS activation command at the same time.

If the target SCell is known to UE and periodic CSI-RS is used for CSI reporting, then Tactivation_time is:

-	max(Tuncertainty_MAC + 5 ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ), where Tuncertainty_MAC=0 if

-	UE receives the SCell activation command and TCI state activation commands at the same time, or

-	Provided this is the initial activation after SCell addition, and

-	UE receives the SCell activation command and TCI-ActivatedConfig is configured for the SCell.

If the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA or if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management, and the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2 dB is fulfilled, then Tactivation_time is:

-	6 ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs  + TL1-RSRP, measure + TL1-RSRP, report  + THARQ + max(Tuncertainty_MAC + TFineTiming + 2 ms, Tuncertainty_SP),

If the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA or if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management, and the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2 dB is fulfilled, then Tactivation_time is:

-	3 ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5 ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}

where,

TSMTC_MAX:

-	In FR1, in case of intra-band contiguous SCell activation or in case of intra-band non-contiguous SCell activation for UE not capable of intraBandNR-CA-non-collocated-r18 or UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided the cell specific reference signals from the active serving cells and the SCells being activated or released are available in the same slot; in case of intra-band non-contiguous SCell activation for UE capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided or in case of inter-band SCell activation, TSMTC_MAX is the SMTC periodicity of SCell being activated.

-	In FR2, in case of intra-band SCell activation, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided that in Rel-15 only support FR2 intra-band CA; in case of FR2 inter-band SCell activation, TSMTC_MAX is the SMTC periodicity of SCell being activated.

-	TSMTC_MAX is bounded to a minimum value of 10 ms.

Trs

-	 It is the SMTC periodicity of the SCell being activated if the UE has been provided with an SMTC configuration for the SCell in SCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the measObjectNRs having the same SSB frequency and subcarrier spacing configured by MN and SN have different SMTC, Trs is the periodicity of one of the SMTC which is up to UE implementation. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement which involves Trs is applied with Trs = 5 ms assuming the SSB transmission periodicity is 5 ms. There are no requirements if the SSB transmission periodicity is not 5 ms.

-	 It is the on-demand periodicity of the SCell being activated if the UE has been provided with an on-demand SSB configuration for the SCell and the on-demand SSB is transmistted

T∆  is the additional time for fine time tracking and acquiring full timing information of the SCell. T∆ = 2* Trs ms for SCell operating with 12 PRB SSB BW. Otherwise, T∆  = 0 ms.

TFirst OD_SSB:

-	is the time to the end of the first complete a second SSB burst indicated by the a second SSB periodicity of the SCell being activated, after slot n + THARQ+3msNR slot length,

TFirst OD_SSB_MAX:

-	is the time to the end of the first complete a second SSB burst indicated by a second SSB periodicity of the SCell being activated, after slot n + THARQ+3msNR slot length, further fulfilling:

-	In FR1, in case of intra-band SCell activation, the occasion when all active serving cells are transmitting always-on SSB bursts and SCells being activated or released are transmitting on-demand SSB bursts in the same slot; in case of inter-band SCell activation, the first occasion when the SCells being activated are transmitting on-demand SSB burst

-	In FR2, the occasion when all active serving cells are transmitting always-on SSB bursts and SCells being activated or released are transmitting on-demand SSB bursts in the same slot

[-	Otherwise, TFirst OD_SSB_MAX= TFirstSSB_MAX for the case 2]

[FFS: TSMTC_MAX, OD-SSB]

TFirstSSB: is the time to the end of the first complete SSB burst indicated by the SMTC, or within 5 ms if SMTC is not configured, after slot n + THARQ+3msNR slot length.

TFirstSSB_MAX: Is the time to the end of the first complete SSB burst indicated by the SMTC, or within 5 ms if SMTC is not configured, after slot n + THARQ+3msNR slot length, further fulfilling:

-	In FR1, in case of intra-band contiguous SCell activation or in case of intra-band non-contiguous SCell activation for UE not capable of intraBandNR-CA-non-collocated-r18 or UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot; in case of intra-band non-contiguous SCell activation for UE capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided or in case of inter-band SCell activation, the first occasion when the SCell being activated is transmitting SSB burst.

-	In FR2, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot.

TFineTiming:

-	 is the time period between UE finish processing the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and the timing of first complete available SSB corresponding to the TCI state, further fulfilling:

[-	the first complete available SSB is the first complete available on-demand SSB when on-demand SSB is provided for SCell activation, and the SCell being activated is transmitting on-denamd SSB burst, or

-	the first complete available SSB is the first complete available either on-demand SSB or always-on SSB, when on-demand SSB and always-on SSB are provided for SCell activation, and the SCell being activated is transmitting on-denamd SSB burst and always-on SSB burst]

[FFS: EMP]

TL1-RSRP, measure is L1-RSRP measurement delay TL1-RSRP_Measurement_Period_SSB or TL1-RSRP_Measurement_Period_CSI-RS based on applicability as defined in clause 9.5 assuming M=1 and TReport=0.

TL1-RSRP, measure, OD-SSB

-	 is L1-RSRP measurement delay TL1-RSRP_Measurement_Period_OD SSB based on applicability as defined in clause 9.5 assuming M=1 and TReport=0 when on-demand SSB is provided for SCell activation, and the SCell being activated is transmitting on-denamd SSB burst

TL1-RSRP, report is delay of acquiring CSI reporting resources.

Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

Tuncertainty_RRC is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

Tuncertainty_SP is the time period between reception of the activation command for semi-persistent CSI-RS resource set for CQI reporting relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

TRRC_delay is the RRC procedure delay as specified in TS 38.331 [2].

TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2].

SCell in FR1 is known if it has been meeting the following conditions:

-	During the period equal to max(5*measCycleSCell,  5*DRX cycles) for FR1 before the reception of the SCell activation command:

-	the UE has sent a valid measurement report for the SCell being activated and

-	the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3.

-	the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

Otherwise SCell in FR1 is unknown.

For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions:

-	During the period equal to 4 s for UE supporting power class 1/5 and 3 s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable):

-	the UE has sent a valid L3-RSRP measurement report with SSB index, and

-	SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation

-	During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

[-EMP case

-	The L3-RSRP reported is based on a first  SSB for the case 1 where a first SSB is provided by od-ssb-config for SCell activation]

Otherwise, the first SCell in FR2 band is unknown. The requirement for unknown SCell applies provided that the activation commands for PDCCH TCI, PDSCH TCI (when applicable), semi-persistent CSI-RS for CQI reporting (when applicable), and configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) are based on the latest valid L1-RSRP reporting.

In addition to CSI reporting defined above, UE shall also apply other actions related to the activation command specified in TS 38.331 [2] for a SCell at the first opportunities for the corresponding actions once the SCell is activated.

The starting point of an interruption window on SpCell or any activated SCell, as specified in clause 8.2, shall not occur before slot n+1+THARQNR slot length  and not occur after slot slot n+1+THARQ+3ms+TX+[Tadditional prociessing time]NR slot length, where NR slot length is with respect to the numerology used in the SCell being activated, and TX is:

-	0, if Tactivation_time is 3 ms;

-	TFirst OD_SSB, for any scenario where Tactivation_time  includes TFirst OD_SSB;

-	TFirst OD_SSB_MAX, for any scenario where Tactivation_time  includes TFirst OD_SSB_MAX;

-	Tuncertainty_MAC +TFineTiming, for any scenario where Tactivation_time  includes only TFineTiming and no TFirst OD_SSB_MAX;

, and Tadditional processing time is:

-	3 and 5 for UE supporting [additional processing timing capability]

The length of the interruption window may be different for different victim cells, and depends on the applicable scenario and on the frequency band relation between the aggressor cell and the victim cell.

The requirements in this clause and requirements on interruption due to SCell activation in clause 8.2 apply provided that the on-demand SSB of the to-be-activated SCell is within the first active DL BWP of the SCell.

Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.