3GPP TSG-RAN WG4 Meeting #116	R4-2507270

Malta, Malta, May 19th – May 23rd, 2025

Source:	Nokia, Ericsson

Title:	(NR_newRAT-Core) NR-E-UTRAN HO delay requirement maintenance

Agenda item:	4.5.2

Document for:	Approval

Introduction

In RAN4#112 meeting in Maastricht, the following RAN4 agreed the CR ‘(NR_newRAT) CR on NR-E-UTRAN HO requirement’ in [1].

In the CR the actual additional delay was left in [] and for further discussion. In this paper we suggest a more reasonable delay for the MIB reception delay.

Under discussion is the following delay in section 6.1.2.1.3

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to 30 ms. If PRACH occasion is configured in radio frames with even SFN and the relative time difference between radio frame i in the current cell and the target cell is larger than or equal to 153600⋅Ts, the UE is allowed an additional MIB reception delay of [50]ms.

This paper is an updated resubmission of [5].

Discussion

Current UE requirements as defined in section 6.1.2.1 (NR – E-UTRAN) allows for a very long and relaxed handover delay (Dhandover) and quite relaxed interruption time (Tinterrupt). Such relaxed performance is of no benefit for the network, UE, operator or end-user as it may lead to noticeable cell changes for the end-user.

Concerning the handover delay the UE is allowed:

Dhandover = TRRC_procedure_delay + Tinterrupt

Where, TRRC_procedure_delay is the RRC procedure delay, which is 50ms. Tinterrupt is the actual interruption due to cell change and is defined as:

Tinterrupt =Tsearch + TIU + 20 ms

For a known cell Tsearch = 0 ms, while for an unknown cell Tsearch = 80 ms provided that the signal quality is sufficient for successful cell detection on the first attempt.

It is important to note that if the signal quality is not sufficient for successful cell detection on the first attempt RAN4 has not defined any value for Tsearch for the unknown cell. In this case the HO interrupt is not well defined.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to 30 ms. This value depends on the PRACH configuration in the target cell.

Hence, in total the Dhandover can be between: TIU + 70 or 180 ms.

RAN4 has now added a new additional delay for TIU for the NR -> E-UTRAN HO. RAN4 agreed that if the PRACH occasion is configured in radio frames with even SFN and the relative time difference between radio frame i in the current cell and the target cell is larger than or equal to 153600⋅Ts, the UE is allowed an additional MIB reception delay of [50]ms.

This additional delay of [50]ms was agreed to allow UE to read the MIB in the target cell to acquire the frame number. However, the 50ms delay for MIB reading is based on assuming worst case conditions for the UE (i.e. -6dB). In such scenario the UE may need up to 4 attempts in reading the MIB (which is assumed repeated every 10ms).

However, applying the worst case assumption for MIB reading during HO, is contradictive to the basic HO assumption. Especially, if assuming the unknown case where RAN4 has only defined requirements for signal conditions when cell detection can be done on the first attempt. In many scenarios RAN4 assumes that UE can detect a cel using a single attempt when the side condition is -2dB.

The added delay for allowing target cell MIB reading, should be aligned with the basic HO assumptions and requirements. Hence, either the cell is known (UE has detected the cell) or the cell is unknown but can be detected in one attempt. This strongly indicates that we’re not talking worst case conditions (-6dB). Instead, we see that normal worst case conditions during HO would be around -2dB. More likely the conditions at the time of HO would be even better.

Based on this, we propose that the additional delay for target cell MIB reading should be based on assumption applicable under more reasonable HO conditions, where it can be assumed that the UE can successfully receive the MIB add in first attempt.

Define the additional MIB reading delay, assuming appropriate side conditions related to the signal quality in the target cell allowing for successful MIB reception on the first attempt.

Allow UE additional 20 ms delay for target cell MIB reading.

This would align the requirements with the existing requirements principle and assumptions.

We have provided a CR capturing the CR in [2].

Same TP is provided below.

##### 6.1.2.1.3	Interruption time

When the inter-RAT handover to E-UTRAN is commanded, the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + 20 ms

Where:

Tsearch is the time required to search the target cell when the target cell is not already known when the handover command is received by the UE. If the target cell is known, then Tsearch = 0 ms. If the target cell is unknown and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch = 80 ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to 30 ms. If PRACH occasion is configured in radio frames with even SFN and the relative time difference between radio frame i in the current cell and the target cell is larger than or equal to 153600⋅Ts, the UE is allowed an additional MIB reception delay of [20]ms, provided the signal quality is sufficient for successful MIB reception on the first attempt.

- NOTE:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant E-UTRAN cell identification requirements are described in clause 9.4.1.

Conclusion

In this paper we discuss the length of the newly introduced additional delay during TIU when PRACH occasion is configured in radio frames with even SFN and the relative time difference between radio frame i in the current cell and the target cell is larger than or equal to 153600⋅Ts.

We propose following the existing principles of HO delay requirements and suggest:

- Define the additional MIB reading delay, assuming appropriate side conditions related to the signal quality in the target cell allowing for successful MIB reception on the first attempt.

- Allow UE additional 20 ms delay for target cell MIB reading.

In [6] we have provided a CR capturing the proposals.

- References

- R4-2413910, (NR_newRAT) CR on NR-E-UTRAN HO requirement, Nokia

- R4-2418376, (NR_newRAT-Core) CR on NR-E-UTRAN HO delay requirement maintenance, Nokia

- R4-2418375, (NR_newRAT-Core) NR-E-UTRAN HO delay requirement maintenance, Nokia

- R4-2501892, (NR_newRAT-Core) CR on NR-E-UTRAN HO delay requirement maintenance, Nokia

- R4-2501891, (NR_newRAT-Core) NR-E-UTRAN HO delay requirement maintenance, Nokia

- R4-2507271, (NR_newRAT-Core) CR on NR-E-UTRAN HO delay requirement maintenance, Nokia