3GPP TSG-RAN WG4 Meeting #115	R4-2507280

St Julian’s, Malta, 19th - 23th May, 2025

Source:	KDDI, Murata, Samsung

Title:	TP to TR 38.746 for adding CA_n18-n41 with UL PC1.5 n41

Agenda item:			6.8.3

Document for:	Approval

- Introduction

This is a text proposal (TP)  to TR38.746 [1] for adding CA_n18-n41 with UL PC1.5 n41.

- References

[1] TR38.746 “High power UE (power class 1.5 or 2) for NR inter-band Carrier Aggregation (CA) / Dual Connectivity (DC) with/without Supplementary Uplink (SUL) with high power on TDD band(s); (Release 19)”

[2] 3GPP TS 38.101-1-i90(2025-03): "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[3] R4-2505143, WF on MSD simplification for HPUE

Text Proposal

<Start of Text Proposal>

5	High Power UE CA of 2 bands DL and 1 or 2 bands UL

5.X 	CA_n18-n41

5.x.1	Configurations

Table 5.X.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n18A-n41A | n418,9 CA_n18A-n41A8 | n18 | 5, 10, 15 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination. NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination |  |  |  |  |

5.X.2	Maximum output power for inter-band CA

Unaffected parts are omitted.

5.x.3	REFSENS requirements.

Based on analysis,

- there is no IMD issue and no harmonic for UL n41 to DL CA_n18A-n41A

-3rd order harmonic mixing of n18 DL falls into n41.

Unaffected parts, such as inter-band cases, are omitted.

5.X.3.9	Power class 1.5 for single uplink n41

In the latest 3GPP TS 38.101-1-i90 [2], the impact of third-order harmonic mixing from n18 DL into n41 for both PC3 and PC2 cases is specified in Tables 7.3A.4-4 and 7.3A.4-4a, respectively. In Table 7.3A.4-4, the MSD values are 26.3 dB for a DL bandwidth (BW) of 5 MHz and 21.3 dB for a DL BW of 15 MHz.

For the PC1.5 case, following the method outlined in the approved Way Forward [3], ΔMSD = 9 dB, and the MSD values are calculated and presented in Table 5.x.5a.

Table 5.x.5a  (for Table 7.3A.4-4b)  : Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n41 | n183 | 10 | 15 | 25 | 5 | 35.3 | NOTE X | UL1/DL3 |
| n41 | n183 | 10 | 15 | 25 | 15 | 30.3 | NOTE Y | UL1/DL3 |
| NOTE X: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. NOTE Y: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

### 5.X.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

<End of Text Proposal>