3GPP TSG-RAN WG4 Meeting #115	R4-2507282

# Malta, MT, 19th – 23rd May, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on CSSF optimization of R19 RRM enhancements

Agenda Item:	7.18.3.2

Document for:	Discussion

- Introduction

During last RAN4 meeting, good progress were achieved around the CSSF enhancement and the following progress were summarized in [1] as below:

Issue 1-1: Applicability requirement of L3 measurement delay reduction by optimizing CSSF Agreement: - the CSSF optimization can be applied to the CSSF tables for the related CA/DC scenarios. - For solution 1: CSSF tables for CA, NR-DC, EN-DC and NE-DC - For solution 3: CSSF tables for CA, NR-DC, EN-DC with FR1 NR only - Details of whether/how the CSSF factor is modified can be left to CR discussion. Issue 1-3-1: whether solution 3 includes NR-DC scenario or not? Agreement in online session: - Solution 3 includes NR-DC of FR1 and FR1 + FR2 Issue 1-3-3: detailed CSSF table design for solution 3 Agreement in online session: - This requirement applies regardless of whether measurement gap is configured or not. Issue 1-4-1: UE capability for CSSF enhancement Agreement: - UE feature #1: Define UE capability for solution 1, further discuss whether it is per-UE or per-FR. - UE feature #2: Define UE capability for solution 3, the UE capability is defined separately for {FR1 only CA and FR1 only NR-DC}, {FR1 only EN-DC}, {FR1+FR2 CA}, {FR1+FR2 NR-DC} in the same FG.

In this paper, we provide our further views on the remaining issues of CSSF enhancement.

- Discussion

Regarding this sub topic, we share our views from the following aspects:

- Whether and how to revise solution 1

- Expand solution 1 to FR1 case

- Detailed design for solution 3

- Whether and how to revise solution 1

- In last meeting, some company proposed revision on the approved solution 1 in 114 meeting as below:

- Proposal 1: For solution 1, remove the sub-case and update the agreements as follows: - Follow the network indication to measure a SCC, if - neither PCC nor PSCC is in band, and - if there is no SCCs on which UE is configured to report SSB based measurements in band, or - if there are multiple SCCs on which UE is configured to report SSB based measurements in band. - Network indication of a specific SCC per-band for measurement. - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation. - Proposal 2: RAN4 to discuss the potential revision on network indication requests of solution 1, and if agreed, send LS to RAN2 for update.

Based on our previous agreements, actually the priority order is PCC/PSCC > SCC configured with SSB based reporting > SCC not configured with SSB based reporting. UE follows such priority order to determine one SCC to measurement both serving cell and neighbour cell per band if no interested SCC is configured by NW, otherwise the interested SCC is to be measured including both serving cell and neighbour cell in this band. It should be noted that such measurement reduction in unit of band is to reduce both serving cell and neighbour cell measurement. Based on the specification in 38.331, UE has to report the measurement results corresponding to all serving cells declared as ServingCellMO regardless the SSB based reporting is configured or not in the relevant MO configuration. We listed relevant IEs as below. Only for the measurement results corresponding to the neighbour cell, whether to report the measurement results, which is optional and up to the reporting configuration.

MeasResults ::=                         SEQUENCE {

measId                                  MeasId,

measResultServingMOList                 MeasResultServMOList,

measResultNeighCells                    CHOICE {

measResultListNR                        MeasResultListNR,

...,

measResultListEUTRA                     MeasResultListEUTRA,

measResultListUTRA-FDD-r16              MeasResultListUTRA-FDD-r16,

sl-MeasResultsCandRelay-r17             OCTET STRING        -- Contains PC5 SL-MeasResultListRelay-r17

}                                                                                                                   OPTIONAL,

...,

}

MeasResultServMOList ::=                SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServMO

MeasResultServMO ::=                    SEQUENCE {

servCellId                              ServCellIndex,

measResultServingCell                   MeasResultNR,

measResultBestNeighCell                 MeasResultNR                                                                OPTIONAL,

...

}

- Observation 1: Based on 38.331 specification, legacy UE should measure and report all serving cells measurement results by default as long as ServingCellMO configured regardless the SSB based reporting is configured or not, i.e. the serving cell measurement reporting is mandatory and the neighbour cell measurement reporting is optional.

So even under the case of neither PCC/PSCC nor SCC configured with SSB based reporting in the band, i.e. only multiple SCCs without SSB based reporting are configured in the band, the NW indication of interested SCC is still meaningful since which could indicate one serving cell to measure and report, otherwise, UE has to measure and report all serving cells.

Proposal 1: Not to revise solution 1. NW indication of interested SCC is meaningful even only multiple SCCs without SSB based reporting are configured in the band. Under such case, the NW indication is used to indicate which serving cell to be measured and reported at least.

- Expand solution 1 to FR1 case

Around the extention of solution 1 to FR1 case, some companies show concern in last meeting since picking one CC to represent the whole band in FR1 is a bit risky. Restriction such measurement reduction within the contiguous intra-band case is feasible and reasonable. Therefore we prefer to expand solution 1 to the intra-band contiguous CA in FR1.

- Proposal 2: Expand the solution 1 to FR1 intra-band contiguous CA, i.e., measure one serving CC per FR1 band if all to-be-measured serving CCs in this FR1 band are contiguous.

- Detailed design for solution 3

- During last meeting, the discussion was held around three tentative solutions:

- Tentative solution 1: measure one serving CC per band

- Tentative solution 2: change the searcher occupancy ratio of measurement among the measured CCs

- Tentative solution 3: 3 searchers for CSSF

- Tentative solution 1 has been concluded in last meeting, no need further discussion. For Tentative solution 2, since the concern on the potential impact on PCell measurement, companies are a kind of conservative on it, so RAN4 decieded not to support Teneative solution 2. In such situation, only the Tentatvie solution 3 can be used to implement the demand of prioritizing some specific frequency layer by NW. So companies are still diverse on Tentative solution 3.

- From our perspective, since solution 2 is decided not to develop, if NW wants to prioritize any SCC frequency layer for inter-band case, which can only reply on Solution 3. So we hope Solution 3 can leave room for such NW demand.

- Proposal 3: For solution 3, the 3rd searcher resouce sharing is same as the 2nd searcher sharing by default. If one SCC is prioritized by NW indication, the 3rd searcher is exclusively used by the prioritized SCC.

- Conclusion

In this contribution, we have the following proposals on the RRM enhancements:

- Observation 1: Based on 38.331 specification, legacy UE should measure and report all serving cells measurement results by default as long as ServingCellMO configured regardless the SSB based reporting is configured or not, i.e. the serving cell measurement reporting is mandatory and the neighbour cell measurement reporting is optional.

- Proposal 1: Not to revise solution 1. NW indication of interested SCC is meaningful even only multiple SCCs without SSB based reporting are configured in the band. Under such case, the NW indication is used to indicate which serving cell to be measured and reported at least.

- Proposal 2: Expand the solution 1 to FR1 intra-band contiguous CA, i.e., measure one serving CC per FR1 band if all to-be-measured serving CCs in this FR1 band are contiguous.

- Proposal 3: For solution 3, the 3rd searcher resouce sharing is same as the 2nd searcher sharing by default. If one SCC is prioritized by NW indication, the 3rd searcher is exclusively used by the prioritized SCC.

- Reference

- R4-2504899 “WF for NR_RRM_Ph5_Part1”, Apple