3GPP TSG-RAN WG4 Meeting #115	R4-2507316

St Julian’s, Malta, 19th - 23th May, 2025

Source:	KDDI, Murata, Samsung

Title:	TP to TR 38.746 for adding CA_n18-n77 with High Power UL n77

Agenda item:			6.8.3

Document for:	Approval

- Introduction

This is a text proposal (TP) to TR38.746 [1] for adding CA_n18-n77 with High Power UL n77.

The impact of harmonic mixing on CA_n18-n77 with UL n77 is not included in 3GPP TS 38-101-1-i90 due to the following reasons.

During the standardization phase for Release 17, studies on REFSENS for operator-specific band combinations were permitted. Band n18 is exclusively allocated in Japan (for KDDI), and for CA_n18-n77, the band n77 is constrained to specific frequency ranges: 3520-3560 MHz, 3700-3800 MHz, and 4000-4100 MHz. Consequently, the fourth-order harmonic mixing of n18 DL falling within 3440-3500 MHz of n77 was deemed insignificant. Thus, throughout Release 17 and early stage of Release 18, it was widely considered unnecessary to evaluate the impact of fourth-order harmonic mixing of n18 into n77 UL for CA_n18-n77.

However, in the later stages of Release 18, RAN4 decided to assess intermodulation distortion (IMD) and mixing spurious distortion (MSD) across the entire global band of n77. As a result, notes such as Note 8 [2] in Table 7.3A.5-1 were introduced, specifying that certain test points could be omitted for Japan.

The PC3 case has been proposed in R4-250xxxx for approval in this meeting. And this TP accounts for the impact of the 4th harmonic mixing of n18 on n77 UL for CA_n18-n77 across n77’s global frequency range (3300-4200 MHz) for both PC2 and PC1.5 cases.

- References

[1] TR38.746 “High power UE (power class 1.5 or 2) for NR inter-band Carrier Aggregation (CA) / Dual Connectivity (DC) with/without Supplementary Uplink (SUL) with high power on TDD band(s); (Release 19)”

[2] 3GPP TS 38.101-1-i90(2025-03): "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[3] 3GPP TR 38.717-02-01 V17.0.0 (2022-06)

[4]  RP-250214, Revised WID: Rel-19 NR Carrier Aggregation (CA)/Dual Connectivity (DC) for x bands DL with y bands UL (x<7, y<3) and Supplementary Uplink (SUL) band combinations/CA band combinations with a single SUL or two SUL cells, Ericsson, ZTE, Huawei, HiSilicon, Mar.,2025, RAN#107

[5] R4-2507279, TP to TR 38.719-02-01 for adding impact of harmonic mixing to CA_n18-n77 with UL n77

[6] R4-2505143,WF on MSD simplification for HPUE

Text Proposal

<Start of Text Proposal>

5	High Power UE CA of 2 bands DL and 1 or 2 bands UL

5.X 	CA_n18-n77

5.x.1	Configurations

Table 5.X.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n18A-n77A | n778, 9 | n18 | 5, 10, 15 | 0 |
|  | CA_n18A-n77A8 | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n18 | See n18 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n18A-n77(2A) | n778, 9 CA_n18A-n77A8 | n18 | 5, 10, 15 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n18 | See n18 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination. NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination |  |  |  |  |

5.X.2	Maximum output power for inter-band CA

Unaffected parts are omitted.

5.x.3	REFSENS requirements.

Based on analysis,

- there is no IMD issue and no harmonic issue for UL n77 to DL CA_n18-n77.

- 4th harmonic mixing of n18 DL falls into n77 UL from 3440 to 3500MHz, within the range of global band 3300-4200MHz.

Unaffected parts, such as inter-band cases, are omitted.

5.X.3.9	Power class 2 and 1.5 for single uplink n77

In the latest 3GPP TS 38.101-1-i90 [2], the impact of the 4th harmonic mixing of n18 to n77 UL for CA_n18-n77 is not included because of the historical background.

The PC3 case is proposed in R4-250xxxx [5] and the corresponding MSD values are 5.4dB for DL BW =5 MHz and 21.3 dB for DL BW=15 MHz.

For PC2 case, when PC3 MSD is 5.4, ΔMSD = 2.3, when PC3 MSD is 2.5,  ΔMSD = 1.6, and the MSD values are calculated and presented in Table 5.x.5.

For PC2 case, when PC3 MSD is 5.4, ΔMSD = 7.3, when PC3 MSD is 2.5,  ΔMSD = 5.1, and the MSD values are calculated and presented in Table 5.x.5a.

Table 5.x.5 : Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n18 | 10 | 15 | 25 | 5 | 7.7 | NOTE X | UL1/DL4 Y |
| n77 | n18 | 10 | 15 | 25 | 15 | 4.1 | NOTE X | UL1/DL4 Y |
| NOTE X: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE Y: For a UE which supports this band combination only when the Band n77 frequency range restriction of 3520 – 4100 MHz applies, the harmonic mixing is out of scope and the MSD(s) can be skipped. |  |  |  |  |  |  |  |  |

Table 5.x.5a: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n18 | 10 | 15 | 25 | 5 | 12.7 | NOTE X | UL1/DL4 Y |
| n77 | n18 | 10 | 15 | 25 | 15 | 7.6 | NOTE X | UL1/DL4 Y |
| NOTE X: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE Y: For a UE which supports this band combination only when the Band n77 frequency range restriction of 3520 – 4100 MHz applies, the harmonic mixing is out of scope and the MSD(s) can be skipped. |  |  |  |  |  |  |  |  |

### 5.X.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

<End of Text Proposal>