3GPP TSG RAN WG4 Meeting #115	R4-2507332

Malta, 19 May 2025– 23 May 2025

Agenda Item:	5.7.2

Source: 	Ericsson

Title:	Discussion on RMC Configurations for Rel-18 NR less than 5 MHz

Document for:	Discussion

# Introduction

In this paper, we will provide our views on the RMC configurations of Rel-18 NR less than 5 MHz.

# Discussion

In Rel-18, performance part, we defined RMC configurations for NR less than 5 MHz. Below, we have copied the RMC configurations from the specifications.

# A.3 RRM test configurations ## A.3.1 Reference measurement channels ### A.3.1.1 PDSCH #### A.3.1.1.1 FDD Table A.3.1.1.1-1: PDSCH Reference Measurement Channels for SCS=15 kHz Parameter; Unit; Value. Reference channel; SR.1.1 FDD; SR.1.2 FDD. Channel bandwidth; MHz; Defined in test case; Defined in test case by the parameter NRB,c. Number of transmitter antennas; 1; 1. Allocated resource blocks for PDSCH Note 1; 24; Defined by test case. Allocated slots per Radio Frame; 10; 10. Radio frame containing SSB; slots; Note 5; Note 5. Radio frame not containing SSB; slots; 10; 10. MCS index; 4; 4. Modulation; QPSK; QPSK. Target Coding Rate; 1/3; 1/3. Number of control symbols; 2; 2. PDSCH mapping type; Type A; Type A. Information Bit Payload. For slots with RMSI Note 2; bits; 1608; 1608. For slots without RMSI; bits; 1864; 1864. Number of Code Blocks per slot; 1; 1. Binary Channel Bits Per slot. For slots with RMSI Note 2, Note 4; bits; 5184; 5184. For slots without RMSI Note 6; bits; 6048; 6048. NOTE 1: Allocated outside the SMTC duration in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. NOTE 2: PDSCH is scheduled on the slots with RMSI. NOTE 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in TS 38.213 [3]. NOTE 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. NOTE 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in clause A.3.10. NOTE 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. NOTE 7: When DRX is configured, PDCCH can be scheduled both for downlink assignment and/or UL grant only during (10ms - drx-InactivityTimer) from timing when drx-onDurationTimer starts, unless otherwise specified in the test case..

SR1.2 FDD is defined for frequencies below 5 MHz. The value for SR1.2 FDD is the same as that for SR1.1 FDD, although SR1.1 FDD is specified for PRB 24.

SR1.2 FDD is defined for 12/15 PRB but has the same value as SR1.1 FDD which is specified for PDSCH with 24 PRB.

Hence, we propose to specify the values for SR1.2 FDD. Based on our calculations, as shown in the Annex, we propose the following:

Table 1: PDSCH Reference Measurement Channels for SCS=15kHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | SR.1.1 FDD | SR.1.2 FDD | SR.1.3 FDD |  |  |  |  |
| Channel bandwidth | MHz | Defined in test case | Defined in test case by the parameter r NRB,c | Defined in test case by the parameter r NRB,c |  |  |  |  |
| Number of transmitter antennas |  | 1 | 1 | 1 |  |  |  |  |
| Allocated resource blocks for PDSCH Note 1 |  | 24 | 15 | 12 |  |  |  |  |
| Allocated slots per Radio Frame |  | 10 | 10 | 10 |  |  |  |  |
| Radio frame containing SSB | slots | Note 5 | Note 5 | Note 5 |  |  |  |  |
| Radio frame not containing SSB | slots | 10 | 10 | 10 |  |  |  |  |
| MCS index |  | 4 | 4 | 4 |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK |  |  |  |  |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 |  |  |  |  |
| Number of control symbols |  | 2 | 2 | 2 |  |  |  |  |
| PDSCH mapping type |  | Type A | Type A | Type A |  |  |  |  |
| Information Bit Payload |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2 | bits | 1608 | 984 | 736 |  |  |  |  |
| For slots without RMSI | bits | 1864 | 1128 | 888 |  |  |  |  |
| Number of Code Blocks per slot |  | 1 | 1 | 1 |  |  |  |  |
| Binary Channel Bits Per slot |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2, Note 4 | bits | 5184 | 3240 | 2592 |  |  |  |  |
| For slots without RMSI Note 6 | bits | 6048 | 3780 | 3024 |  |  |  |  |
| Note 1: Allocated outside the SMTC duration in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. Note 2: PDSCH is scheduled on the slots with RMSI. Note 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in TS 38.213 [3]. Note 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. Note 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in clause A.3.10. Note 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. Note 7: When DRX is configured, PDCCH can be scheduled both for downlink assignment and/or UL grant only during ([10]ms - drx-InactivityTimer) from timing when drx-onDurationTimer starts, unless otherwise specified in the test case |  |  |  |  |  |  |  |  |

RAN4 should discuss and agree on the values for SR.1.2 FDD and SR.1.3 FDD for 15 PRB and 12 PRB, respectively.

Additionally, RAN4 should consider whether we need to define RMC configurations for both 12 and 15 PRB configurations, or if 15 PRB alone would be sufficient. From our understanding, 12PRB applies only to the n100 migration scenario. For the standard 3MHz CBW, we typically use 15PRB, as outlined in TS 38.101-1, Table 5.3.2-1. Therefore, RAN4 should focus only on 15 PRB for SR.1.2 FDD as the RMC Test configuration for NR less than 5 MHz.

RAN4 should discuss and define RMC configurations SR.1.2 FDD for 15 PRB stated in Table 1.

# Summary and Conclusion

In this contribution, we have analyzed RAN4 aspects for NR CA/DC less than 5 MHz and made the following observations and proposals.

Observation 1	SR1.2 FDD is defined for 12/15 PRB but has the same value as SR1.1 FDD which is specified for PDSCH with 24 PRB.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	RAN4 should discuss and define RMC configurations SR.1.2 FDD for 15 PRB stated in Table 1.

# References

- 3GPP TS 38.133: "NR; Requirements for support of radio resource management"

- 3GPP TS 38.214: "NR; Physical layer procedures for data"

- 3GPP TS 38.101-1: "NR; Requirements for support of radio resource management"

# Annex

Assumptions and Parameters

- Resource Element (RE) per Resource Block (RB):
Total REs per RB per slot = 12 subcarriers × 12 symbols = 144 REs

- Control Allocation Assumption:
The first slot of each radio frame, slot 0, is allocated for the control signal.

⇒ On average 11 data symbols available per RB per slot (when SSB periodicity is 10ms)

- Usable REs per RB per slot :12 subcarriers × 11 symbols = 132 REs

- DMRS Overhead:

- DMRS with RMSI= 2 symbols × 2 CDM groups × 6 RE =  24 REs

DMRS without RMSI= 1 symbols × 1 CDM group x 6RE = 6

  - With RMSI: 132-24 = 108 REs usable

  - Without RMSI: 132-6 = 126 REs usable

Channel Bit Capacity per Slot

| PRB Count | With RMSI | Without RMSI |
| --- | --- | --- |
| 15 PRBs | 108 × 2 × 15 = 3240 bits | 126 × 2 × 15 = 3780 bits |
| 12 PRBs | 108 × 2 × 12 = 2592 bits | 126 × 2 × 12 = 3024 bits |

Note: 2 bits per RE is assumed based on QPSK modulation.

Overhead Considerations

| Overhead Component | Size (bits) |
| --- | --- |
| TB CRC | 16 |
| CB CRC | 24 |
| MAC Control Element (CE) | 80 |
| MAC Subheader | 8 + 24 = 32 |
| Total Overhead (approx.) | 152 bits |

Information Bit Payload Calculation

Calculation: Info Payload = (Total Bits / Code Rate) - Overhead, assuming a code rate of ~1/3.

| PRB Count | With RMSI | Approximate TBS | Without RMSI | Approximate TBS |
| --- | --- | --- | --- | --- |
| 15 PRBs | (3240 / 3) – 152 = 928 bits | 984 bits (Table 5.1.3.2-1) | (3780 / 3) – 152 = 1108 bits | 1128 bits (Table 5.1.3.2-1) |
| 12 PRBs | (2592 / 3) – 152 = 712 bits | 736 bits (Table 5.1.3.2-1) | (3024 / 3) – 152 = 856 bits | 888 bits (Table 5.1.3.2-1) |

Note: Closest Transport Block Sizes (TBS) are derived from 3GPP TS 38.214 Table 5.1.3.2-1