3GPP TSG RAN WG4 Meeting #115	R4-2507334

Malta, 19 May 2025– 23 May 2025

Agenda Item:	7.3.3

Source: 	Ericsson

Title:	Discussion on supporting High Speed in Rel-19 NR CA DC in less than 5 MHz

Document for:	Discussion

# Introduction

In 3GPP RAN4#114-bis, we discussed the support of High Speed in Rel-19 NR CA DC in less than 5 MHz. The issue is open for discussion at this meeting.

In this paper, we will provide our views on the support of high-speed scenarios in Rel-19.

# Discussion

## Scope of High-Speed Scenario

According to Release 19 WID [1], the railway case is mentioned but does not specify the high-speed train (HST) scenarios for NR less than 5 MHz CA/DC case. Furthermore, it is not explicitly mentioned in the WID objective.

High-speed scenario is not explicitly mentioned in the WID objective of NR channel BW less than 5 MHz for FR1 Phase 2.

However, as high-speed scenario is already supported in Rel-18 for NR less than 5 MHz and specified in Rel-18 in RAN4#113 meeting.

- Specify requirements for UE speeds up to 500km/h for Band n100.

We believe we should support HST feature in Rel-19 as well.

RAN4 should define requirements for high-speed feature with NR less than 5 MHz CA/DC.

## SSB Index reading in High Speed case

In rel-18, for SSB index reading and MIB reading, the following agreement was made.

## SSB Index reading and MIB reading Agreement: - SSB index identification delay - Unknown intra-frequency cell (Es/Iot≥-6 dB): [3+4] samples - Unknown inter-frequency cell (Es/Iot≥-4 dB): [3+3] samples

However, the SSB index identification delay was not agreed upon for high-speed measurement in Rel-18. In Rel-18, many companies have proposed delay compensation for HST from no compensation needed to 2 extra samples. Hence, RAN4 should discuss and agree on the SSB index identification delay in HST case with NR less than 5 MHz. Based on the simulation results in [2], we think 2 extra samples are enough for the HST case.

RAN4 should discuss the following SSB index identification delay

- Option 1: Unknown intra-frequency cell (Es/Iot≥-6 dB): 2 extra samples for HST measurement.

- Option 2: Unknown intra-frequency cell (Es/Iot≥-6 dB): 4 extra samples for HST measurement.

- Option 3: Unknown intra-frequency cell (Es/Iot≥-6 dB): FFS for HST measurement.

# Summary and Conclusion

In this contribution, we have analyzed RAN4 aspects for NR CA/DC less than 5 MHz and made the following observations and proposals.

Observation 1	High-speed scenario is not explicitly mentioned in the WID objective of NR channel BW less than 5 MHz for FR1 Phase 2.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	RAN4 should define requirements for high-speed feature with NR less than 5 MHz CA/DC.

Proposal 2	RAN4 should discuss the following SSB index identification delay

Option 1: Unknown intra-frequency cell (Es/Iot≥-6 dB): 2 extra samples for HST measurement.

Option 2: Unknown intra-frequency cell (Es/Iot≥-6 dB): 4 extra samples for HST measurement.

Option 3: Unknown intra-frequency cell (Es/Iot≥-6 dB): FFS for HST measurement.

# References

- RP- 241622 “Revised WID: NR channel BW less than 5MHz for FR1 Phase 2”, Intel

- R4-2313692, Discussion on RRM requirements for NR in less than 5 MHz bandwidth, Ericsson

- R4-2321629, WF on NR_FR1_lessthan_5MHz_BW WI, Nokia

- 3GPP TS 38.133: "NR; Requirements for support of radio resource management"