3GPP TSG RAN WG4 Meeting #115	R4-2507336

Malta, 19th May 2025 – 23rd May 2025

Agenda Item:	7.12.5

Source: 	Ericsson

Title:	Discussion on Rel-19 ATG CA in RRM Core Open Issues

Document for:	Discussion

# Introduction

In the last meetings, we discussed and agreed on several general issues regarding the enhancement of the ATG CA. The approved WF of RAN4#114-bis can be found in [1]. This contribution discusses the remaining open issues related to ATG CA enhancement.

# Discussion

## General Issues

## UE Beam Type Signaling

In the previous meeting, we agreed on issue 1-1-2-1 as below.

Issue 1-1-2-1: The description of capability definition Agreement: - Indicates whether the UE supports one common Rx beam when UE capable of antennaArrayType-r18 on both PCC and SCC. If the field is absent, UE supports two simultaneous separate Rx beams when UE capable of antennaArrayType-r18 on both PCC and SCC. - The UE indicating support of this feature shall also indicate support of antennaArrayType-r18 on each band of the supported Band combination. Issue 1-1-2-2: The Report granularity of UE capability FFS: - Per BC

During the discussion on report granularity, we realized that the band combination (BC) is not discussed in the RF session yet. So far, in the RF and RRM sessions, the discussion has centered on the n3 and n39 BC examples stated in WID [1]. If that is the only supported band for ATG Common Rx Beam (CRxB) in Rel-19, then we do not see the necessity of introducing, per BC report granularity. Instead, we propose to introduce a single bit indication to indicate whether the UE is using CRxB or Separate Rx Beam (SRxB).

38.101-1 has no supported band combination stated for ATG.

RAN4 RF session has not yet provided any clear indications or discussions on ATG BC yet.

Therefore, we do not see any strong motivation to introduce Per BC if it is only one BC or no specific BC specified for ATG in 38.101-1 in Rel-19.

Furthermore, even if a device indicates antennaArrayType-r18 and multiple band combinations (i.e., BC1 and BC2), it is not clear how the network knows BC1 is being used for CRxB or SRxB when the Per BC indication is used for multiple BC indications. Hence, we believe 1 explicit indication of CRxB or SRxB should be supported. Also, a network indication based on certain BC activation for CRxB can be considered.

Explicit indication of UE Common Rx Beam should be supported.

Alternatively, we can discuss the possibility that a CRxB declared UE cannot configure SRxB and vice versa.

Furthermore, we believe an LS should send out to RAN2. A draft of the LS can be found in Annex A.

RAN4 should send an LS to RAN2 based on LS draft in Annex A.

## Conditions for CBM vs IBM

RF Session agreed that an ATG UE with an antenna array on the PCC and SCC bands using a common Rx beam is possible for specific band combinations. In TS 38.133, CBM is not specified for SCell addition/activation delay in FR1, leaving a gap in how such configurations should be handled. Given that a common Rx beam allows the UE to reuse PCell measurements for the co-located SCell, applying CBM can reduce measurement overhead and activation latency. However, beam-specific measurements are required to ensure accuracy in scenarios where the UE supports SRxB. Therefore, RAN4 should discuss and clearly capture the conditions under which CBM should be applied.

CBM conditions are not yet specified in the RF specifications.

It is unclear how to distinguish between ATG IBM and CBM. Thus, the definition of the UE capability regarding when and under what conditions a UE can use CRxB is also not clear.

RAN4 should discuss and capture the conditions under which CBM should be applied for SCell activation.

According to the WID and RF agreement, a representative inter-band CA example in FR1 is CA_n3-n39, where band n3 spans 1805–1880 MHz and band n39 spans 1880–1920 MHz. With a minimum channel bandwidth of 5 MHz, the frequency separation between these bands remains within 100 MHz. In principle, CBM assumes that beam characteristics, such as directionality and gain patterns, remain sufficiently similar across PCC and SCC to reuse measurements (e.g., SS-RSRP, CSI-RSRP) from one carrier to another. However, beam behaviour is frequency-dependent, and even within FR1, a 100 MHz separation, especially in mid-band frequencies, can cause divergence in beam alignment due to differences in antenna beamwidth and steering. While 100 MHz may still be acceptable for some UEs equipped with carefully designed wideband antenna arrays, for robust and standards-aligned CBM operation, a reduced separation (e.g., ≤50 MHz) may be safer to ensure that a common Rx beam delivers consistent gain and directionality across both carriers. Therefore, it is proposed that CBM may be supported if the frequency separation between co-located inter-band DL CA bands is within a threshold, tentatively 50 to 100 MHz, beyond which the UE should default to IBM. RAN4 should evaluate and define a clear separation threshold, documenting when CBM or IBM applies, considering UE antenna architecture, carrier frequency proximity, and the need for accurate and reliable measurement behaviour.

RAN4 should evaluate and define a clear separation threshold, document when CBM or IBM applies.

RAN4 should discuss and agree on the following conditions to differentiate between CBM and IBM.

- -If the frequency ranges of inter-band co-located DL CA bands are less than or equivalent to X MHz, then the ATG UE may support CBM with an antenna array on the PCC and SCC in FR1.

- -Otherwise, ATG UE with an antenna array on the bands of PCC and SCC will adhere to IBM in FR1.

RAN4 should discuss and agree on the DL frequency separation (i.e., X MHz) between the two bands for FR1 CBM.

- Option 1: ≤50 MHz.

- Option 2: ≤100 MHz.

- Option 3: Between 50- 100 MHz, FFS.

## Signaling Characteristics

## SCell Addition/Activation Delay

Upon reviewing the current SCell activation delay requirements, we found that SCell activation from CBM and IBM is not specified for the FR1 UE antenna array. Therefore, we propose to discuss and capture the following points.

#### SCell Addition/Activation Delay for Common Rx Beam

To support SCell Activation Delay for Common Rx Beam, the following scenarios need to be captured in the specification.

- Scenario 1: Known SCell with CBM

- Scenario 2: PCell information can be reused for unknown SCell with CBM

In Scenario 1, the Scell is already known to the UE but is initially deactivated. Since the SCell and PCell have co-located beams and operate in close frequency bands, the same beam refinement process can be used. UE does not require additional beam acquisition for SCell activation, and CRxB steering reduces activation delay by minimizing the need for additional CSI-RS measurements. In such a scenario, ATG UE can skip the Cell search and L1-RSRP measurement. Hence, the Tactivation_time can be set to 3ms.

For Known SCell with CBM ATG UE can skip Cell search, and L1- RSRP measurement.

RAN4 should discuss and agree that In Known SCell with common Rx beam, the Tactivation_time time can be 3ms.

In Scenario 2,  the SCell is unknown and not pre-configured to the UE, but the UE can reuse the PCC beam information. Since the SCell is not pre-configured, the UE must perform an SSB search. The beam management delay is zero because the UE uses the PCell beam information to determine the beam for the SCell. Hence, if the target SCell is unknown with CRxB then Tactivation_time is 3ms+TFirstSSB+ 2ms.

For unknown SCells, CBM beam information can be reused, but the UE still needs to perform the SSB search in the SCell.

RAN4 should discuss and agree that if the target SCell is unknown with common Rx beam, Tactivation_time is 3ms+TFirstSSB+ 2ms.

## Interruption Requirements

In the last meeting, following proposals were discussed but not concluded.

Issue 1-2-2: Interruption requirements (ZTE) - Proposals: - Proposal 1: - For the interruptions caused by SCell addition/release and any types of SCell activation/deactivation, distinguish the interruption requirements for Category 1 and 2. - Category 1: antenna array assumption in intra-band CA and Case 4 in inter-band CA - Category 2: other cases For other interruptions, define unified interruption requirements for all UE antenna types.

Based on the assumptions of an antenna array, the differences between common and separate beam categories correspond to the variations between UE utilizing IBM in FR2 band pair and CBM within the FR2 band. In clauses 8.2.2.2.1, 8.2.2.2.2, 8.2.2.2.5, and 8.2.2.2.17, similar logic should apply: interruption durations should be distinguished based on beam assumptions. Common beam cases, with shared spatial domains, support shorter interruptions due to lower beam-switching overhead. Separate beam cases, involving independent beam control and training, require longer interruptions. Differentiating the interruption requirements reflects ATG DL UE CA scenarios and aligns with existing antenna array distinctions.

For clauses 8.2.2.2.1, 8.2.2.2.2, 8.2.2.2.5, and 8.2.2.2.17 distinguish the interruption requirements for Category A and B.

Category A: Common Rx beam assumption (e.g., co-located beams with aligned timing and shared spatial domain)

- Category B: Separate Rx beam assumption (e.g., independent beams with distinct directionality and control)

Moreover, we think below changes to make in the interruption length in 8.2.2.2.1, 8.2.2.2.2 and wherever applicable in the spec.

Table1: Interruption length X1 for SCell addition/release for inter-band CA

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 | 1 |  |
| 1 | 0.5 | 2 |  |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 4 |
|  |  | Both aggressor cell and victim cell are on FR1 | 5 |
|  |  | Either aggressor cell or victim cell is on FR1 | 5 |
| 3 | 0.125 | Aggressor cell is on FR2 | 8 |
|  |  | Aggressor cell is on FR1 | 9 |
| 5 | 0.03125 | Aggressor cell is on FR1 | 33 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 65 |

RAN4 should discuss and agree on the length of the interruption to capture the impact of case 4 as indicated in Table 1.

## Measurement Procedure

    - Sharing Factor between L1 measurement and RRM measurement

In the last meeting, one of the companies proposed 1 extra symbol before and after each consecutive SSB symbol and the RSSI symbols of serving cells when inter-band carrier aggregation within FR1 is configured, and the UE supports Case 4. The proposal is the following:

Issue 1-3-2: Sharing factor between L1 measurement and RRM measurement - Proposal 1: (LG) - Consider impact on Klayer1_measurement due to 33usec MRTD for Case 4 antenna type, e.g., - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 2 symbol before each consecutive SSB symbols and the RSSI symbols, and 2 symbol after each consecutive SSB symbols and the RSSI symbols of serving cells when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement

We believe the assumption that a 33μs MRTD results in the loss of an entire data symbol is overly conservative, especially when the ATG DL UE is co-located and in line of sight, where the usual RTD is often no more than 5 μs. UEs with a common Rx beam should support sufficient beam-switching speed and accuracy, while fast intra-symbol alignment techniques can further minimize data symbol loss.

Moreover, specification often leaves beam management specifics, like timing alignment and beam sweep duration, open to UE implementation. Applying a standard-wide Layer 1 measurement penalty assumes a worst-case UE design and restricts flexibility. Efficient UEs could handle Rx beam sweeping without affecting RLM or L1-RSRP measurements..

SMTC-based measurements are designed for flexibility, allowing adaptive scheduling of PDSCH and CSI-RS without strict symbol overlap limitations. The existing specification accommodates optional scheduling configurations, making additional mandatory adjustments unnecessary. There is no evidence in document 38.133 indicating significant measurement inaccuracies or radio link degradation due to a 33 μs MRTD, some empirical data is needed to motivate this issue. Hence, we believe the baseline specification is valid, and no extra relaxation is needed in this case.

We propose to reuse the legacy specification, i.e.

- 	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols of serving cells when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement

    - Measurement enhancement for ATG UE with antenna array

The following proposal is discussed but not concluded and need further discussion in this meeting.

Issue 1-3-5: Measurement enhancement for ATG UE with antenna array (CMCC) Candidate options: - UE could skip the neighbour cell measurement of {SCC and/or CC with deactivated SCell} by network control. - Option 1: This can already be achieved by the existing configuration. - Option 2: Consider enhancement for Rel-19 ATG CA

Based on our understanding, neighboring cell measurement is typically associated with mobility procedures, which are managed by the PCell (or Master Node in Dual Connectivity). The configuration of neighboring cell measurements, whether for PCell or SCell frequencies, is always network-controlled via RRC signaling from the PCell or Master Node. While the SCell can serve as the frequency source for such measurements, it does not initiate or control the measurement process independently. If a neighboring cell measurement is not configured for a particular SCell frequency, the UE will not measure that frequency. Hence, we believe this is already possible with existing configurations, and no need for any further enhancement, emissions. Furthermore, we have only one meeting left to finalize this WI. Hence, we can deprioritize this issue.

RAN4 should not support Measurement enhancement for ATG UE with antenna array.

# Summary and Conclusion

In this contribution, we have analyzed RAN4 RRM aspects for ATG UE DL CA and made the following observations and proposals.

Observation 1	38.101-1 has no supported band combination stated for ATG.

Observation 2	RAN4 RF session has not yet provided any clear indications or discussions on ATG BC yet.

Observation 3	CBM conditions are not yet specified in the RF specifications.

Observation 4	It is unclear how to distinguish between ATG IBM and CBM. Thus, the definition of the UE capability regarding when and under what conditions a UE can use CRxB is also not clear.

Observation 5	For Known SCell with CBM ATG UE can skip Cell search, and L1- RSRP measurement.

Observation 6	For unknown SCells, CBM beam information can be reused, but the UE still needs to perform the SSB search in the SCell.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	Explicit indication of UE Common Rx Beam should be supported.

Proposal 2	RAN4 should send an LS to RAN2 based on LS draft in Annex A.

Proposal 3	RAN4 should discuss and capture the conditions under which CBM should be applied for SCell activation.

Proposal 4	RAN4 should evaluate and define a clear separation threshold, document when CBM or IBM applies.

Proposal 5	RAN4 should discuss and agree on the following conditions to differentiate between CBM and IBM.

-If the frequency ranges of inter-band co-located DL CA bands are less than or equivalent to X MHz, then the ATG UE may support CBM with an antenna array on the PCC and SCC in FR1.

-Otherwise, ATG UE with an antenna array on the bands of PCC and SCC will adhere to IBM in FR1.

Proposal 6	RAN4 should discuss and agree on the DL frequency separation (i.e., X MHz) between the two bands for FR1 CBM.

Option 1: ≤50 MHz.

Option 2: ≤100 MHz.

Option 3: Between 50- 100 MHz, FFS.

Proposal 7	RAN4 should discuss and agree that In Known SCell with common Rx beam, the Tactivation_time time can be 3ms.

Proposal 8	RAN4 should discuss and agree that if the target SCell is unknown with common Rx beam, Tactivation_time is 3ms+TFirstSSB+ 2ms.

Proposal 9	For clauses 8.2.2.2.1, 8.2.2.2.2, 8.2.2.2.5, and 8.2.2.2.17 distinguish the interruption requirements for Category A and B.  Category A: Common Rx beam assumption (e.g., co-located beams with aligned timing and shared spatial domain)

Category B: Separate Rx beam assumption (e.g., independent beams with distinct directionality and control)

Proposal 10	RAN4 should discuss and agree on the length of the interruption to capture the impact of case 4 as indicated in Table 1.

Proposal 11	We propose to reuse the legacy specification, i.e.

	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols of serving cells when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement

Proposal 12	RAN4 should not support Measurement enhancement for ATG UE with antenna array.

# References

- R4-2504898, WF on RRM requirements for NR_ATG_enh, CMCC

- R4-2502591, WF on RRM requirements for NR_ATG_enh, CMCC

- R4-2420237, WF on RRM requirements for NR_ATG_enh, CMCC

- R4-2420338, WF on NR ATG UE RF, CMCC

- R4-2415002, (NR_ATG_enh-Core) Discussion on RRM core requirements for R19 ATG, CMCC

- R4-2417716, Discussion on enhancements for Rel-19 ATG RRM requirements, CATT

# Annex A: LS to RAN2: UE Antenna Type Signalling

Title:	LS on UE capability for UE Antenna Type Signalling in ATG

Response to:

Release:	Release 19

Work Item:	NR_ATG_enh-Core

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

Name:	Istiak Hossin

E-mail Address:	istiak.hossain@ericsson.com

Attachments: -

# 1	Overall description

RAN4 has discussed and agreed that 5 antenna type combinations can be used for ATG DL CA. The agreed antenna types are the following:

1. ATG UE with omnidirectional antennas on the bands of PCC and SCC.

2. ATG UE with an omnidirectional antenna on the band of PCC and an 	antenna array on the band of SCC.

3. ATG UE with an antenna array on the band of PCC and an omnidirectional 	antenna on the band of SCC.

4. ATG UE with an antenna array on the bands of PCC and SCC, and one 	common Rx beam assumed.

5. ATG UE with an antenna array on the bands of PCC and SCC, and two 	simultaneous separate Rx beams assumed.

To differentiate between different UE array antenna types (i.e. case 4 and case 5) for Inter-band co-located CA, an additional UE capability is needed to indicate whether the Rx beam is common or independent. It is agreed that the UE can inform the network about the antenna/ beam type used for measurements in an NR ATG cell group via a 1-bit indicator or explicit signaling in UE capability information IE. The indicator specifies whether the common or independent Rx beam is used for PCell and SCell measurements, as depicted in Table 1.

| Value | Antenna type |
| --- | --- |
| 0 | Antenna arrays with separate antennas |
| 1 | Antenna arrays with only one antenna panel |

Table  Example of UE indicating antenna type

# 2 Actions

To RAN2

ACTION: 	RAN4 respectfully asks RAN2 to take above information into account and capture the UE antenna type signaling in the required specification/s.

# 3	Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #116	            	  25 – 29 August 2025, India

TSG RAN WG4 Meeting #116-bis	            13 – 17 October 2025, Prague