3GPP TSG-RAN4 Meeting #115	R4-2507342

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0780 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_HST_FR2-Perf) CR on 38.101-4 for correcting FRC Table |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR2-Perf |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Incorrect implementation from mirror CR R4-2312216. The parent Rel-17 CAT F CR R4-2313933 was correctly implemented. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the cells in Table A.3.2.2.5-12, which were incorrectly updated from the CR R4-2312216. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect FRC configuration in Table A.3.2.2.5-12 and Rel-18 is not consistent with Rel-17 spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change 1>

#### A.3.2.2.5	Reference measurement channels for SCS 120 kHz FR2

<Unchanged parts omitted>

Table A.3.2.2.5-12: PDSCH Reference Channel for TDD UL-DL pattern FR2.120-1 and HST-DPS scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-12.1 TDD | R.PDSCH.5-12.2 TDD |  |  |  |
| Channel bandwidth | MHz | 200 | 200 |  |  |  |
| Subcarrier spacing | kHz | 120 | 120 |  |  |  |
| Allocated resource blocks | PRBs | 132 | 132 |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 9 | 9 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} |  | 13 | 13 |  |  |  |
| For Slot i=1 |  | N/A (Note 1) | N/A(Note 4) |  |  |  |
| For Slot i=2 |  | 13 | N/A (Note 4) |  |  |  |
| For Slot i=3 |  | 9 | N/A (Note 4) |  |  |  |
| Allocated slots per 2 frames |  | 126 | 124 |  |  |  |
| MCS table |  | 64QAM | 64QAM |  |  |  |
| MCS index |  | 17 | 17 |  |  |  |
| Modulation |  | 64QAM | 64QAM |  |  |  |
| Target Coding Rate |  | 0.43 | 0.43 |  |  |  |
| Number of MIMO layers |  | 2 | 2 |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 18 | 18 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 18 | 18 |  |  |  |
| For Slot i = 1 |  | N/A (Note 1) | N/A (Note 4) |  |  |  |
| For Slot i=2 |  | 18 | N/A (Note 4) |  |  |  |
| For Slot i=3 |  | 18 | N/A (Note 4) |  |  |  |
| Overhead for TBS determination |  | 6 | 6 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 57376 | 57376 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 90176 | 90176 |  |  |  |
| For Slot i = 1 | Bits | N/A (Note 1) | N/A (Note 4) |  |  |  |
| For Slot i = 2 | Bits | 90176 | N/A (Note 4) |  |  |  |
| For Slot i = 3 | Bits | 57376 | N/A (Note 4) |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 24 | 24 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 24 | 24 |  |  |  |
| For Slot i = 1 | Bits | N/A (Note 1) | N/A (Note 4) |  |  |  |
| For Slot i = 2 | Bits | 24 | N/A (Note 4) |  |  |  |
| For Slot i = 3 | Bits | 24 | N/A (Note 4) |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | CBs | 7 | 7 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | CBs | 11 | 11 |  |  |  |
| For Slot i = 1 | CBs | N/A (Note 1) | N/A (Note 4) |  |  |  |
| For Slot i=2 | CBs | 11 | N/A (Note 4) |  |  |  |
| For Slot i=3 | CBs | 7 | N/A (Note 4) |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A |  |  |  |
| For Slots i = 5 and 85 (Note 3) | Bits | 195696 | 181968 |  |  |  |
| For Slots i = 6 and 86 (Note 3) | Bits | 195696 | 181968 |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {8,…,84, 87,…,159} | Bits | 137808 | 137808 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {7,…,84,87,…,159} | Bits | 210672 | 210672 |  |  |  |
| For Slot i = 1 | Bits | N/A (Note 1) | N/A (Note 4) |  |  |  |
| For Slot i= 2 | Bits | 210672 | N/A (Note 4) |  |  |  |
| For Slot i= 3 | Bits | 137808 | N/A (Note 4) |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 515.62 | 508.25 |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 and slot #1 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. Note 4: SS/PBCH block is transmitted in slot #0, slot #1, slot #2 and slot #3 with periodicity 20ms |  |  |  |  |  |  |

# <End of change 1>