3GPP TSG-RAN WG4 Meeting #115	R4- 2507345

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.31.3.1

Source:	MediaTek Inc.

Title:	Discussion on Rel-19 NR-NTN RedCap UE RF requirements

Document for:	Approval

- Introduction

In RAN#102 plenary meeting, the Rel-19 WID of Non-Terrestrial Networks (NTN) for NR Phase 3 was approved in [1]. One of the main objectives is to support Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN band.

In RAN4#114bis meeting, the WF [2] for guiding further discussion on residual issues was proposed. In this contribution, our views on the residual issues are provided.

- Discussion

2.1		Guidance for Rel-19 phase 3 NR-NTN UE RF requirements

The Rel-19 WID [1] objectives indicate that NTN for NR Phase 3 supports Redcap and eRedcap with NR NTN operating in FR1-NTN band. The objectives are shown below.

4.1 Objective of SI or Core part WI or Testing part WI Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, check whether any essential changes are needed for their support (i.e. focusing on HD collision rules) by end of Q2/2024 [RAN1] - Depending on feasibility assessment above, define the RF and RRM requirements [RAN4] Notes for this objective: GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

2.2		Previous Meeting Agreements

In RAN4#114bis, the approved WF [2] provided the guidance for following works. The partial content is listed below for reference.

Issue 1-1: Output power enhancement for NR NTN PC3 Agreements: - If defined, additional output power enhancement will be optional for UE to support. - FFS on need of new UE capability - Evaluate potential to further enhance output power for RAN4#115: - Amount of output power enhancement [0.5 to 1 dB] - Less than 0.5 dB improvement is not specified - Initially consider all RB allocations and modulation orders - Consider both MPR reduction and MOP increase - FFS how to capture MOP increase in the specification - Choice between MPR reduction and MOP increase can be different for inner/outer allocations and modulation orders - Evaluate both HD-FDD and FD-FDD - Consider at least insertion loss impact - FFS whether to apply same enhancement to both HD-FDD and FD-FDD - Evaluate the option to support both rel-18 and rel-19 output power enhancement schemes simultaneously - FFS whether simultaneous support is specified - Consider whether PC3 or PC2 ACLR requirement should apply - FFS Applicability to PC3 non-RedCap NR NTN

In RAN1#120bis meeting, the approved LS [3] provided the information for the following works. The content is listed below for reference.

Title: LS on requirements for the phase continuity and power consistency for OCC with PUSCH in NR NTN Ph3 1. Overall description RAN1 assumes that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH. - FFS: under which conditions the above applies, e.g., under the same conditions that phase continuity applies for DMRS bundling. RAN1 agreed the following: 2. Actions: RAN1 respectfully ask RAN4 whether the same phase continuity requirements as for DMRS bundling can be applied for OCC length 2 and/or OCC length 4 under the same conditions as for DMRS bundling, or if new requirements are needed.

2.3		PC3 output power for (e)RedCap

It is observed that the HD-FDD RF front-end insertion loss (FE IL) is lower compared to FD-FDD FE IL and can improve the RX REFSENS and TX maximum output power (MOP) simultaneously.

Proposal 1: The output power enhancement for PC3 NTN (e)RedCap due to HD-FDD insertion loss(IL) improvement can be 0.8dB.

Proposal 2: It may be possible to have TX MOP increment due to FD-FDD IL improvement, but it should be studied whether FD-FDD further [0.5] dB IL improvement would be achieved or not as this is not easy.

Proposal 3: Regarding “Initially consider all RB allocations and modulation orders for PC3 NTN (e)RedCap output power enhancement”, the new UE capability would be needed.

Proposal 4: Regarding HD-FDD IL improvement, it is okay to consider both MPR reduction and MOP increase by leveraging power boosting framework (e.g., powerBoostPi2BPSK, powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18).

Proposal 5: Regarding R18 PAPR-reduction power boost or R19 HD-FDD IL improvement, they can be simultaneous supported since they are based on different technology improvement.

2.4		Phase continuity and power consistency for OCC

It is expected that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH.

Observation 1: It is expected that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH.

Proposal 6: Consider that the same phase continuity requirements as for DMRS bundling can be applied for OCC length 2 and/or OCC length 4 under the same conditions as for DMRS bundling.

3 Conclusion

In this paper, our views on NTN-RedCap UE RF requirements are provided as follows:

Proposal 1: The output power enhancement for PC3 NTN (e)RedCap due to HD-FDD insertion loss(IL) improvement can be 0.8dB.

Proposal 2: It may be possible to have TX MOP increment due to FD-FDD IL improvement, but it should be studied whether FD-FDD further [0.5] dB IL improvement would be achieved or not as this is not easy.

Proposal 3: Regarding “Initially consider all RB allocations and modulation orders for PC3 NTN (e)RedCap output power enhancement”, the new UE capability would be needed.

Proposal 4: Regarding HD-FDD IL improvement, it is okay to consider both MPR reduction and MOP increase by leveraging power boosting framework (e.g., powerBoostPi2BPSK, powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18).

Proposal 5: Regarding R18 PAPR-reduction power boost or R19 HD-FDD IL improvement, they can be simultaneous supported since they are based on different technology improvement.

Observation 1: It is expected that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH.

Proposal 6: Consider that the same phase continuity requirements as for DMRS bundling can be applied for OCC length 2 and/or OCC length 4 under the same conditions as for DMRS bundling.

4 Reference

- RP-234078, “New WID: Non-Terrestrial Networks (NTN) for NR Phase 3”

- R4-2504708, “Way Forward for [114bis][309] NR_NTN_Ph3_General_UE_SAN_RF”, Qualcomm

- R1-2503071, “LS on requirements for the phase continuity and power consistency for OCC with PUSCH in NR NTN Ph3”, MediaTek Inc.