3GPP TSG-RAN WG4 Meeting #115-	R4-2507352

Malta, MT, 19th – 23rd May, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for SSB adaptation

Agenda item:	7.27.3.2

Document for:	Discussion

Introduction

The discussion on SSB adaptation as part of the third objective “adaptation of common signal/channel transmissions” of the Rel-19 WID on Enhancements of network energy savings for NR [1], was continued at RAN4#114bis meeting [2] and the agreements were captured in the WF [3]. In this contribution, we continue the discussion on the RRM impact from SSB adaptation.

Discussion

  - Progress status in RAN4 and RAN1

At RAN4#114bis meeting, the following has been agreed on RRM requirements for SSB adaptation [3].

- Sub-topic 1-1 SSB adaptation – Scenario Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - Further clarification on the scope of RRM impact for SSB adaption: - Existing SSB burst periodicity (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms). - SSB adaptation in SCell for NCD-SSB. - Time domain positions of SSBs within a burst is same before and after SSB adaptation - Note: Above could be further updated based on further RAN1 progress if any. - Sub-topic 1-2 SSB adaptation – L1 Requirements impacts Issue 1-2-1: L1 requirements at transition Agreement - RAN4 FFS how to define Tfirst_SSB considering the processing/preparation time based on further RAN1/2 progress on SSB adaptation command timeline. - No impacts on section 9.5.3 of L1 reporting requirements due to SSB adaptation, unless identified by other WGs. - Sub-topic 1-3 SSB adaptation – L3 Requirements impacts Issue 1-3-1: L3 requirements for SSB adaptation Agreement: - RAN4 to define requirements based legacy L3 measurement framework for SSB adaptation. For neighbour cell L3 measurement, legacy requirements apply.

Open issue for further discuss: - For SCell with SSB adaptation (deactivated SCell or activated SCell), wait for more RAN2 progress on additional SMTC configuration, and discuss following aspects: - Whether to define L3 measurement requirement at transition. - Any other RRM requirements impacts due to additional SMTC configurations. - Sub-topic 1-4 SSB adaptation – Other requirements impacts Issue 1-4-2: SCell activation requirement for SSB adaptation – applicable SCell activation requirements Agreement: - Normal SCell activation - Direct SCell activation - PUCCH SCell activation - FFS Rel-18 SCell activation with L3 reporting - FFS Multiple SCells activation

Open issues for SSB adaptation are contained in [2] and previous RAN4 agreements are listed in Annex 1.

RAN1 agreements are listed in Annex 2.

  - Scenarios for SSB adaptation

In this section we consider open issues related to scenarios for SSB adaptation.

## SSB periodicities

Regarding the number of supported SSB periodicities in SCell, RAN1#120bis achieved following agreement.

Agreement For adaptation of SSB in time-domain, UE can be configured with X (<=Xmax) additional SSB burst periodicities for an SCell. - Xmax=2

Hence, UE may be configured with default SSB periodicity, i.e. 20 ms, and up to 2 additional SSB periodicities, used for serving SCell measurements. According to RAN1 discussion, two cases need to be distinguished:

Case A: Legacy UEs and Rel-19 NES capable UEs in the cell

In this case, SSB periodicity cannot be changed to a longer periodicity than the default SSB periodicity. The additional SSB periodicities (1 or 2) need to be shorter than the default SSB periodicity.

Case B: Only Rel-19 NES capable UEs in the cell

In this case, SSB periodicity can be changed to a longer or shorter periodicity than the default SSB periodicity, thus the additional SSB periodicities (1 or 2) can be shorter or longer than the default SSB periodicity.

Observation 1: The additional SSB periodicities, configured for SSB adaptation, may be longer or shorter than the default SSB periodicity depending on whether legacy UEs are also served in SCell or not.

## Applicable SCell activation scenarios with SSB adaptation

RAN4 #114bis discussed the applicable SCell activation scenarios with SSB adaptation and agreed to specify/study requirements for the following scenarios [3].

Issue 1-4-2: SCell activation requirement for SSB adaptation – applicable SCell activation requirements Agreement: - Normal SCell activation - Direct SCell activation - PUCCH SCell activation - FFS Rel-18 SCell activation with L3 reporting - FFS Multiple SCells activation

In our view, aside the first three SCell activation scenarios, agreed at RAN4 #114bis, the fourth scenario

- Rel-18 SCell activation with L3 reporting,

which targets to improve the activation delay in case of unknown single SCell, should also be considered for SSB adaptation to reduce the cell detection delay. Thereagainst, multiple SCells activation is a further enhancement on top of single SCell activation. This scenario has not been discussed in RAN1 so far, neither has multiple OD-SSB activation related to objective 1 been agreed in RAN1. Hence RAN4 should exclude this scenario for Rel-19 NES enhancements.

Proposal 1:	RAN4 to add the scenario of Rel-18 SCell activation with L3 reporting to the applicable SCell activation scenarios with SSB adaptation.

Proposal 2:	RAN4 to exclude multiple SCells activation scenario with SSB adaptation from Rel-19 NES enhancements.

    - Signaling methods for SSB adaptation

RAN4#114bis kept the discussion open on signaling methods for triggering SSB adaptation awaiting further progress in RAN1 and RAN2 [2].

In fact, as SSB adaptation represents a network-controlled procedure, as agreed under issue 1-1-3 at RAN4 #113, see Annex 1, the UE needs to process the signaling message, depending on the signaling method, which will be defined by RAN1 and RAN2 (e.g., based on group common DCI, MAC-CE or RRC). However, RAN2#129bis decided not to specify a MAC-CE based solution for SSB adaptation aside the group common DCI based solution designed by RAN1. Thus, RAN4 needs to define RRM requirements for SSB adaptation triggered only by group common DCI command and by RRC based signaling.

Proposal 3: 	RAN4 to consider specifying RRM requirements for SSB periodicity adaptation for NW triggered group common DCI and RRC signaling methods, designed by RAN1 and RAN2.

  - Layer 1 measurement impact

RAN4#114 achieved the agreement on requirements needing updating as listed in Annex 1. Based on this, SSB based L1 measurements in RRC CONNECTED, agreed to be affected by SSB adaptation, are identified to be the following ones:

- CBD,

- L1-RSRP,

- L1-SINR.

RAN4 #114 also agreed to consider the L1 measurement impact from SSB periodicity adaptation is limited to transition periods when changing the SSB periodicity.

## L1 transition requirements

RAN4#114 achieved following agreement related to L1 transition requirements,

Issue 1-2-2: Transition requirements applicability & Issue 1-2-3: Transition requirements – details Agreement: - For L1-RSRP/L1-SINR/CBD - The measurement/evaluation period based on SSB periodicity after adaptation apply from Tfirst_SSB. - Tfirst_SSB is the uncertainty time from the timepoint when SSB adaptation occurs till the first SSB after SSB adaptation.

RAN4#114bis achieved following agreement related to L1 transition requirements.

Issue 1-2-1: L1 requirements at transition Agreement - RAN4 FFS how to define Tfirst_SSB considering the processing/preparation time based on further RAN1/2 progress on SSB adaptation command timeline. No impacts on section 9.5.3 of L1 reporting requirements due to SSB adaptation, unless identified by other WGs.

RAN1#120bis agreed the following working assumption.

Working Assumption When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission on a second serving cell, the UE assumes SSB is transmitted on the second serving cell according to the indicated SSB burst periodicity from the beginning of the first slot containing the first [actually] transmitted SSB within the first [possible] SSB burst according to the indicated SSB burst periodicity that is no earlier than the slot m+d of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell [in Table 11.5-1 of TS 38.213]. FFS: how to determine the first [possible] SSB burst

Thus, based on this agreed working assumption, to be confirmed by RAN1, UE assumes that the first SSB burst belonging to the new SSB periodicity, will not occur earlier than d slots after DCI 2_9 reception (minimum time gap d=3 slots, i.e. 3ms, for SCS=15 kHz in TS 38.213, Table 11.5-1).

RAN4 thus should identify if 3ms is sufficient time for processing SSB adaptation or additional time is needed to schedule the receiver for the next SSB reception. It is assumed that RAN4 will define some additional time as done for OD-SSB. In the following, we use the term TSSB_adaptation , which includes the minimum time gap d and any additional processing time Tprocessing RAN4 may define.

The term Tfirst_SSB refers to the first SSB that the UE is able to receive and to the first measurement in the new evaluation period based on the new indicated SSB burst periodicity. Tfirst_SSB depends on one side on the new SSB burst periodicity and on the other side on the time when the SSB adaptation command is sent with regard to the last SSB.

For defining Tfirst_SSB, different use case scenarios 1 to 4 are distinguished, which is illustrated in Figure 1 to 4.

- Scenario 1: SSB adaptation to lower periodicity with early SSB adaptation command received after completed L1 measurement period (MP), see Figure 1.

- Scenario 2: SSB adaptation to lower periodicity with late SSB adaptation command received after completed L1 measurement period (MP), see Figure 2.

- Scenario 3: SSB adaptation to higher periodicity with early SSB adaptation command received after completed L1 measurement period (MP), see Figure 3.

- Scenario 4: SSB adaptation to higher periodicity with late SSB adaptation command received after completed L1 measurement period (MP), see Figure 4.

Figure 1: SSB adaptation to lower periodicity with early SSB adaptation command after completed L1 MP.

Figure 2: SSB adaptation to lower periodicity with late SSB adaptation command after completed L1 MP.

Figure 3: SSB adaptation to higher periodicity with early SSB adaptation command after completed L1 MP.

Figure 4: SSB adaptation to higher periodicity with late SSB adaptation command after completed L1 MP.

In Figure 1, if the DCI command is sent timely after the last SSB burst, the UE has sufficient time to receive the new SSB burst even when SSB periodicity is getting shorter. Thereagainst in Figure 2, if the DCI command is not sent timely, the UE cannot prepare for the next (=first) SSB reception and hence the UE will only receive the next (=second) SSB. This means Tfirst_SSB depends on whether the group DCI command 2_9 including the SSB adaptation indication is timely received, i.e. whether UE is ready to receive the next SSB.

In Figure 3, if the DCI command is sent timely after the last SSB burst, the UE has sufficient time to receive the new SSB burst when SSB periodicity is getting longer. Thereagainst in Figure 4, if the DCI command is not sent timely, the UE cannot prepare for cancelling the next (=first) SSB reception and hence will still attempt to receive it without success. In this case, UE will need to process the group DCI command 2_9 and will detect the new indicated SSB periodicity, thus will be ready to receive the next (= first) SSB. This means Tfirst_SSB depends on whether the group DCI command 2_9 including the SSB adaptation indication is timely received, i.e. whether UE is ready to receive the next SSB with the longer SSB periodicity.

It is noted, the above timeline is based on the assumption that the new SSB periodicity, indicated in the group DCI command 2_9 is already applied between the last SSB before the SSB adaptation command and the next SSB after the SSB adaptation command. RAN1 is about determining Tfirst_SSB . If RAN1 does not proceed this way, the above case distinction does not apply.

Proposal 4: 	For defining L1 transition requirements, RAN4 should consider the following use case scenarios with SSB adaptation command received sooner or later in the interval between two SSB bursts and taking into account the SSB adaptation delay consisting of d plus any additional processing time Tprocessing:

-	Scenario 1: SSB adaptation to lower periodicity with early SSB adaptation command received after completed L1 measurement period.

-	Scenario 2: SSB adaptation to lower periodicity with late SSB adaptation command received after completed L1 measurement period.

-	Scenario 3: SSB adaptation to higher periodicity with early SSB adaptation command received after completed L1 measurement period.

-	Scenario 4: SSB adaptation to higher periodicity with late SSB adaptation command received after completed L1 measurement period.

Based on RAN1’s definition for Tfirst_SSB, RAN4 needs to discuss whether and how to define L1 transition requirements for the case the SSB adaptation command is timely received after the last SSB burst and for the case it is not timely received. RAN4 should also distinguish the cases when SSB adaptation command is received after completion of the L1 measurement period and during the L1 measurement period.

Proposal 5: RAN4 to discuss whether to define transition requirements for timely/non-timely reception of SSB adaptation command and for reception of SSB adaptation after completion / during L1 measurement period.

In case the SSB adaptation command is received during the L1 measurement period, it is proposed to restart the L1 measurement being based on samples with the new SSB burst periodicity.

Proposal 6: In case the SSB adaptation command is received during the L1 measurement period, it is proposed to restart the L1 measurement being based on samples with the new SSB burst periodicity.

  - Layer 3 measurement impact

RAN4 #112bis sent LS to RAN2 [4] asking about the impact on SMTC configuration due to SSB adaptation. The RAN2 reply was received at RAN4#114bis meeting [5] and is depicted below. L3 measurement impacts for deactivated SCell and activated SCell are considered subsequently. Also, the impact due to colliding MG occasion is considered. Finally, L3 measurement impacts in transition periods are investigated.

## RAN2 Reply LS on SMTC configuration

RAN2#129 sent LS to RAN4 [5] replying to RAN4’s LS [4] on SSB adaptation related to SMTC configuration for SSB adaptation. The agreement is replicated below.

Agreements on SSB adaptation

- RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

- For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

This agreement means that further additional SMTC configuration(s) may be configured by the network based on the adapted SSB periodicity being used for serving cell measurements in SCell, whilst neighbor cell measurements in SCell will be based on legacy SMTC. RAN2 did not provide further details on the additional SMTC configuration(s) for serving SCell to RAN4. However, RAN2#129bis agreed to configure multiple additional SMTC configurations via RRC, one per each SSB periodicity. With this understanding, RAN4 can proceed defining RRM impacts due to additional SMTC configurations configured to UE.

Proposal 7: 	RAN4 to proceed defining RRM impacts due to additional SMTC configurations (up to 2, each indicating another SSB periodicity) for serving SCell configured to UE.

The RAN2 agreement also means that there is no impact on L3 neighbor cell measurements, since the legacy SMTC applies and thus, only serving cell measurements in SCell are subject to SSB adaptation, whilst neighbor cell measurements are based on legacy SMTC. Hence L3 measurement impact has to be distinguished for activated SCell / deactivated SCell measurements.

## L3 measurement impact to deactivated SCell

For deactivated SCell measurements, RAN4 #114 achieved following agreement:

Issue 1-3-1: L3 requirements for SSB adaptation – general Agreement: Deactivated SCell measurement: - Option 1: For SSB adaption, reuse the same conclusion from OD-SSB case#2 (scenario#4) discussion to define RRM requirements. - At least for the case when the SSB periodicity is adapted to be shorter than before - Option 2: Same as legacy - Other options are not precluded. The requirement can be updated based on further progress in other WG or other aspects are identified that the conclusion can not be directly reused.

The issue was not concluded at RAN4 #114bis [3].

Issue 1-3-1: L3 requirements for SSB adaptation Agreement: - RAN4 to define requirements based legacy L3 measurement framework for SSB adaptation. - For neighbour cell L3 measurement, legacy requirements apply. Open issue for further discuss: - For SCell with SSB adaptation (deactivated SCell or activated SCell), wait for more RAN2 progress on additional SMTC configuration, and discuss following aspects: - Whether to define L3 measurement requirement at transition. - Any other RRM requirements impacts due to additional SMTC configurations.

Legacy requirements for L3 measurements of deactivated SCells in RRC_CONNECTED state are contained in clauses 9.2.5 and 9.2.7, being specified under intra-frequency requirements. This includes the case that NCD-SSB based L3 measurements are used in the SCell, if the NW indicates it as reference SSB.

RRM impacts exist for cell identification of unknown cells (PSS/SSS detection, time index detection) and cell measurements for known cells, both for measurements without gaps (from Rel-15) and with NCSG (from Rel-17) as the measurement period is proportional to the measCycleSCell value up to a defined limit.

In all cases, the measurement period is depending on the parameter measCycleSCell, configurable between 160 ms and 1280ms, hence it is greater or equal to the largest existing SSB periodicity (160ms). Thus, there is no misconfiguration possible between SSB periodicity and measCycleSCell. In addition, these measurements will cause interruptions in the serving cell, either unknown (of measured without gaps) or known (if measured with NCSG). Higher number of these interruptions may result in more required retransmissions in the PCell which should be minimized. Hence it is recommended to stay with the legacy UE behavior for deactivated SCell measurements, as throughput in serving cell may be degraded due to caused interruptions and both UE / network energy savings may be compromised in case of lower SSB periodicity after adaptation.

Proposal 8: 	RAN4 to define measurement requirements for deactivated SCell in case of SSB adaptation reusing the legacy requirements for measurements without gaps or with NCSG.

## Collision of SSB measurements with MG occasion

Related to OD-SSB, the scenario of OD-SSB transmissions within FMW colliding with MG occasion has been considered at RAN4#114bis [6].

Sub-issue 3: OD-SSB in FMW collided with MG Wayforward - FFS UE is allowed to skip the OD-SSB measurement within FMW if the OD-SSB occasion in FMW collides with MG occasion.

The same issue is present for SSB adaptation in connected mode, if the SMTC occasion of the selected additional SMTC configuration (like the SMTC occasion for the legacy SMTC configuration) collides with MG occasion. In this case, the UE should be allowed to skip measurements in the overlapping SMTC occasion of serving SCell.

Proposal 9: 	RAN4 to agree, if the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell.

## L3 transition requirements for activated SCell

RAN4#114bis did not further discuss the submitted proposals related to L3 transition requirements for activated SCell. The following progress is depicted in [3].

### Sub-topic 1-3 SSB adaptation – L3 Requirements impacts Issue 1-3-1: L3 requirements for SSB adaptation Open issue for further discuss: - For SCell with SSB adaptation (deactivated SCell or activated SCell), wait for more RAN2 progress on additional SMTC configuration, and discuss following aspects: - Whether to define L3 measurement requirement at transition. - Any other RRM requirements impacts due to additional SMTC configurations.

In fact, the LS from RAN2 [5] indicates SSB adaptation is based on the additional SMTC configuration for SCell for serving cell measurements only. It may be used for SCell activation, e.g. to fasten the measurement, and after SCell activation, e.g. to slow down the measurement, until the SCell is deactivated.

If SSB periodicity in SCell is adapted, then measurement period requirements only apply from the first SMTC occasion after reception of the SSB adaptation command, no matter whether the periodicity gets higher or lower.

Proposal 10: 	RAN4 to agree measurement period requirements only apply from the first SMTC occasion after reception of the SSB adaptation command, no matter whether the periodicity gets higher or lower.

  - SCell activation requirements

For measurements related to SCell activation, RAN4 #114 achieved following agreement [6]:

Issue 1-4-1: SCell activation requirement for SSB adaptation Agreement: For SCell activation: - Define the SCell activation requirements based on the SSB periodicity after SSB adaptation. - For the case SSB adaptation command comes together with SCell activation command. The requirement can be updated based on further progress in other WG

RAN4#114bis did not further reach a conclusion on this issue beyond the recommended way forward [2].

- Recommended WF: - RAN4 to discuss the conditions for SCell activation with SSB adaptation (i.e. based SSB periodicity after SSB adaptation), considering following aspects: - SSB adaptation command (e.g. DCI/MAC-CE/RRC), which subject to conclusion from other WG - Time relation between SSB adaptation command and SCell activation command.

For SCell activation in RRC_CONNECTED state, with current requirements specified in TS 38.133, clause 8.3.2, SSB periodicity adaptation to lower SSB periodicity can be used to fasten the serving cell measurements required to perform SCell activation based on lower time of TFirstSSB. This includes the case that NCD-SSB based measurements are used in the SCell, if the NW indicates it as reference SSB.

If the SSB adaptation command comes together with SCell activation command in the RRC message, the SCell activa-tion delay may be somewhat higher due to additional processing for SSB adaptation as part of Tactivation_time, such as

Tactivation_time = TFirstSSB + X, with X>5ms for measurement period lower or equal to 2400ms and known SCell in FR1.

No separate SSB adaptation delay is needed in this case and the lower SSB periodicity, part of the additional SMTC configuration, is immediately used after SCell activation and will fasten L3 measurements in the activated SCell.

Proposal 11: 	RAN4 to define measurement requirements for SCell activation in case of concurrent SCell activation and SSB adaptation command in same RRC message, i.e. based on

Tactivation_time = TFirstSSB + X, with X>5ms for measurement period lower or equal to 2400ms and known SCell in FR1.

- Conclusion

In this contribution, as part of the objective on “adaptation of common signal/channel transmissions” of the Rel-19 WID [1] to further enhance network energy savings for NR, we discussed scenarios for investigating the RRM impact from SSB periodicity adaptation in time domain based on RAN1/2 progress and RAN4#114 agreements and presented our views on the potential RRM impact.

Following observations and proposals are made.

Scenarios for SSB adaptation

Observation 1: The additional SSB periodicities, configured for SSB adaptation, may be longer or shorter than the default SSB periodicity depending on whether legacy UEs are also served in SCell or not.

Proposal 1:	RAN4 to add the scenario of Rel-18 SCell activation with L3 reporting to the applicable SCell activation scenarios with SSB adaptation.

Proposal 2:	RAN4 to exclude multiple SCells activation scenario with SSB adaptation from Rel-19 NES enhancements.

Proposal 3: 	RAN4 to consider specifying RRM requirements for SSB periodicity adaptation for NW triggered group common DCI and RRC signaling methods, designed by RAN1 and RAN2.

Layer 1 measurement transition requirements

Proposal 4: 	For defining L1 transition requirements, RAN4 should consider the following use case scenarios with SSB adaptation command received sooner or later in the interval between two SSB bursts and taking into account the SSB adaptation delay consisting of d plus any additional processing time Tprocessing:

-	Scenario 1: SSB adaptation to lower periodicity with early SSB adaptation command received after completed L1 measurement period.

-	Scenario 2: SSB adaptation to lower periodicity with late SSB adaptation command received after completed L1 measurement period.

-	Scenario 3: SSB adaptation to higher periodicity with early SSB adaptation command received after completed L1 measurement period.

-	Scenario 4: SSB adaptation to higher periodicity with late SSB adaptation command received after completed L1 measurement period.

Proposal 5: RAN4 to discuss whether to define transition requirements for timely/non-timely reception of SSB adaptation command and for reception of SSB adaptation after completion / during L1 measurement period.

Proposal 6: In case the SSB adaptation command is received during the L1 measurement period, it is proposed to restart the L1 measurement being based on samples with the new SSB burst periodicity.

Layer 3 measurement impact

Proposal 7: 	RAN4 to proceed defining RRM impacts due to additional SMTC configurations (up to 2, each indicating another SSB periodicity) for serving SCell configured to UE.

Proposal 8:	RAN4 to define measurement requirements for deactivated SCell in case of SSB adaptation reusing the legacy requirements for measurements without gaps or with NCSG.

Proposal 9: 	RAN4 to agree, if the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell.

L3 transition requirements for activated SCell

Proposal 10: RAN4 to agree measurement period requirements only apply from the first SMTC occasion after reception of the SSB adaptation command, no matter whether the periodicity gets higher or lower.

SCell activation requirements with concurrent SSB adaptation

Proposal 11: 	RAN4 to define measurement requirements for SCell activation in case of concurrent SCell activation and SSB adaptation command in same RRC message, i.e. based on

Tactivation_time = TFirstSSB + X, with X>5ms for measurement period lower or equal to 2400ms and known SCell in FR1.

- References

[1] 	RP-250343, “Revised WID: Enhancements of network energy savings for NR”, Ericsson

[2]	 R4-2503626, “Topic summary for [114bis][226] Netw_Energy_NR_enh_Part2, Moderator (Huawei)”, Moderator 	 (Huawei)

[3]	R4-2504907, “WF on RRM requirements for Netw_Energy_NR_enh_Part2”, Huawei

[4]	R4-2416911, “LS on SSB adaptation”, RAN4, RAN4#112bis

[5]	R2-2501484, “Reply LS on SSB adaptation”, RAN2, RAN2#129

[6] 	R4-2504906, “WF on RRM requirements for Netw_Energy_NR_enh_Part1”, Ericsson

- Annex 1: Previous RAN4 agreements on SSB adaptation

RAN4 has discussed the SSB adaptation enhancement in their last meetings. At RAN4#114bis meeting, the following has been agreed on RRM requirements for SSB adaptation.

At RAN4#114 meeting, the following has been agreed on RRM requirements for SSB adaptation.

- Sub-topic 1-1 SSB adaptation – Scenario Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - Further clarification on the scope of RRM impact for SSB adaption: - No RRM impacts for SSB adaptation requirement for PCell: - Frequency location is same before and after SSB adaptation. - No change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation - Note: Above could be further updated based on further RAN1 progress if any. - Sub-topic 1-2 SSB adaptation – L1 Requirements impacts Issue 1-2-1: Applicable L1 requirements for SSB adaptation Agreement: - No impact of SSB adaptation: - RLM - BFD - Define requirements if SSB adaption happens: - L1-RSRP for serving cell - L1-SINR - CBD - Whether and how to define requirements at transition is discussed in other issues. Issue 1-2-2: Transition requirements applicability & Issue 1-2-3: Transition requirements – details Agreement: - For L1-RSRP/L1-SINR/CBD - The measurement/evaluation period based on SSB periodicity after adaptation apply from Tfirst_SSB. - Tfirst_SSB is the uncertainty time from the timepoint when SSB adaptation occurs till the first SSB after SSB adaptation.

- Sub-topic 1-3 SSB adaptation – L3 Requirements impacts Issue 1-3-1: L3 requirements for SSB adaptation – general Agreement: Deactivated SCell measurement: - Option 1: For SSB adaption, reuse the same conclusion from OD-SSB case#2 (scenario#4) discussion to define RRM requirements. - At least for the case when the SSB periodicity is adapted to be shorter than before - Option 2: Same as legacy - Other options are not precluded. The requirement can be updated based on further progress in other WG or other aspects are identified that the conclusion can not be directly reused. - Sub-topic 1-4 SSB adaptation – Other requirements impacts Issue 1-4-1: SCell activation requirement for SSB adaptation Agreement: For SCell activation: - Define the SCell activation requirements based on the SSB periodicity after SSB adaptation. - For the case SSB adaptation command comes together with SCell activation command. The requirement can be updated based on further progress in other WG Issue 1-4-2: TCI state switching requirements for SSB adaptation Agreement: - No requirements when SSB adaptation happens during TCI state switching. - Sub-topic 2-1 RRM requirements impacts for OD-SIB1 Issue 2-1-1: RRM requirements impacts for OD-SIB1 Agreement: RAN4 to discuss the potential RRM impact for OD-SIB1 from the next meeting.

At RAN4#113 meeting, the following has been agreed on RRM requirements for SSB adaptation.

- Sub-topic 1-1 SSB adapation – Scenario Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - No RRM impacts for SSB adaptation requirement for IDLE mode. - No RRM impacts for Cell DTX. Issue 1-1-2: Scenario clarification- SSB periodicity Agreement: - Postpone the discussion and wait for RAN1 progress. Issue 1-1-3: Scenario clarification- SSB adaptation triggering Agreement: - RAN4 to focus on NW controlled SSB adaptation firstly. Issue 1-1-4: Scenario clarification- SSB adaptation mechanism Agreement: - Postpone the discussion and wait for RAN1 progress. - Sub-topic 1-2 SSB adaptation – L1 Requirements impacts Issue 1-2-1: L1 requirements at transition Agreement: - L1 evaluation/measurement period is the maximum of the period corresponding to the SSB before and after adaptation. - FFS the applicable L1 requirements. Issue 1-2-2: L1 requirements subsequent to “transition period” Agreement: - Subsequent to transition period, requirements apply based on the SSB periodicity after transition for SSB adaptation of periodicity. - Note: The transition period is discussed in other issues. - Sub-topic 1-3 SSB adaptation – L3 Requirements impacts Issue 1-3-1: L3 requirements for SSB adaptation – general Agreement: - Wait for LS reply on whether there is impact on SMTC.

At RAN4#112bis meeting, the following has been agreed on RRM requirements for SSB adaptation.

### Sub-topic 1-1 SSB adapation – Scenario Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - Per RAN1 agreements, RAN4 to take following as starting point for SSB adaption discussion: - CONNECTED mode, and - SSB adaptation for SCell, and - SSB adaptation for NCD-SSB, and - SSB periodicity adaptation in time-domain Note: Above could be further updated based on further RAN1 progress (e.g. PCell, other adaptation mechanism) Issue 1-2-1: Requirements need updating Agreement: - RAN4 to discuss whether there are any impacts on at least the following RRM requirements due to SSB adaption: - L1-RSRP for serving cell - L1-SINR - BFD - CBD - SCell activation - TCI state switching - NR intra-frequency L3 measurement - NR inter-frequency L3 measurement Note: Above list could be further updated based on RAN4 discussion.

At RAN4#112 meeting, the following has been agreed on RRM requirements for SSB adaptation.

- Sub-topic 3-1: SSB Adaptation requirements Issue 3-1-1: SSB adaptation in IDLE/CONNECTED mode - Agreement - RAN4 to prioritize the SSB adaptation discussion in CONNECTED mode. - Whether to discuss SSB adaptation in IDLE mode depends on RAN1 progress Issue 3-1-2: SSB adaptation impact in RRM requirement - Agreement: SSB adaptation impact in RRM requirement - RAN4 to discuss the L1/L3 measurement requirement impact as starting point. - Wayforward: - RAN4 to discuss SSB periodicity adaptation in time-domain as starting point. - Other SSB adaptation mechanism (s) and requirements are not precluded depending on RAN1 progress.

- Annex 2: RAN1 agreements on SSB adaptation

RAN1 has discussed the SSB adaptation enhancement in their last meetings. At RAN1#120bis meeting, the following conclusion and agreements were achieved:

Agreements (RAN1#120bis) Agreement For adaptation of SSB in time-domain, for DCI 2_9-based SSB burst periodicity adaptation for an SCell, - The DCI is scrambled a new RNTI, - Same search space and DCI size as that of cell DTX/DRX DCI if gNB configures both Agreement For DCI 2_9-based SSB burst periodicity adaptation for an SCell - the starting location of the information block for SSB burst periodicity indication for a SCell within the DCI format 2_9 is configured using a new RRC parameter - the length of the information block is given by ceil(log2(1+X)), where UE is configured with X additional SSB burst periodicities for the SCell Agreement For adaptation of SSB in time-domain, UE can be configured with X (<=Xmax) additional SSB burst periodicities for an SCell. - Xmax=2 Working Assumption When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission on a second serving cell, the UE assumes SSB is transmitted on the second serving cell according to the indicated SSB burst periodicity from the beginning of the first slot containing the first [actually] transmitted SSB within the first [possible] SSB burst according to the indicated SSB burst periodicity that is no earlier than the slot m+d of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell [in Table 11.5-1 of TS 38.213]. - FFS: how to determine the first [possible] SSB burst

At RAN1#120 meeting, the following conclusion and agreements were achieved:

Agreements (RAN1#120) Conclusion There is no RAN1 consensus to support the following in Rel-19 - New SSB burst periodicity values other than the legacy values (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms). - New UE trigger - Adapting the transmitted number of SSBs within a SSB burst - Adaptation of the time domain positions of SSBs within a burst Conclusion There is no RAN1 consensus to support time domain adaptation for CD-SSB in Rel-19 for Scell Agreement For adaptation of SSB in time-domain, support the following to adapt SSB burst periodicity for an SCell - UE is configured with SSB burst periodicity using legacy signalling for the SCell - UE is configured with X additional SSB burst periodicity for the SCell - FFS: Value of X - SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity - FFS: Whether there is specification impact - Note: This does not impact the discussion on OD-SSB - For switching the periodicity, down-select between - (MAC-CE) - MAC-CE details for SSB burst periodicity adaptation is up to RAN2. - (DCI) - Alt 1-3: DCI based signalling is used Agreement For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell - Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI] - FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1) Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation

At RAN1#119 meeting, the following conclusion and agreements were achieved:

Agreements (RAN1#119) Agreement Reply to Q1(What is the relation in terms of time location before and after SSB adaptation?): - RAN1 agreed that at least SSB burst periodicity is adapted. - There are no RAN1 agreements to adapt the time location of the SSB burst other than the periodicity but RAN1 is still discussing other options. Agreement Reply to Q2(What is the relation in terms of frequency location before and after SSB adaptation?): - The frequency location is same before and after SSB adaptation. Agreement Reply to Q3(What is the spatial relation before and after SSB adaptation?): - There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation. - At least the case where there is no change to actually transmitted SSBs within a burst before and after SSB adaptation is supported. Further update to be made based on RAN1#119 progress if any. Conclusion There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode).

At RAN1#118bis, the following conclusion was reached:

Agreements (RAN1#118bis) [Adaptation of SSB in time domain] Conclusion​ There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19​. Proposal 2.1.4-rev1​ For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select Option 3, i.e. Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior).​ - No spec impact

At RAN1 #118 meeting, the following agreements were reached:

Agreements (RAN1#118) [Adaptation of SSB in time domain] Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’) Agreement For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select from following options - Option 1: One SSB burst periodicity is configured for the UE and UEs assumes SSB transmissions are not present during Cell DTX non-active period - Option 2: UE assumes SSB transmission with different periodicities during Cell DTX non-active period and during Cell DTX active period - Option 3: Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior) – no spec impact

At RAN1#117 meeting, the following agreement was reached:

Agreement (RAN1#117) For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g., two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB