3GPP TSG-RAN WG4 Meeting #115	R4-2507354

Malta, MT, 19th – 23rd May, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for On-demand-SIB1

Agenda item:	7.27.3.3

Document for:	Discussion

Introduction

In the Rel-19 WID on Enhancements of network energy savings for NR [1], the second objective for core requirements relates to the support for on-demand SIB1 (OD-SIB1). The objective was discussed at RAN4#114bis [2] and agreements were captured in the WF [3]. In this contribution, we continue the discussion on the RRM impact from OD-SIB1.

Discussion

  - Progress status in RAN4

At RAN4 #114bis, the following has been agreed on RRM requirements for On-demand SIB1 [3].

Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption Way forward: - Option A: Clarify that paging interruption includes UL WUS transmission and RAR reception for OD-SIB1 acquisition. (HW, OPPO, Apple, Nokia, Xiaomi, CATT, MTK, vivo) - Option B: No impacts on core requirements. Discuss whether to extend TSI-NR in test case. (CMCC, QC, LGE) - Option C: No RRM impacts on the requirements, i.e., UL-WUS can be postponed during the paing time. (E///, LGE, MTK) Issue 2-1-2: RRM requirements impacts for OD-SIB1 – timing Agreement - Clarify in transmit timing requirements to include UL WUS transmission (PRACH) for OD-SIB1: - reference cell is the NES cell - NTA for UL-WUS is defined as 0 - NTA offset is the [n-TimingAdvanceOffset] from UL-WUS configuration if configured, if not, follow the default value defined in Table 7.1.2-2 TS 38.133 - Note: The IE can be updated based on other WG’s progress.

  - RRM impacts for OD-SIB1

In this section we consider potential RRM impacts for OD-SIB1 for idle/inactive RRC states as discussed during RAN4 #114bis [2].

## Paging interruption

Impact to paging interruption due to longer time for cell reselection was raised at RAN4 #114. In fact, SIB1 is not regularly present in the NES cell supporting OD-SIB1, and hence it must be activated in order to identify whether the NES cell is available or is barred for the UE.

Current requirements for paging interruption in idle mode are contained in TS 38.133, clause 4.2.2.6 and are replicated below.

#### 4.2.2.6 Maximum interruption in paging reception UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. In addition, when the UE is configured with eDRX_IDLE cycle, the UE shall not miss any paging in a PTW provided the paging is sent in at least 2 DRX cycles before the end of that PTW. At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms. Ttarget_cell_SMTC_period is the periodicity of the SMTC occasions configured for the target NR cell. If the target cell is in the PCI list of smtc2-LP, the SMTC periodicity follows smtc2-LP; otherwise, the SMTC periodicity follows smtc. At inter-RAT cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels for paging reception of the target inter-RAT cell. For NR to E-UTRAN cell re-selection the interruption time must not exceed TSI-EUTRA + 55 ms. TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell. TSI-EUTRA is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 36.331 [16] for an E-UTRAN cell. These requirements assume sufficient radio conditions, so that decoding of system information can be made without errors and does not take into account cell re-selection failure.

In case of OD-SIB1, the delay to activate the SIB1 in the NES cell needs to be added to the allowed paging reception interruption when UE is performing cell reselection to a cell supporting OD-SIB1. The additional delay is composed of

- the delay for UL-WUS transmission and possible retransmission using PRACH, and

- the delay in receiving and decoding RAR, and

- the delay for receiving OD-SIB1, as indicated in the UL-WUS and by the start of the RAR window, after RAR decoding.

In case of OD-SIB1, SIB1 needs to be activated first and then UE is able to receive all the other relevant system information from the NES cell. During cell reselection UE is not available for paging in the serving cell while it is requesting OD-SIB1 in the NES cell, receiving OD-SIB1 and receiving other necessary SIBs for paging reception. Hence paging interruption will be extended in case of OD-SIB1 due to above factors 1) and 2) versus legacy case.

Proposal 1: 	RAN4 to consider impacts to paging interruption during cell reselection to an OD-SIB1 cell due to additional delays for 1) UL-WUS transmission and 2) reception of RAR, versus legacy case.

At RAN4 #114bis, some companies argued that paging reception should be prioritized over OD-SIB1 request.  However, when cell reselection to candidate NES cell is triggered depends on radio conditions of UE and hence may occur at any time. In addition, the collision case of receiving paging occasions in serving cell (Cell A) and concurrently receiving SIB1 and other SIBs in NES cell, may occur and UE behavior needs to be clarified here in specifications. In our view a longer paging interruption as described in Proposal 1 above should be specified to cover all scenarios.

## Cell reselection delay

In idle mode requirement, the time for cell reselection is specified as the delay within which the UE shall be able to evaluate whether a cell meets the reselection criteria e.g. in TS 38.133 clause 4.2.2.3 for intra-frequency NR cells as cited below.

TS 38.133 clause 4.2.2.3: #### 4.2.2.3 Measurements of intra-frequency NR cells The UE shall be able to identify new intra-frequency cells and perform SS-RSRP and SS-RSRQ measurements of the identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities. The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS 38.304 [1] within Tdetect,NR_Intra when that Treselection= 0. An intra frequency cell is considered to be detectable according to the conditions defined in Annex B.1.2 for a corresponding Band. The UE shall measure SS-RSRP and SS-RSRQ at least every Tmeasure,NR_Intra (see table 4.2.2.3-1, table 4.2.2.3-2, table 4.2.2.3-3, table 4.2.2.3-4 or table 4.2.2.3-5) for intra-frequency cells that are identified and measured according to the measurement rules. The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Intra/2. The UE shall not consider an NR neighbour cell for cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

When OD-SIB1 cell is considered, the cell reselection is considered completed according to existing rules and in addition if OD-SIB1 is successfully obtained in the NES cell. RAN4 needs to discuss if the cell reselection delay shall be extended due to UL-WUS procedure when UE performs cell reselection to an OD-SIB1 cell.

Proposal 2: 	RAN4 to discuss if the cell reselection delay shall be extended due to UL-WUS procedure when UE performs cell reselection to an OD-SIB1 cell.

## UL-WUS Msg1 retransmission attempts

Another RRM impact which was raised during RAN4 #114 [2] is whether a minimum of UL-WUS Msg1 retransmissions per paging cycle should be considered. In such case, UL-WUS Msg1 retransmission(s) would be performed in case UE does not receive a RAR. RAN1 has sent a list of RRC parameters for UL-WUS transmission which comprise a maximum number of UL-WUS retransmissions preambleTransMax [4]. Thereagainst, UL-WUS repetitions were already excluded by RAN1.

On one side, this makes the UL-WUS transmission more robust in the NES cell, on the other side it also increases the paging interruption time discussed in section 2.2.1. In our view, priority may be given either to better coverage or to lower delay, which is then up to network decision. Hence RAN4 should investigate RRM impacts for both scenarios.

Proposal 3: 	RAN4 to consider the additional RRM impact for one or more UL-WUS Msg1 retransmission attempts in the NES cell, if configured.

- Conclusion

In this contribution, as part of the second objective to “Specify support for on-demand SIB1 for UEs in idle/inactive mode” of the Rel-19 WID [1] to further enhance network energy savings for NR, we discussed RRM impacts for OD-SIB1.

Following proposals are made.

Paging interruption

Proposal 1: 	RAN4 to consider impacts to paging interruption during cell reselection to an OD-SIB1 cell due to additional delays for 1) UL-WUS transmission, 2) reception of RAR, 3) time for SIB1 reception.

Cell reselection delay

Proposal 2: 	RAN4 to discuss if the cell reselection delay shall be extended due to UL-WUS procedure when UE performs cell reselection to an OD-SIB1 cell.

UL-WUS Msg1 retransmission attempts

Proposal 3: 	RAN4 to consider the additional RRM impact for one or more UL-WUS Msg1 retransmission attempts in the NES cell, if configured.

- References

[1] 	RP-250343, “Revised WID: Enhancements of network energy savings for NR”, Ericsson

[2]	 R4-2503626, “Topic summary for [114bis][226] Netw_Energy_NR_enh_Part2, Moderator (Huawei)”, Moderator 	 (Huawei)

[3]	R4-2504907, “WF on RRM requirements for Netw_Energy_NR_enh_Part2”, Huawei