3GPP TSG-RAN WG4 Meeting #115	R4-2507364

Malta, MT, May 19th –May 23rd, 2025

Source:	Nokia

Title:	On the measurement framework for the new IOT NTN TDD Punctured frame

Agenda item:	7.32.6

Document for:	Discussion

Introduction

In the plenary meeting RAN #105, a new WI for Rel-19 was approved to introduce a TDD frame operation for NTN [1]. The work item is designed to provide extended support of 3GPP NB-IoT features leveraging existing satellite systems deployed in Non-Geostationary Satellite Orbit (NGSO). It also aims at enabling low-complexity payloads for satellite operations that are non-reliant on a diplexer.

To achieve the aforementioned goals, the WID was designed to introduce an NTN-compatible TDD frame, that while based on minimum necessary changes on the NB-IoT NTN FDD specification provides a new approach to TDD periodicity.  RAN4 has analysed the potential impacts of such frame and provided their view on [2].

In the plenary meeting RAN #106, the WID was updated to provide the objectives for RAN4 based on identified challenges [3]. In this document we discuss the potential RRM impacts.

Discussion

The measurement framework for the newly proposed IOT NTN TDD punctured frame pattern was extensively discussed in the previous RAN4 meeting. There are multiple different views and objects of discussion related to this Work Item, which did not allow further agreements to be made. In light of such situation, the moderator has proposed in the WF [4] a set of questions to be addressed by the companies to set the common ground for discussion:

Agreement: Further discuss the following questions in the next meeting: - Q1: Is the measurement/evaluation solely based on NPSS, NSSS or NRS? - Q2: How many samples of the reference signal(s) were needed in a normal coverage scenario with SNR ≥ [-6, -1] dB? - Q3: Is the UE required to monitor the reference signals during DL NB-IoT subframes that are in the middle of ongoing UL transmissions? Question Q1; Question Q2; Question Q3. Issue 1: Cell reselection (4.6A.2). Issue 2: Cell measurement in connected mode (8.14A). Issue 3: RRC re-establishment (6.5A). Issue 4: RRC Connection redirection to Non-Anchor Carrier (6.9A). Issue 5: RLM (7.23A).

In Table 1we provide our views as proposed by the moderator. The answers for Q2 are partially based on the discussion we provided in our previous contribution and repeated here for convenience.  In what regards the number of samples needed for PSS and SSS detection, the current requirements were developed based on simulation results back in Release 13. For this case the number of DRX cycles used to define the measurement requirements assumed that the number of instances of NPSS and NSSS per DRX cycle is equal to [5] :

  - DRX Cycle 1.28s: 5

  - DRX Cycle: 2.56s: 10

  - DRX Cycle 5.12s: 10

  - DRX Cycle: 10.24s: 20

In the 1.28 s cycle, there are at least 14 occasions of DL transmission where the UE might be capable of acquiring the NPSS and NSSS, which is more than the minimum assumption above. Assuming this is the most stringent case, then, the number of NPSS and NSSS samples available per DRX cycle is consistent with the previous assumption.

Moreover, the same reference [5] also provide simulation results for -3dB and 0 dB which provide very similar results and might be used as reference to establish the number of samples needed for -1 dB which was agreed to be the side condition for the new frame structure.

Table 1 Evaluation of the measurement channels under punctured frame structure

|  | Question Q1 | Question Q2 | Question Q3 |
| --- | --- | --- | --- |
| Issue 1: Cell reselection (4.6A.2) | NRS (With side conditions related to NPSS and NSSS) | Specification says “at least” 2 measurements per filtered sample. Current values can be maintained. |  |
| Issue 2: Cell measurement in connected mode (8.14A) | Tdetect is based on NPSS and NSS Tmeasure is based on NSSS or NRS | NPSS: 85 for -6 dB; 35 for -1 dB NSS: 35 for -6dB; 5 for -1 dB |  |
| Issue 3: RRC re-establishment (6.5A) | NPSS/NSSS | NPSS: 85 for -6 dB; 35 for -1 dB NSS: 35 for -6dB; 5 for -1 dB |  |
| Issue 4: RRC Connection redirection to Non-Anchor Carrier (6.9A) |  |  |  |
| Issue 5: RLM (7.23A) | NRS | Specification says “at least” 2 measurements per filtered sample. Current values can be maintained. |  |

Take the Table above as input for the RAN4 discussions.

Question 3 seems more generic and we see no need to reply it separately for the different clauses. In our understanding, it is possible to measure the DL signals that occur in between consecutive sets of 8 consecutive UL slots where UE is performing UL transmissions, and the UE is required to do so.

From RAN4 point of view, the UEs are expected to perform measurements in the DL subframes that occur in between sets of UL subframes where UE is performing active transmission.

Issue 1: Cell Reselection

For the cell reselection the current open issue was left for discussion in the way forward:

Agreement (online): For the cell reselection for TDD NB-IoT in NTN, - The requirements are applicable only if UE is provided with the active periods of the neighbour cells configured for the measurements. - For earth fixed cell and earth moving cell: - The existing values for Tdetect, Tmeasure, and Tevaluate in Table 4.6A.2.2-1 and Table 4.6A.2.2-2 can be applicable if it is confirmed that the current requirement is achievable without requiring more than one NPSS/NSSS sample for half the DRX cycle length [due to the TDD pattern] - Further discuss the values if -1dB will be used. Ksatellite = 1 for the cells measured in DL opportunities of a neighbour satellite that are in non-overlapping with DL opportunities to be measured in the serving satellite Further discuss the following: - For earth moving cell: - Option 2-A: The existing values for Tdetect, Tmeasure, and Tevaluate in Table 4.6A.2.2-1 and Table 4.6A.2.2-2 can be applicable if it is confirmed that the current requirement is achievable without requiring more than one NPSS/NSSS sample for half the DRX cycle length - Option 2-B: Re-evaluate the maximum latency in cell detection, measurement, and evaluation required for this scenario, as well as the minimum number of NPSS, NSSS, and NRS samples needed for cell detection and measurement at an SNR of [-1] dB, and enhance Tdetect, Tmeasure, and Tevaluate in Table 4.6A.2.2-1 and Table 4.6A.2.2-2.

Based on our analysis provided in Table 1 and on the discussion above, where we indicate that, in a given number of DRX Cycles (see Table 4.6A.2.2-1) the number of PSS/SSS opportunities is maintained and although in smaller number the number of NRS symbols is enough to acquire multiple versions of the signal to enable L1 and L3 measurements, we propose that:

Confirm the new side condition of -1 dB for both earth moving and earth fixed cell. And re-use the values for Tdetect, Tmeasure and Tevaluate from Release 17/18.

However, it was raised during the meeting that some companies are concerned that although the number of samples assumed to be measured within a DRX cycle is preserved compared to the legacy reference, the total number of opportunities available to obtain such samples is reduced. This means that the UEs shall wake up for a higher percentage of the PSS/SSSoccasions. Besides, if the UE has to measure across multiple DL opportunities, the total wake time will be much larger. Therefore, when the UE has to measure cells across multiple DL opportunities (see Figure 1), the UE will be awake for most of the time just to collect measurements of the cells to be measured, which goes against the principle of NB-IoT to limit power consumption. Because of this, as a compromise, we can scale the measurements when the UE has to measure 3 or more different DL opportunities, in order to give the UE more chances to save power and space its measurements in time.

Scale Tmeasure and Tdetect by a factor of [2], when the UE measures cells across [3] or more different DL opportunities.

Issue 2: Cell measurements in connected mode

For the cell measurements in connected mode, the current way forward states that:

Issue 2: Cell measurement in connected mode (8.14A) Further discuss the following: For the cell measurement period in connected mode for TDD NB-IoT in NTN: - Requirements are applicable if the UE is provided with assistance information on the DL periodic patterns of the cells to be measured. - For intra-frequency measurement for both cases where DRX is not configured and is configured, - Option 1-A: Max {existing values, the time period for the UE to obtain the same number of NPSS and NSSS samples as those assumed for the existing requirements} - Option 1-B: Delay requirements are scaled by a factor of [2 or 9] - FFS on side condition, e.g. only when the UE measures cells across 3 or more different patterns of DL opportunities - FFS for earth moving cell - For inter-frequency measurement for both cases where DRX is not configured and is configured, - Option 2-A: Max {existing value, periodicity of corresponding RS} - Option 2-B: Reused the existing Tmeasure and Tdetect - The minimum length of measurement occasions is [82] ms - FFS for earth moving cell

For intra-frequency measurements, in legacy specification, Tdetect and Tmeasure are given by:

|  | No DRX Cycle | 0.256<DRX-cycle≤10.24 |
| --- | --- | --- |
| Tdetect | 1400 ms | 6 DRX Cycles |
| Tmeasure | 800 ms (NRS-based) or 1600 ms (NSS-based) | 5 DRX Cycles |

In our analysis above, the number of PSS samples needed for the current side condition (-1 dB) is equal to 35 and the number of SSS samples required is equal to 5. Those numbers are significantly smaller than the previous values (85 and 35, respectively). However, the periodicity of both symbols has been enlarged by a factor of 9 (PSS went from 10 to 90 ms and SSS from 20 to 180 ms). This means that within 1400 ms, there will be 7 SSS opportunities and 15 PSS opportunities. In order to keep it consistent with the values aforementioned, it would be needed to increase the total time at least to 3150 ms (to allow for 35 PSS samples to be collected). For consistency, the value of Tmeasure shall also be enlarged for the case based on NSSS measurements.

When no DRX is configured, for the new TDD punctured pattern, the value of Tmeasure and Tdetect shall be 3200 ms.

For the cases where DRX is implemented, we believe the best approach is to avoid the legacy definition based on number of DRX cycles regardless of the DRX Cycle length. Also based on the analysis above, we have established that for DRX Cycle length of 1.28 ms, 5 PSS and SSS samples are considered to be available. (10 samples is available for DRX Cycle length equal to 2.56 and 5.12 and 20 samples available for DRX Cycle length of 10.24). Based on such numbers we propose:

When DRX is configured, for the new TDD punctured pattern, the value of Tdetect for intra-frequency measuremetns shall be:

- 20 DRX Cycles for 0.256 s<DRX-cycle<1.24 s

- 7 DRX Cycles for 1.24≤DRX-cycle<2.56

- 4 DRX Cycles for 2.56≤DRX-cycle<5.12

- 3 DRX Cycles for 5.12≤DRX-cycle

When DRX is configured, for the new TDD punctured pattern, the value of Tmeasure for intra-frequency measuremetns shall be one DRX Cycle shorter than the value of Tmeasure.

Besides, based on the same reasoning aforementioned for idle mode mobility, when the UE has to measure across multiple DL opportunities the measurements can be scaled:

Scale Tmeasure and Tdetect  by a factor of [2], when the UE measures cells across 3 or more different patterns of DL opportunities.

Inter-frequency measurements

For the inter-frequency measurements, requirements were already established based on the number of samples to be collected (factors N and M in the respective formulas for time to detect and time to measure). Therefore, they scale naturally with the time for the UE to collect the samples. So it is natural that assuming that the number of samples do not need to be enlarged, we propose that:

Do not further scale Tmeasure and Tedetect for inter-frequency measurements due to the punctured pattern

For the length of the measurement occasions, the pre-conditions for the requirements applicability also establish that:

- Length of MOdetect_inter_NB1-NC is at least 200 ms. - Length of MOmeasure_inter_NB1-NC is at least 50 ms.

For the measurement window opportunities of the time to detect, we believe it can be made smaller than 200ms. As there are more well defined opportunities for measuring in the new frame pattern (due to inactivity in the serving cell), it might be useful to establish smaller MO to avoid overlapping with the DL opportunities of the serving cell, while allowing for measurements at DL opportunity the UE sees fit. One potential approach is to have the minimum length of measurement equal to the interval between the end of one DL occasion X (X=1,2,3 or 4) to the start of the next DL occasion X (90 ms – 8 subframes = 82 ms).

For inter-frequency measurement, the minimum length of measurement occasions is [82] ms.

Issue 3: RRC Re-establishment

The discussion for RRC Re-establishment is related to the number of PSS and SSS samples available for the UE within a period of time. As discussed in section 2.2 of the present document, the total time of 1400 ms shall be enlarged to 3200 ms to accommodate the number of samples to be collected

The search time per frequency layer for the RRC Re-establishment shall be 3200 ms per frequency layer.

Issue 4: RRC Connection Redirection to non-anchor carrier

There is one additional issue discussed in the previous WF that needs addressing. It is related to the DL-UL switch times in the RRC connection redirection.

## Issue 4: RRC Connection redirection to Non-Anchor Carrier (6.9A) Agreement (online): For the latency requirement (TRRC_procedure_delay + Tperiod_DL_bitmap + TUL_grant + TDL-UL switch + TNPUSCH) of RRC Connection redirection to Non-Anchor Carrier for TDD NB-IoT in NTN, - No update to TRRC_procedure_delay, TUL_grant, and TNPUSCH - Update the following parameters - Tperiod_DL_bitmap: to be updated based on RAN1 design - TDL-UL switch: to be updated based on RAN1 design

The discussion below was presented for the first time in our previous contribution and is repeated hereby:

The requirements for RRC Connection Redirection to Non-Anchor Carrier establish the maximum delay in time for the UE to complete the transition (RRC Redirection) to the Non-Anchor Carrier upon receiving the reconfiguration indication through the IE “CarrierConfigDedicated-NB”.  This delay depends on several components, as described below:

6.9A.2 Requirements The UE shall be capable of performing the RRC connection redirection to the non-anchor carrier within Tconnection_redirect_non-anchor. The time delay (Tconnection_redirect_non-anchor) is the time between the end of the last subframe in the repetition period of NPDSCH containing the IE, “CarrierConfigDedicated-NB” received on the anchor carrier and the end of the last subframe in the repetition period of NPUSCH transmitted on the target non-anchor carrier. The time delay (Tconnection_redirect_non-anchor) shall be less than: Tconnection_redirect_non-anchor = TRRC_procedure_delay + Tperiod_DL_bitmap + TUL_grant + TDL-UL switch + TNPUSCH - TRRC_procedure_delay: It is the RRC procedure for processing the received message “CarrierConfigDedicated-NB”. It shall be less than 110 ms. - Tperiod_DL_bitmap: It is the periodicity of the downlink subframe configuration for downlink transmission on the non-anchor carrier. It is configured via IE DL-Bitmap-NB [2] and can be 10 ms or 40 ms. TDL-UL switch: It is the time between the end of the last subframe in the repetition period of NPDCCH received on the non-anchor carrier and the start of the first subframe in the repetition period of the corresponding NPUSCH transmitted on the non-anchor carrier. TDL-UL switch is 8 ms. - TNPUSCH: It is the time required to transmit NPUSCH on the non-anchor carrier. The value of TNPUSCH depends on the number of repetitions of NPUSCH used in the non-anchor carrier. When the NPUSCH ACK transmission for the received RRC message takes longer than 110ms, the overall RRC connection redirection delay may be extended.

As we highlight above, two of the parameters need to be revisited during this WI, as they are impacted by the periodic pattern introduced.

The Tperiod_DL_bitmap defines the periodicity of the DL subframe configuration on the non-anchor carrier. It could be previously configured for 10 or 40 ms. However, in the current WI, due to periodic patterns with cycle periods of 9 radio frames, the “off time”  of a cell might be close to 90 ms. Therefore, the value of the parameter requires updating.

Update Tperiod_DL_bitmap to a value compatible with a period of 90 ms.

Also the TDL-UL switch  which measures the time between the end of the last subframe in the repetition period of the NPDCCH received on the non-anchor carrier and the start of the first subframe in the repetition period of the NPUSCH transmitted on the non-anchor carrier requires updating. Currently the delay value is set to 8 ms. But in practice this value will be much longer than 8 ms.

In fact, regarding the DL-UL switch, RAN1 has reached a significant agreement that might be used as a baseline for establishing the new requirement:

Agreement The guard period between the end of the downlink subframes and the beginning of the uplink subframes is fixed in specifications to be 50ms at the ULSRP. - NOTE: This roughly corresponds to the largest separation between DL and UL Iridium slot pairs. - Note: Adjustments to align with different UL/DL pair can be achieved by adjusting the common TA.

Update TDL-UL switch to a value equal to the guard period between DL-UL  subframes to be introduced in specifications by RAN1.

On the applicability of the requirements

The 3GPP specification shall thrive to be future proof. Most of the discussions provided in this WI are dependent on the punctured TDD frame (see Figure 1). It has to be very clear in the specification that these requirements are not applicable for any TDD frame (as in the future new TDD frame structures might be approved for specification). It is strictly related to the frame structure where there is a period of 90 ms, with DL and UL occasions equal to 8 ms.

Capture in specification clearly that the new requirements are applicable only for the new punctured TDD Frame structure where N=9 and D=U=8, as described in the WID.

In the future, if different TDD patterns are applied (for example, non-punctured TDD pattern) the discussions of this WI might not be applicable, and a different set of requirements might be needed.

Figure 1

Reply LS to R1-2503142

RAN1 has sent a LS to RAN4 to discuss the potential implications of the adoption of multiple UL repetition (with segmentation) when operating under the new punctured frame structure.

RAN1 has asked RAN4 to clarify whether there are feasibility concerns with the following working assumption:

Working assumption For precompensation, from RAN1 perspective: - The UE adjusts its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes. - FFS: Whether it is supported to perform segmented pre-compensation within the set of 8 consecutive uplink subframes, and whether in this case a pre-compensation gap is needed. - FFS: whether spec impact is in RAN1, RAN4 or both

From our point of view, the UE has significantly more time to complete the time adjustments between two consecutive UL occasions (bursts of UL subframes) then it had before. Therefore, there should be no issue to adjust the timing before the beginning of each set of consecutive 8 uplink frames and there is no need to apply the pre-compensation gap.

For the case where the segmented pre-compensation is applied within the 8 subframes, the pre-compensation gap might be needed when the timing advance is increasing over time, as the UE will only be capable of adjusting its time for the next segment transmission if it doesn’t overlap with unfinished transmissions of the previous segment. There is no feasibility concerns with this approach.

RAN4 to indicate to RAN1 that:

  - There are no issues with UE performing timing adjustments before the beginning of each set of consecutive 8 uplink subframes and no pre-compensation gap is needed in this case.

  - If the segmentation takes place within the set of 8 consecutive subframes, pre-compensation gaps are needed when the timing advance is increasing over time.

Conclusion

In this paper we discussed the TDD punctured pattern for IoT NTN UEs. The following Observations and Proposals were made:

Proposal 1: Take the Table above as input for the RAN4 discussions.

Proposal 2: From RAN4 point of view, the UEs are expected to perform measurements in the DL subframes that occur in between sets of UL subframes where UE is performing active transmission.

Proposal 3: Confirm the new side condition of -1 dB for both earth moving and earth fixed cell. And re-use the values for Tdetect, Tmeasure and Tevaluate from Release 17/18.

Proposal 4: Scale Tmeasure and Tdetect by a factor of [2], when the UE measures cells across 3 or more different DL opportunities.

Proposal 5: When no DRX is configured, for the new TDD punctured pattern, the value of Tmeasure and Tdetect shall be 3200 ms.

Proposal 6: When DRX is configured, for the new TDD punctured pattern, the value of Tdetect for intra-frequency measuremetns shall be:

Proposal 7: When DRX is configured, for the new TDD punctured pattern, the value of Tmeasure for intra-frequency measuremetns shall be one DRX Cycle shorter than the value of Tmeasure.

Proposal 8: Scale Tmeasure and Tdetect  by a factor of [2], when the UE measures cells across 3 or more different patterns of DL opportunities.

Proposal 9: Do not scale Tmeasure and Tedetect for inter-frequency measurements.

Proposal 10: For inter-frequency measurement, the minimum length of measurement occasions is [82] ms.

Proposal 11: The search time per frequency layer for the RRC Re-establishment shall be 3200 ms per frequency layer.

Proposal 12: Update Tperiod_DL_bitmap to a value compatible with a period of 90 ms.

Proposal 13: Update TDL-UL switch to a value equal to the guard period between DL-UL  subframes to be introduced in specifications by RAN1.

Proposal 14: Capture in specification clearly that the new requirements are applicable only for the new punctured TDD Frame structure where N=9 and D=U=8, as described in the WID.

Proposal 15: RAN4 to indicate to RAN1 that:

a.	There are no issues with UE performing timing adjustments before the beginning of each set of consecutive 8 uplink subframes and no pre-compensation gap is needed in this case.

b.	If the segmentation takes place within the set of 8 consecutive subframes, pre-compensation gaps are needed when the timing advance is increasing over time.

- References

- RP-242415, “New WID on introduction of IoT-NTN TDD mode”, September 2024

- R4-2504915,  WF on RRM requirements for IoT_NTN_TDD,   Moderator (Qualcomm Incorporated),  R4-114bis, Wuhan, Hubei, China, 07th – 11th April, 2025

- RP-243293, “Revised WID on introduction of IoT-NTN TDD mode”, Iridium Satellite LLC, RAN#106, December 2025

- R4-2504915, On the measurement framework for the new IOT NTN TDD Punctured frame, Qualcomm

- R1-2409038, “Feature lead summary #1 on IoT-NTN TDD mode” , Qualcomm, RAN1#118bis

- R4-79AH-0138, “Cell detection simulation results”, Nokia, Hong Kong, China, 28rd – 30th June 2016