3GPP TSG-RAN WG4 Meeting #115						R4-2507375

Malta, MT, 19th – 23rd May, 2025

Source:	Nokia

Title:	Discussion on RF requirements for Fragmented CA

Agenda item:	7.5.4

Document for:	Discussion

Introduction

Agreements at previous meetings

The latest WF for NR FR1 DL fragmented carriers study [1] presents the open issues as found below:

#### Issue 2-3: Triggering condition to enable ”One Rx RF chain”

RAN4 agree there are two aspects regarding the triggering condition enable/disable the one Rx RF chain mode:

- Option 1: Rely on existing channel quality reporting and procedures. No new indication from UE is needed

- Option 2: The UE need the measurement to determine and indicate to the network whether it can support non-contiguous CCs with one RX RF chain or not, assuming the associated degradation is no greater than allowed degradation that is specified

- Note: detailed conditions to enable/disable can be further discussed and decided separately.

#### Issue 2-1: How to indicate UE capability for supporting ”one Rx RF chain” for DL fragmented carriers

- Consider the following alternatives

  - Alternative 1: use the legacy band combination reporting scheme + Rx sharing capability indication

    - Alternative 1 has no modification of CA configurations in RAN4

  - Alternative 1a: use the legacy band combination reporting scheme + Rx sharing capability and other related information including the frequency separation between two carriers.

    - Alternative 1a has no modification of CA configurations in RAN4

  - Alternative 2: use the new notation for CA configurations with single Rx RF Chain

  - Alternative 2a: use the new bandwidth class conditioned on the frequency separation between two carriers

#### Issue 2-2: Discussion on semi-statically switch hardware resources procedure and fallback considerations

- Further discussions on

  - CA fallback with/without UE architecture switching

    - Consider the fall back

      - from high order CA to lower order CA

      - from CA to single carrier mode

    - Decided by network

  - UE architecture switching:

    - Switching between 2 Rx RF chains and 1 Rx RF chain on CCs of intra-band NC CA

    - FFS on whether it is decided by UE itself or decided by network

      - FFS on whether the additional indication from UE side is needed

#### Issue 2-4: Discussion on applicability for PCell and SCell

A more detailed proposal based on the tentative agreement in online discussion:

- To address the potential impacts on the DL performance for PCell, RAN4 agrees to further discuss the following aspects:

  - To limit the use of this feature to CA configurations where the impact is limited

  - In the CA configurations where there is limited impact, how to further mitigate the impact by means including restricting the PCC and SCC frequency location, or restricting UE locations (e.g., the feature is not enabled when the UE is located at cell edge where wanted signal level is close to receiver sensitivity)

And from RAN4#114 [2] presents a bit more details of the open issues as found below:

How to indicate UE capability for supporting ”one Rx RF chain” for DL fragmented carriers

Option 1: Legacy band combination reporting scheme + Rx sharing capability indication

- FFS on whether the capability is specified as ”per-band per-BC” or “per CC per band per BC”.

Other options are not precluded

Discussion on semi-statically switch hardware resources procedure and fallback considerations

Triggering condition to enable ”One RF Rx chain”

FFS:

- FFS on whether the UE indication solution is needed or not, with understanding that enabling one Rx chain is under the control of network.

- Triggering condition for UE working in one Rx RF chain is needed

- Triggering condition for UE working in legacy Rx RF chains is needed

- The switching should be not be dynamic and be under network control with UE indication of condition.

Discussion on applicability for PCell and SCell

Discussion on the number of Rx and DL MIMO layer configuration for the UE supporting ”one Rx RF chain” mode

Discussion

Number of Rx, DL MIMO layer and ”one Rx RF chain”

RAN4 have not concluded whether antennas and DL MIMO layer configuration may play a role in relation to fragmented CA configuration at the UE. The reason why the option is essential is that DL MIMO layers and rank doesn’t cause carrier reduction if lowered, nor are the layers guaranteed optimal conditions, since the rank depends on propagation conditions. Receiving with more antennas than rank provides the UE with diversity gain, but from fragmented carrier operation it is not lack of sensitivity that puts the operation at risk. It is the presence of interference. The trade-off between sensitivity from more than 1 antenna to the resilience towards interference plays a definitive role in discussing fragmented carrier operation.

Definition of Rx RF chain

In [3] we made the observation:

It is not always clear within RAN4 what is meant by “Rx RF Chain”.

Our observation is that there still are some different understandings even a reference architecture has been tried captured in the TR. Hence, we would like to further elaborate our understanding.

Let’s assume a typical 4 antenna UE implementation configured to receive fragmented carriers as illustrated in Fig. 1.

Fig.  1: A 4RX UE implementation example for receiving fragmented carriers with one Rx RF chain.

The design shows that the two fragmented carriers CC1 and CC2 will be received at 4 antennas at the UE. These 4 antennas may connect to 4 modules, with at least one having uplink support. Higher levels of integration of the front-end architecture are not excluded, but the purpose remains: 4 antennas are enabled for receiving 2 carriers through 4 separate receive paths of the front end.

In the example the split of receive chains is considered inside the transceiver chip. Sometimes referred to as split-LNA. This feature allows one Rx path to be split to two separate down-conversion systems. This is actually the Rx RF chain, because this is what we count in relation to this study item. Therefore, looking at the system of Fig 1. In aspects of this study the UE is receiving with One Rx RF chain, but it is physically 4 receivers that are active to support 4Rx and up to rank 4 in DL MIMO. We don’t say 4 Rx RF chains for this system, but one Rx RF chain when we discuss fragmented carrier operation, and as we pointed out in the very start of this study, this will become confusing.

Further the system above does not have 4 Rx RF chains in spare due to the fragmented carrier operation configuration. It has 1 Rx RF chain in spare that may support 4Rx for DL MIMO reception. The reason is that the UE only gets one spare LO from the fragmented carrier operation, and therefore only supports one additional frequency band/carrier, but has 4 receiver paths available inside the transceiver confined to the one available LO for down-conversion.

From this it is clear that the determining factor on whether fragmented carrier operation can be supported by the UE is on the ability of covering multiple CCs with a single filter, down-conversion and base-band processing. Not how many antennas that are utilized.

RAN4 shall decouple the discussion on spare or freed-up Rx RF chains from the number of antennas.

Mandatory Rx antenna support

The current specification states whether a band is mandated to have support for 2 or 4Rx. In this relation the 2 or 4 refers to assumptions for REFSENS and also the possibility for DL MIMO rank. The intention from our side is not to reduce the minimum number of required Rx, 2 or 4, for a given band. Rather the assumption from our side is to use “idle” Rx RF chains or change the assumption of separate Rx RF chains per CC to a shared Rx RF chain for multiple CCs.

RAN4 shall not consider reducing the minimum number of required Rx, 2 or 4, for a given band to enable fragmented carrier operation for receiving a NC IB CA using one Rx RF chain.

As discussed from our side in multiple meetings, the NR receiver mandated to support 4Rx only for some bands. Of those found in this study we see that n7 and n41 must support 4Rx, but for other bands it may be UE implementation specific. Since the antennas are shared for CA operation, then implementing 4Rx for some bands may naturally serve other bands sharing the antennas. For that reason, we have the situation that some bands of the study support a higher antenna count than required.

Some bands may have a higher antenna count than mandated available, dependent on UE implementation.

These bands with more Rx antennas than required may have the option to improve the reception of the fragmented carrier operation configuration using an Rx antenna reduction. Such improvement may be considered not as a fallback, but as a mitigation.

The mitigation does however spend the LO that was in spare from the fragmented carrier operation, thereby allowing only additional down-conversion paths, but no LO to operate these paths as shown in Fig. 2.

Fig.  2: A 2RX UE implementation example for receiving fragmented carriers with two Rx RF chains.

The mitigation scheme to improve the reception conditions in presence of a large in-gap interferer has “the price” that a UE supporting 4Rx for fragmented carrier reception, must have a spare LO (not the down-conversion paths) to reduce the 4Rx to 2Rx at each CC1 and CC2, for receiving with optimum filtering.

It converts the fragmented carrier operation 4Rx One Rx RF chain reception to a Non-contiguous Intra-band CA reception with 2Rx using Two Rx RF chains.

The trade-off is the DL MIMO rank reduction in favour for protecting the reception of the two fragmented carriers.

UEs with LO availability, but no down-conversion paths may benefit of allowing the transition associated with Fig. 2 operation.

Conclusively we support that a UE may support fragmented carrier operation with DL MIMO layer configuration as a mitigation scheme. A UE declaring support for any CA combination that includes a fragmented carrier combinations, regardless if the CA only consists of the fragmented carriers, or if it is supporting additional carriers of the CA combination, the UE may declare a lower DL MIMO layer support of the fragmented carriers, using this lower DL MIMO support declaration for providing resilience towards in-gap interference in a mode of fragmented carrier operation.

Impact on guardbands and suppression of outer interference

As also discussed at RAN4#113 in [4], the analogue Rx channel filter bandwidth may have an impact on the received fragmented carriers, in particular in the case of the minimum carrier bandwidth of 5 MHz of the CCs and a large frequency offset – a frequency span of up to 100 MHz needs to be considered according to the SID.

For two fragmented carriers with 5 MHz channel BW each (-25 MHz to -20 MHz, 20 MHz to 25 MHz) and a gap of 40 MHz between them, Fig. 7 sketches two dedicated Rx channel filter responses in blue colour and, at the same filter order, an Rx channel filter with a passband of 50 MHz in red colour. The sketch indicates a challenge for the single, 50 MHz wide Rx channel filter in red colour to provide the same low attenuation inside the fragmented carriers' transmission BW configurations and the same stop-band attenuation outside the Rx channel filter BW as the two dedicated Rx channel filters in blue colour.

Fig. 7: 50 MHz channel filter and 5 MHz channel filters at the top and the bottom of the 50 MHz span, same filter order

At a fixed filter order, supporting a narrow guard band between a wanted carrier's transmission BW configuration (passband) and a potential adjacent channel interferer (stopband) is the more difficult, the wider the Rx channel filter is.

Fig. 8 shows the exemplary band n25. Let us focus on the operator with the grey spectrum. We see that the span needed for receiving the fragmented carriers is 50 MHz – this determines the Rx channel filter BW for the fragmented carrier operation. The minimum CC BW is 10 MHz, and the minimum guard band for a 10 MHz wide carrier is, according to TS 38.101-1 table 5.3.3-1 for 15 kHz SCS, 312.5 kHz. An Rx channel filter with a BW of 50 MHz, however, usually needs to be only designed for a guard band of 692.5 kHz according to the same table. The difference in guard band size is more than 2 RBs wide. This means that, to avoid too large attenuation of the outermost 2 RBs by a transition BW of the Rx channel filter designed according to the minimum guard band of a 50 MHz wide carrier instead of a 10 MHz wide CC, a UE supporting fragmented carrier operation may need a rather high filter order of the Rx channel filter.

Fig. 8: Fragment separation in band n25

There is a precedent for the need of a rather high filter order of the Rx channel filter: In the case of contiguous intra-band CA – assuming that the aggregated CCs are received by a single Rx RF chain – the Rx channel filter for the joint reception of the adjacent CCs needs to be at least twice as wide as for the smallest CC. The requirements for the minimum guard bands, however, are rather based on the respective minimum guard bands for each CC than on the total BW of the Rx channel filter. UEs supporting contiguous intra-band CA using a single Rx RF chain can obviously cope with a guard band that is narrower than the minimum guard band listed in TS 38.101-1 table 5.3.3-1 for the actual Rx channel filter BW. TS 38.101-1 table 5.5A.1-1 shows, for example, a combination of 5 MHz channel BW with 25 MHz channel BW, resulting in a minimum guard band of 242.5 kHz for 5 MHz instead of 592.5 kHz for 30 MHz. In fragmented carrier operation, however, the ratio between the narrowest CC BW and the total Rx channel filter BW can be more extreme. Thus, the difference in minimum guard band according to TS 38.101-1 table 5.3.3-1 for the actual Rx channel filter BW and the minimum guard band for the narrowest CC of the fragmented carriers may be more challenging than in contiguous intra-band CA. Furthermore, contrary to the channel spacing flexibility in contiguous intra-band CA according to TS 38.101-1 subclause 5.4A.1, the network cannot increase the outermost guard bands of the fragmented carriers.

In CA using one Rx RF chain, the Rx channel filter needs to support a narrower guard band than what is needed for a single carrier filling the entire Rx channel filter BW, and this aspect may require a higher filter order for fragmented carrier operation than for contiguous intra-band CA.

In order to achieve the same throughput as in legacy non-contiguous intra-band CA, the transmission BW configuration in terms of number of RBs cannot be reduced for increasing the guard band.

Furthermore, if, in an operating band with 2 fragments of the serving operator's spectrum, one of which is initially received, a CA is started by

- making the current carrier the PCell,

- changing the center frequency and increasing the channel filter's passband bandwidth for receiving both fragments using one Rx RF chain and

- adding the other fragment as SCell,

it is highly desirable to keep the PCell's BWP as it was before. Since the purpose of the CA is to increase the DL bandwidth that the UE can use, it can be assumed that the active DL BWP is the carrier's maximum transmission BW configuration. The network should not have to reconfigure (narrow) a UE's DL BWP of a carrier when this carrier becomes part of a CA. In order to keep the PCell's BWP when the CA begins, the transmission BW configuration must remain the maximum transmission BW configuration for the PCell, and thus the guard band must remain the minimum guard band for the PCell's channel BW. Slightly lower performance of the subcarriers at the edge of the channel filter's passband may be compensated by a good implementation margin of the remaining subcarriers so that the overall throughput requirement can be fulfilled.

RAN4 shall consider UEs supporting fragmented carrier operation to support the CC's respective guard bands as in legacy non-contiguous intra-band CA although they are narrower than the minimum guard band specified for a single carrier filling the entire Rx channel filter BW.

A high filter order of the analog channel filter may also be useful

- for fulfilling the out-of-gap ACS and blocking requirements and

- for (in addition to a duplex filter) further attenuating the UE's own TX signal.

Inside the gap between the fragments, relaxations of the required Rx performance robustness against ACI and blocking will be needed, but outside the fragments and the gap between them, the legacy ACS and blocking requirements for the non-contiguous intra-band CA must be fulfilled to prevent an unexpected link performance degradation when the UE starts the fragmented carrier operation using one Rx RF chain. Additional measurements by the UE for checking the expected Rx performance of the fragmented carrier operation may include the gap between the fragments but should not need to cover frequencies below the lower or above the upper fragment.

To prevent an unexpected link performance degradation when fragmented carrier operation starts, the UE's Rx channel filter needs to support, w.r.t. interference below the lower and above the upper CC, the ACS and blocking requirements of a legacy non-contiguous intra-band CA which are tighter than what is needed for a single carrier filling the entire Rx channel filter bandwidth.

RAN4 shall for the non-contiguous intra-band CA using a single Rx RF chain, consider the ACS and in-band blocking requirements are only relaxed inside the gap between the CCs.

If, in the DL, the PCell (having DL+UL) is closer to the UL band than the SCell (having only DL), the UE should, even in fragmented carrier operation, still fulfil the normal sensitivity requirements. If, in the DL, the SCell is closer to the UL band than the PCell, ΔRIBNC should not be worse than what is required for legacy non-contiguous intra-band CA because otherwise, there could be a considerable and potentially rather unpredictable DL desensitization.

In the non-contiguous intra-band CA using a single Rx RF chain, i.e. fragmented carrier operation, ΔRIBNC shall be as for a legacy non-contiguous intra-band CA.

This discussion on which guardband assumptions apply for a UE configured for fragmented carrier operation should be concluded as an initial assumption if this work continues into work item phase. Therefore, it is suggested to capture this aspect in the TR as presented in [5].

RAN4 shall capture the aspects of the proposals 3-5 in the TR for the SI.

Conclusion

In this contribution, we discuss the study item on fragmented carriers. The following Observations and Proposals were made:

Observation 1: It is not always clear within RAN4 what is meant by “Rx RF Chain”.

Proposal 1: RAN4 shall decouple the discussion on spare or freed-up Rx RF chains from the number of antennas.

Proposal 2: RAN4 shall not consider reducing the minimum number of required Rx, 2 or 4, for a given band to enable fragmented carrier operation for receiving a NC IB CA using one Rx RF chain.

Observation 2: Some bands may have a higher antenna count than mandated available, dependent on UE implementation.

Observation 3: At a fixed filter order, supporting a narrow guard band between a wanted carrier's transmission BW configuration (passband) and a potential adjacent channel interferer (stopband) is the more difficult, the wider the Rx channel filter is.

Observation 4: In CA using one Rx RF chain, the Rx channel filter needs to support a narrower guard band than what is needed for a single carrier filling the entire Rx channel filter BW, and this aspect may require a higher filter order for fragmented carrier operation than for contiguous intra-band CA.

Proposal 3: RAN4 shall consider UEs supporting fragmented carrier operation to support the CC's respective guard bands as in legacy non-contiguous intra-band CA although they are narrower than the minimum guard band specified for a single carrier filling the entire Rx channel filter BW.

Observation 5: To prevent an unexpected link performance degradation when fragmented carrier operation starts, the UE's Rx channel filter needs to support, w.r.t. interference below the lower and above the upper CC, the ACS and blocking requirements of a legacy non-contiguous intra-band CA which are tighter than what is needed for a single carrier filling the entire Rx channel filter bandwidth.

Proposal 4: RAN4 shall for the non-contiguous intra-band CA using a single Rx RF chain, consider the ACS and in-band blocking requirements are only relaxed inside the gap between the CCs.

Proposal 5: In the non-contiguous intra-band CA using a single Rx RF chain, i.e. fragmented carrier operation, ΔRIBNC shall be as for a legacy non-contiguous intra-band CA.

Proposal 6: RAN4 shall capture the aspects of the proposals 3-5 in the TR for the SI.

- References

- R4-2505109, WF on NR FR1 DL fragmented carriers study, RAN4#114bis

- R4-2502873, WF on NR FR1 DL fragmented carriers study, RAN4#114

- R4-2415390, Discussion on methods for reducing the number of UE Rx RF chains for Fragmented Carriers, RAN4#112-bis, Nokia

- R4-2419455, Discussion on Fragmented Carrier impacts on UE RF requirements, RAN4#113, Nokia

- R4-2507376, TPs to TR 38.755 with aspects of Fragmented CA, Nokia