3GPP TSG-RAN WG4 Meeting #115	R4-2507385

St Julian’s, Malta, May 19th –May 23rd, 2025

Source:	Nokia

Title:	Discussion on LBCA RRM requirements

Agenda item:	7.7.4

Document for:	Discussion

Introduction

In this contribution we continue the discussion about the necessary RRM requirements for LBCA with switching.

Assumptions

SSB assumption on SDL SCell

In the last meeting, RAN4 discussed the support of SSB-based and SSB-less SCell operation in LBCA.

Issue 1-1-1: SSB assumptions for R19 LB CA via switching - Option 1 (OPPO, LGE, MTK, Nokia): RAN4 to use SSB based operation as baseline for Rel-19 low band CA via switching. - Option 1a (LGE): RAN4 to specify SSB-based SCell operation as baseline, and also specify SSB-less SCell operation. - Option 1b (MTK): RAN4 to focus on discussing and defining requirements for SDL SCell operation for SSB-based Scell, i.e. RRM to deprioritise the scenario of SSB-less Scell based. - Option 2 (Apple): RAN4 to use SSB-less based operation as baseline for Rel-19 low band CA via switching. - Option 3 (xiaomi): RAN4 shall discuss and define requirement for SDL SCell operation for both SSB-based and SSB-less. - Option 4 (Ericsson): If the SSB less SCell is considered to be in scope, use NES rel-18 WI as baseline for specifying LB-LB CA between collocated FDD-carrier and SDL-carrier for via switching in rel-19. - Recommended WF - [moderator]: Decide between option 1 and option 2. The SSB-less operation here does not mean SSB is not transmitted on SDL SCell, and that means UE may use the same operation as for SSB-less case, e.g., skip L3 measurement on SSB-less SCell. - Option 1 (OPPO, LGE, MTK, Nokia,Xiaomi, QC): RAN4 to use SSB based operation as baseline for Rel-19 low band CA via switching. - Option 2 (Apple): RAN4 to use SSB-less based operation as baseline for Rel-19 low band CA via switching.

Two options were listed in the recommended way forward: either to define requirements based on SSB-based SCell operation or to define the requirements based on SSB-less operation. In our view SSB-based operation should be the baseline. While SSB-less operation would simplify many of the RAN4 requirements, we do not see that this scenario alone can ensure the feasibility of the feature. Hence, RAN4 should at least define RRM requirements for SSB-based SCell operation. If there is time, SSB-less operation can be supported if the specification impact is not major.

Additionally, RAN1 already agreed on a switching pattern that should be flexible enough for enabling measurements on both FDD and SDL carriers. RAN1 agreement is below [3]:

Agreement - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support bitmap design - NW configures a periodicity, denoted as P slots, and a bitmap of P bits. Each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2 - The pattern starts from the beginning of SFN 0 of PCell - Restriction of maximum X switch(es) within Y slot(s), - FFS: detailed value and relation of X and Y. - FFS: other restriction

According to the RAN1 agreement, the network configures a bitmap with P bits, 1 bit per slot, where each bit indicates which carrier is used, The periodicity of the pattern is of P = 40 ms. With such a pattern, the UE can configure either non-overlapping SSB/SMTC; or it can configure the UE with a pattern where the one SSB is used for FDD and another used for SDL.

Figure 1 shows one example of how the LBCA-switching pattern can be used. In this example, the SMTC/SSB window starting at slot #0 is used for the PCell, while the SMTC/SSB window starting at slot 20 is used for the SCell.

Figure 1 Example of LBCA-sw pattern covering 2 SMTC periods

RAN1 agreement on switching pattern is flexible enough to enable a configuration that allows for measurements on SDL and FDD carriers.

Use SSB-based SCell operation as the baseline for LBCA.

Interruption requirements

Interruption requirements between FDD PCell and SDL SCell

In the last RAN4 meeting, the following was left open for discussion [1]

Issue 1-2-1A: Necessity of interruption requirement in RRM between FDD CC and SDL CC

RF has agreed the duration of the switching period, which network will take into account in the switching pattern configuration and by not scheduling the UE during the switching. As the timing of the switching is known, the network has no reason to schedule the UE during this time. Hence, we do not think the switching requires an interruption requirement between the PCell and SCell. RAN4 can of course still wait for finalization of the switching pattern by RAN1, based on which  RAN4 can decide whether there is anything to capture in RAN4 spec regarding the switching period. However, our current view is that this is taken care by RAN1 and there is no need for RAN4 to capture interruption or other requirements regarding UE not being able to receive/transmit during the switching period.

No need to define interruption requirements or scheduling restrictions for the switching period between PCell and SCell. Assume that RAN1 pattern design takes the switching period into account sufficiently.

Interruption on a third band

Issue 1-2-1B: Necessity of interruption requirement in RRM when more carriers are considered

In RAN4 #114, RF agreed that there may be a third carrier combined with 2 low band carriers, which not to be used with a switching pattern.

Issue 1: Scope clarification - Specify the switching period(s) based on the scenario with the switching between two low bands in Rel-19 WI - It is not precluded that such two low bands are further combined with the additional band - The combination of those two low bands with other band can be further discussed in the future release

Considering the RF agreement, it is not precluded that additional carriers can be combined with the LBCA-sw, although the scenario to be specified in the ongoing Rel-19 work item only considers two band. In this scenario, we have 3 band, 2 low band carriers which operate with carrier switching, and another band that does not need carrier switching. If this is allowed by RF and UE capabilities, we need do discuss from RRM perspective whether the 3rd carrier may experience interruptions or not.

RF has reached agreement that a scenario with 2 low bands operating with an additional band is not precluded.

RAN4 to discuss the need for interruption requirements in the following scenario

  - UE configured with 2 low band carriers operating with carrier switching, and a 3rd band carrier operating without switching

Regarding the scenario above, allowing for interruptions could be very detrimental for performance of the 3rd band. In L3 measurements, RAN4 has specified interruption requirements for measurements without gaps, and those interruptions can be controlled by the network by configuring the SMTC periodicity, e.g. 40, 80, 160 ms. In the case of LBCA-sw the situation is different. If the UE architecture is such that it causes interruptions on the 3rd carrier every time it switches between FDD and SDL carrier, the interruption overhead would be very high for the 3rd carrier. We still don’t have the RAN1 design of the carrier switching pattern, but it can be assumed that it needs much more frequent switching than what is used for L3 measurements. In one example, if the LBCA-sw pattern has a periodicity of 10 ms, and the UE causes 2 interruptions during this period, the number of interruptions on the 3rd carrier would be very high. If that is needed, RAN4 needs to discuss what are the interruption requirements, and how the UE can signal to the network which band combinations generate interruptions on the non-switched carriers.

Since this WID has only few meetings to conclude the work, we believe that the best approach is to consider only the cases without interruptions on the 3rd carrier as part of this WID.

If a third band is configured without switching pattern, no interruption is allowed on the third band due to LBCA-sw operation on PCell and SDL SCell.

RRM requirement applicability with switching

The following issue was discussed in RAN4 #114bis about requirements for LBCA switching [2]

Issue 1-1-4: RRM requirement applicability for R19 LB CA via switching - Recommended WF - [Moderator]: check which method can be used in P1. P2 can be further discussed in the detailed requirement design. - Discuss which method can be used: - Method 1: Agree on the principle of extending the delay based on missed reception occasion of necessary DL signals/channels and apply this principle to all the aforementioned procedures. - Method 2: Specify an applicability condition under which the requirements are valid for low-band CA via switching

Given the agreement on RAN4 #114bis that the switching pattern only applies after the activation of the SDL SCell, we consider that this issue is only for the activated SDL.

In our view, the RRM requirements can be revised in a straightforward way using something similar to Method 1 proposed under issue 1-1-4. If we take the example of Figure 1, half of the SSB&SMTC occasions are available for PCell and the other half are available for the SCell. In that case, we can determine that the measurement delays are extended accordingly. On the other hand, if the configured LBCA-sw pattern is not covering SSB/SMTC on both carriers, then we can assume that the requirements don’t apply.

For this discussion we can consider 4 scenarios:

- Scenario 1: SSB occasions from FDD and SDL carriers do not overlap, and the LBCA-sw pattern allows the UE to monitor all SSB occasions.

  - In that case, no measurement delay extension is needed, and the legacy requirements can be reused without changes.

- Scenario 2: SSB occasions from FDD and SDL carriers do not overlap, and the LBCA-sw pattern doesn’t allow the UE to monitor all SSB occasions.

  - In that case, since there are less SSB occasions available, the measurement delay can be extended taking into account how many SSB occasions are available.

- Scenario 3: SSB occasions from FDD and SDL carriers overlap, and the LBCA-sw pattern doesn’t allow the UE to monitor all SSB occasions.

  - In this scenario, since the SSB occasions overlap, it is not possible to configure the LBCA-sw pattern in a way that solves the overlapping conflict. Therefore, delay extension is expected.

- Scenario 4: LBCA-sw pattern does not cover any of the SSB occasions of one carrier

  - This can be considered as an atypical scenario, as the LBCA-sw pattern should be configured to guarantee that there are atl least some SSB occasions available for each carrier. Therefore RAN4 can define an applicability rule excluding the requirements for this case.

For an activated SDL SCell with LBCA-sw, for a serving cell which can be either FDD PCell or SDL SCell:

  - If all the SSB occasions of the serving are contained within the LBCA-sw pattern, no delay extension is allowed

  - If a fraction the SSB occasions of the serving are contained within the LBCA-sw pattern, the delay is extended in accordance to the fraction of available SSB occasions

  - If no SSB occasion of the serving cell is contained within the LBCA-sw pattern, the requirements do no apply

SCell requirements

L3 measurements on activated and deactivated SDL SCell

Issue 1-3-1: Activated SDL SCell L3 measurement and corresponding interruption

For L3 measurements for activated SDL SCell, the principles proposed in section 4 of this contribution can be used. If the SSB availability is guaranteed on both PCell and SCell and the SSBs are not overlapping, the existing requirements can be considered to apply. If the availability of SSB is either not guaranteed with the same periodicity or the SSBs are overlapping, the measurement delay can be extended.

Apply the principle of Proposal 5 to L3 measurements on activated SCell.

Issue 1-3-2: Deactivated SDL SCell measurement and corresponding interruption

Regarding deactivated SCell measurements, it was agreed that the switching pattern is not to be applied when the SCell is deactivated.  In previous releases, deactivated SCell measurements have been considered using gap-less measurements following measCycleSCell. The measurements without gaps are only possible in this situation because the UE has a spare RF chain that can be used for measurement of the deactivated SCell. However, for the case of LBCA-sw it is not clear how the UE can perform the measurements. In this scenario, whenever the UE is monitoring the signals from the SDL SCell it cannot receive DL data in the FDD PCell.

If the UE cannot receive DL signals for FDD and SDL carriers simultaneously, then measurements without gaps cannot be used for LBCA-sw.

Since the UE cannot monitor the SDL while receiving data in the FDD carrier, we need some kind of gap-assisted measurements to monitor the deactivated SDL SCell. On the other hand, once the SDL carrier is activated, maintaining the measurement gap for the SDL carrier would not be efficient. One efficient way of performing measurements with gaps for the deactivated SCells would be to update the PreMG conditions to activate and deactivate the PreMG based on the activation of the SDL carrier operating with LBCA-sw.

RAN4 to discuss how to perform efficient deactivated SCell measurements using gaps.

SCell activation delay

Issue 1-3-3: SDL SCell activation/deactivation

In the last meeting RAN4 agreed and shared with RAN1 in an LS the understanding that switching pattern is only to be applied when the SCell is activated:

Issue 1-1-3: switching pattern applicability for RRM requirement in R19 LB CA via switching Agreement: - For the SCell which has been activated(i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133), - The switching pattern for SDL SCell is applied. - For the SCell which is in activation procedure, - The switching pattern for SDL SCell is not applied. - RAN4 defines SCell activation requirement assuming UE utilizes the RS occasions to perform SDL SCell activation - the above RS occasions are FFS - For the SCell which is deactivated, - The switching pattern for SDL SCell is not applied. - If the deactivated SDL SCell measurement requirement is needed, FFS whether UE follows legacy deactivated SCell measurement requirements to perform deactivated SDL SCell measurement or follow a new measurement requirements for deactivated SDL SCell measurement. Note: the above FFS parts will be discussed and decided in RAN4.

Based on this agreement, RAN4 now needs to agree how to define SCell activation delay requirements for an SDL SCell that cannot be used without switching. There were proposals on the table to define the activation delay by redefining interruption requirements and/or defining scheduling restrictions.

Amend the previous agreement with the following

  - The UE shall continuously monitor PCell disregarding the configured switching pattern.

Existing  SCell activation delay requirements shall apply for activation of SDL SCell.

Considering that the switching pattern is not applied, it is reasonable to assume that the UE would complete the SCell activation as fast as possible. Hence, the UE would need to be active on the SCell to be activated during the measurement occasions that are needed for SCell activation. With this understanding, the SCell activation delay itself should not be changed from the legacy. However, RAN4 has to define interruption and/or scheduling restrictions that take into account that the UE will be switching from PCell to SCell when necessary.

RAN4 to define interruption requirements and/or scheduling restrictions for PCell to cover UE switching to the SDL SCell to perform the necessary measurements during SCell activation.

Regarding SCell deactivation delay, this does not require any measurements to be performed on the SCell to be deactivated, and therefore we do not see an impact on the deactivation delay requirements.

Reuse the existing SCell deactivation delay requirement for SDL SCell deactivation.

There have been proposals that SCell activation delay can be optimized in the LBCA-sw scenario. For the specific scenario where the PCell and SCell are collocated and closely spaced in frequency, they can be considered synchronized, Under such conditions, the network can indicate the UE to reuse the PCell measurements without the need for extensive measurement on the to be activated SCell, for e.g. performing time/frequency tracking and AGC using the SCell SSBs. Nor would there be need for measuring CSI-RS. Therefore, by skipping several steps involved in SCell activation the SCell activation delay can be made shorter. However, this WID don’t have enough time for studying exactly which conditions can be used and whether they always apply. Therefore, if companies want to develop such kind of faster switching behaviour, at least this should be controlled by the network and not be based on conditions that are not necessarily clear and known to the UE and the network.

The network can indicate when the UE can reuse PCell measurements for fast SCell activation.

Furthermore, RAN4 needs to clarify which types of SCell activation are supported when the UE supports LBCA.

Clearly PUCCH SCell activation/deactivation delay requirements are not needed as the SDL SCell does not have uplink. It is also not necessary to define delay requirements for activation of multiple SCells. Furthermore, Rel-19 introduced fast SCell activation through EMR, which we do not see should be impacted as in general interaction between parallel WIs of the same release should be avoided.

PUCCH SCell activation, Rel-19 fast SCell activation through EMR, and activation of multiple SCells is not in the scope of the WI.

Considering the requirements that may be impacted, RAN4 should agree if all of these are in the scope of the WI or not:

- MAC-CE based SCell activation

- Direct SCell activation at SCell addition

- Direct SCell activation at handover

- Fast SCell activation

- SCell activation with L3 reporting during activation

Furthermore, RAN4 needs to clarify whether both known and unknown SCell activation are in the scope.

RAN4 to clarify which of the existing SCell activation types and delay requirements are in the scope of the LBCA WI:

  - MAC-CE based SCell activation

  - Direct SCell activation at SCell addition

  - Direct SCell activation at handover

  - Fast SCell activation

  - SCell activation with L3 reporting during activation

RAN4 to clarify whether both known and unknown SCell activation is in the scope of the WI.

Other SCell RRM requirement impact

Issue 1-3-4: BFD/CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirements for SDL SCell

Considering other requirements than SCell activation and measurements, the following requirements seem needed for SDL SCell:

- BFD/CBD requirement

- TCI state switching delay

For BFD/CBD evaluation delay requirements we think the proposal under section 4 in this contribution can be followed. Depending on the availability of RS to be used for BFD/CBD evaluation, the delay can be either the same as legacy or extended if the availability of the RS is impacted by the pattern.

Define BFD/CBD requirements based on the availability of the RS to be used, where the legacy requirements can either be reused if RS availability is not changed, or the evaluation delay can be extended if the RS availability is reduced from the legacy by the switching pattern.

For TCI state activation, what is impacted is the definition of Tfirst-SSB, which is in legacy the time until the first SSB after processing the TCI state activation MAC-CE. Small edit should be sufficient here, where the definition is updated to”the first SSB after processing of the TCI state activation command which is available according to the switching pattern”. Furthermore, if the target TCI state is unknown, the delay includes L1-RSRP measurement period. Here we think the impact of RS availability can be directly reflected.

TCI state activation delay impact is on the definition of:
- Tfirst-SSB, which shall be the first SSB after MAC-CE processing based on the switching pattern, and 
- TL1-RSRP, if the TCI state is unknown, where the availability of the RS to be used may be impacted by the switching pattern

PCell requirements

Timing requirements for PCell

Issue 1-4-1: Impact on timing requirement for PCell

Regarding timing requirements for PCell, RAN4 informed RAN1 already in the LS about the necessity of SSB availability at least every 160 ms. In our understanding, this can easily be taken into account through the switching pattern, as we have shown previously in this contribution. We do not see a reason why the network would not ensure SSB to be available with this periodicity, so we do not see a need for new requirements.

Switching pattern allows network to ensure SSB availability every 160 ms, so no need to define new timing requirements for PCell.

Conclusion

The following Observations and Proposals were made:

Observation 1: RAN1 agreement on switching pattern is flexible enough to enable a configuration that allows for measurements on SDL and FDD carriers.

Proposal 1: Use SSB-based SCell operation as the baseline for LBCA.

Proposal 2: No need to define interruption requirements or scheduling restrictions for the switching period between PCell and SCell. Assume that RAN1 pattern design takes the switching period into account sufficiently.

Observation 2: RF has reached agreement that a scenario with 2 low bands operating with an additional band is not precluded.

Observation 3: RAN4 to discuss the need for interruption requirements in the following scenario

a.	UE configured with 2 low band carriers operating with carrier switching, and a 3rd band carrier operating without switching

Proposal 3: If a third band is configured without switching pattern, no interruption is allowed on the third band due to LBCA-sw operation on PCell and SDL SCell.

Proposal 4: For an activated SDL SCell with LBCA-sw, for a serving cell which can be either FDD PCell or SDL SCell:

a.	If all the SSB occasions of the serving are contained within the LBCA-sw pattern, no delay extension is allowed

b.	If a fraction the SSB occasions of the serving are contained within the LBCA-sw pattern, the delay is extended in accordance to the fraction of available SSB occasions

c.	If no SSB occasion of the serving cell is contained within the LBCA-sw pattern, the requirements do no apply

Proposal 5: Apply the principle of Proposal 5 to L3 measurements on activated SCell.

Observation 4: If the UE cannot receive DL signals for FDD and SDL carriers simultaneously, then measurements without gaps cannot be used for LBCA-sw.

Proposal 6: RAN4 to discuss how to perform efficient deactivated SCell measurements using gaps.

Proposal 7: Amend the previous agreement with the following

a.	The UE shall continuously monitor PCell disregarding the configured switching pattern.

Proposal 8: Existing  SCell activation delay requirements shall apply for activation of SDL SCell.

Proposal 9: RAN4 to define interruption requirements and/or scheduling restrictions for PCell to cover UE switching to the SDL SCell to perform the necessary measurements during SCell activation.

Proposal 10: Reuse the existing SCell deactivation delay requirement for SDL SCell deactivation.

Proposal 11: The network can indicate when the UE can reuse PCell measurements for fast SCell activation.

Proposal 12: PUCCH SCell activation, Rel-19 fast SCell activation through EMR, and activation of multiple SCells is not in the scope of the WI.

Proposal 13: RAN4 to clarify which of the existing SCell activation types and delay requirements are in the scope of the LBCA WI:

a.	MAC-CE based SCell activation

b.	Direct SCell activation at SCell addition

c.	Direct SCell activation at handover

d.	Fast SCell activation

e.	SCell activation with L3 reporting during activation

Proposal 14: RAN4 to clarify whether both known and unknown SCell activation is in the scope of the WI.

Proposal 15: Define BFD/CBD requirements based on the availability of the RS to be used, where the legacy requirements can either be reused if RS availability is not changed, or the evaluation delay can be extended if the RS availability is reduced from the legacy by the switching pattern.

Proposal 16: TCI state activation delay impact is on the definition of: - Tfirst-SSB, which shall be the first SSB after MAC-CE processing based on the switching pattern, and  - TL1-RSRP, if the TCI state is unknown, where the availability of the RS to be used may be impacted by the switching pattern

Proposal 17: Switching pattern allows network to ensure SSB availability every 160 ms, so no need to define new timing requirements for PCell.

- References

- R4-2504895, WF on RRM requirements for NR_LBCA_Sw, Apple

- R4-2503614, Topic summary for [114bis][204] NR_LBCA_Sw, Apple

- R4-2502588, WF on RRM requirements for NR_LBCA_Sw, Apple

- RAN1 Chair’sNotes, RAN1 120 bis

- Annex A: Relevant RAN1 agreements

Agreements from RAN1 120bis:

- Low band carrier aggregation via switching Please refer to RP-250247 for detailed scope of the WI for low band carrier aggregation via switching. Agreement - For Rel-19 low NR band carrier aggregation via switching, support only semi-static configuration of switching pattern to a UE based on UE-specific RRC configuration between the following two cases - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - FDD carrier 1 is PCell and SDL carrier 2 is SCell - SCS 15KHz on both carriers Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, it is not expected that the same slot (1ms) contains both symbol(s) configured as FDD carrier and symbol(s) configured as SDL carrier. Agreement For Rel-19 low NR band carrier aggregation via switching, the semi-static switching pattern and corresponding switching gap are based on the downlink timing (DL reception time at UE side) of Pcell. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the time unit (resolution) of the semi-static switching pattern configuration is slot (1ms for 15kHz SCS). Agreement - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support bitmap design - NW configures a periodicity, denoted as P slots, and a bitmap of P bits. Each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2 - The pattern starts from the beginning of SFN 0 of PCell - Restriction of maximum X switch(es) within Y slot(s), - FFS: detailed value and relation of X and Y. - FFS: other restriction Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap, - For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier. - For switching from FDD carrier to SDL carrier, downselect one of the following solutions - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier - Alt 3: Switching gap is always assumed at the SDL carrier Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, semi-static switching pattern is periodic with periodicity of P slots (ms), support P= 40 ms Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the duration of the switching gap is RRC configured by the NW, - Switching gap duration can be different for FDD to SDL switch, and SDL to FDD switch. - Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed) - For the UL TA, - Switching gap for SDL to FDD switch ends at the end of slot on SDL - Note: RAN1 assumes that switching gap for SDL to FDD switch includes UL TA - FFS: whether it has specification impact