3GPP TSG-RAN WG4 Meeting #115                                      R4-2507475

St Julian’s, Malta, May 19th – May 23rd, 2025

Source: 	Huawei, HiSilicon

Title: 	On UE’s fallback behavior

Agenda Item:	7.5.2

Document for:	Discussion

Introduction

At RAN4#114bis meeting, the agreement about fallback behaviour was captured in the WF.

- Issue 2-2: Discussion on semi-statically switch hardware resources procedure and fallback considerations

- Further discussions on

  - CA fallback with/without UE architecture switching

    - Consider the fall back

      - from high order CA to lower order CA

      - from CA to single carrier mode

    - Decided by network

  - UE architecture switching:

    - Switching between 2 Rx RF chains and 1 Rx RF chain on CCs of intra-band NC CA

    - FFS on whether it is decided by UE itself or decided by network

      - FFS on whether the additional indication from UE side is needed

This contribution proposes the text proposal in the TR 38.755 to capture the discussion on fallback behaviour.

Text Proposal

<<Start of Change 1 for TR 38.755>>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”.

[2]	RP-241669, “New SID: Study on NR FR1 DL Fragmented Carriers”.

[3] 	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[4] 	3GPP TS 38.101-2: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

[5] 	3GPP TS 38.101-3: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”.

[6] 	R4-2413339 Discussion on methods for reducing the number of UE Rx chains for Fragmented Carriers, RAN4#112, Nokia

[7] 	R4-2415390 Discussion on methods for reducing the number of UE Rx RF chains for Fragmented Carriers, RAN4#112bis, Nokia

[8] 	RP-233374, Fragmented carriers in the DL, TELUS, Bell Mobility, T-Mobile USA, Telstra Limited, US Cellular Corporation, Nokia, Nokia Shanghai Bell

[9] 	R4-2411691 On general aspects of fragmented carriers, RAN4#112, Huawei

[10] 	RP-241269, On Fragmented Carriers in Rel-19, Apple

[11] 	RP-241361, Recommended WF on Fragmented Carriers, MediaTek

[12] 	R4-2417201 WF on NR FR1 DL fragmented carriers study, RAN4#112bis

[13] 	R4-2413031 On architecture options for fragmented spectrum reception, RAN4#112, Skyworks

[14] 	R4-2416138 Discussion on UE Rx chain architecture and network assumption, RAN4#112bis, MediaTek

[15] 	R4-2420519 WF on NR FR1 DL fragmented carriers study, RAN4#113

[16] 	R4-2415804 On methods for reducing the number of UE Rx chain, RAN4#112bis, Huawei

[17]	R4-2503479 TP to TR 38.755 on Clause 5, RAN4#114bis, CATT

[18]	R4-2503558 TP for TR38.755: Company C evaluation results, RAN4#114bis, Qualcomm

[19]	R4-2504452 TP on UE RF requirements impact for fragmented carriers, RAN4#114bis, Apple

[20]		R4-2504214 Further discussion of impacts on UE RF requirements and DL performance impacts, RAN4#114bis, MediaTek

[21]	R4-2504129 General aspects of Fragmented CA, RAN4#114bis, Nokia

[aa]	R4-2503366 On fallback behavior and signaling aspect for fragmented carriers, RAN4#114bis, Samsung

[bb] 	R4-2504213 Discussion on remaining signalling aspects for DL fragmented carriers study, RAN4#114bis, MediaTek

[cc] 	R4-2504450 On methods for reducing the number of UE Rx chains for fragmented carriers, RAN4#114bis, Apple

[dd]	R4-2504260 Views on the usage of the FR1 fragmented carriers, RAN4#114bis, CHTTL

<<Start of Change 2 for TR 38.755>>

- 8.2	CA fallback and architecture switching

According to the evaluation result of RF requirements in section 7, the performance loss could be observed in some scenarios when UE works in “1 Rx RF chain” mode, such as when there is severe Tx leakage, or in-gap interference from non-collocated operators. This section discusses the possible UE behaviour when there a need to leave the “1 Rx RF chain” mode.

In following discussion, two concepts are used: “CA fallback” and “architecture switching”. The “CA fallback” follows the definition of “Fallback band combination” in TS 38.306. When mentioning about “CA fallback”, it means at least one SCell is released, either from the band where “1 Rx RF chain” mode is being used, or from any other bands. The “architecture switching” means the switching from “1 Rx RF chain” to legacy CA architecture (i.e., fully separated Rx Chain or partially shared Rx Chain) at least for one band of a band combination [aa].

Following options are different combinations of “CA fallback” and “architecture switching”. Other parameters such as “DL MIMO layers” might also change along with “CA fallback” or “architecture switching”.

- Option 1: CA fallback without architecture switching

- Option 2: CA fallback with architecture switching

- Option 3: Architecture switching without CA fallback

Below figures is an example to illustrate how the UE’s work mode transfers from “1 Rx RF chain” to options 1~3. The figure is generated based on Figure 3 in [bb] and Figure 1 in [aa].

Figure X-1	UE transfers from “1 Rx RF chain” to options 1~3

The Option 1 doesn’t require additional UE capability and network configuration. As a default, any UE supports the “1 Rx RF chain” will support the Option 1 as the legacy CA fallback configuration. From network point of view, if the performance on CC2 is not at satisfactory level, the network could just release the CC2 using existing CA operation process. Based on the evaluation result in section 7, the performance loss of “1Rx RF chain” is comparable with that of the other CA configurations in the RAN4 specification. It’s therefore feasible to handle the CA configuration with “1Rx RF chain” as a normal CA configuration. It is also beneficial for early deployment of the fragmented carrier feature without requiring upgrading the network nodes. On the other hand, as [cc] pointed out, in some cases the Option 1 might not be the best choice from network’s point of view if the CC2 has larger bandwidth than CC3, or the CC2 could achieve better channel quality using legacy CA architecture than CC3. The Option 2 could address this scenario.

The Option 2 keeps the CC2 and releases CC3 if the network decides the CC2 could provide what the network expects with better performance than CC3. This option requires architecture switching, which means the RF chains previously used by CC3 need to be retuned to CC2 in case all the Rx chains are used up. However, whether the RF chains for different bands could be flexibly switched across the bands depends on UE implementation and is band combination specific. A new UE capability indication of ‘CA fallback combination with architecture switching’ would be needed. The Option 2 would be useful especially when the performance of CC2(CC1) could be improved a lot when switching back to legacy CA architecture, such as when a Tx spectrum for the CA combination is very close to one of the DL spectrum, or the in-gap interference is quite high considering the non-collocated scenario. Additional UE measurement might be needed to assist the network to compare the performance of CC2 and CC3.

The Option 3 doesn’t reduce the number of CCs, but requires architecture switching with reduced DL MIMO layers (Option 3a) or the same DL MIMO layers (Option 3b). Similar to Option 2, both Option 3a and 3b require a new UE capability indication on the ‘Same CA combination with architecture switching” and the UE measurement to assist network configuration. The Option 3b assumes there are enough spare Rx chains. [dd] mentions the benefit of adopting “1 Rx RF Chain” is not very clear from the RF performance aspect if there are spare Rx chains. On the other hand the UE might get the benefit of power saving from “1 Rx RF Chain” if the relevant conditions could ensure a satisfactory Rx performance.

It needs to be considered whether the transition from “1 Rx RF Chain” to Option 1~3 is ‘network initiated’ or ‘UE initiated’. ‘Network initiated’ means the network will send a command to the UE to trigger the transition, with or without assistant indication or measurement report from the UE. ‘UE initiated’ means the UE could execute the transition without receiving the network’s command. UE may inform the network after the transition happened. Option 1, 2 and 3a involve the change of number of CCs or number of DL MIMO layers, which has to be configured by the network, i.e. ‘network initiated’. For option 2, the network configuration might explicitly indicate the architecture switching, or the switching could be done by the UE autonomously. The option 3b could be transparent to the network if the UE has spare Rx chains. The network could also intentionally configure the option 3b. Therefore both ‘network initiated’ or ‘UE initiated’ could apply to option 3b.

As analysed above, the 3 options could fit for different scenarios. Option 1 could be supported by all UEs, while Option 2/3 depend on UE’s capability. The operators could choose to apply Option 1 only, or Option 1+ Option 2 and/or Option 3 based on the knowledge of network deployment, frequency separation between Tx and Rx spectrum and the strength of in-gap interferences.

- References

- RP-241669, “New SID: Study on NR FR1 DL Fragmented Carriers”, MediaTek Inc, RAN#104 Meeting, June 2024

- R4-2505109, “WF on NR FR1 DL fragmented carriers study”, Moderator (Mediatek Inc.), Apple, Qualcomm, RAN4#114bis Meeting, April 2025